| GOVERNMENT OF THE PEOPLE’S REPUBLIC OF BANGLADESH |  |
| --- | --- |
| Local Government Engineering Department (LGED) Local Government Division Ministry of Local Government, Rural Development, and Cooperatives |  |
| # Environmental Assessment (EA) REPORT Name of the Subproject: 1) Rehabilitation of Fulbaria Road from Gohata Bazar to End of Pourashava Boundary with Link Dhanikhola Road (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m), Total Length = 3210 m under Trishal Pourashava. 2) Construction of RCC Drain at Fulbari Road from Gohata Bazar to End of Pourashava Boundary under Trishal Pourashava (Ch. 0+00 to 0+437 m). 3) Construction of 2 nos of Box Culvert (2.00 m* 2.00 m) at Ch. 1215 m & Ch. 2670 m on Fulbaria Road from Gohata Bazar to End of Pourashava. 4) Supplying, fitting & fixing of Street Lights from Gohata Bazar to End of Pourashava under Trishal Pourashava (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m), Total Length = 3210 m Package No: MGSP/TRI/2017-18/W-07 Trishal Pourashava, Mymensingh Municipal Governance and Services Project (MGSP) Design, Supervision, and Management (DSM) Consultancy Services |  |
| Joint Venture of Hifab International AB, Sweden AQUA Consultant and Associates Ltd., Bangladesh |  |

TABLE OF CONTENTS

|  |  |  |  | Page |
| --- | --- | --- | --- | --- |
| 1.0 |  | INTRODUCTION |  | 1-3 |
|  | 1.1 | Subproject Background |  | 1 |
|  | 1.2 | Objectives of the Study |  | 2 |
|  | 1.3 | Scope and Methodology of the Study |  | 2 |
| 2.0 |  | SUBPROJECT DESCRIPTION |  | 4-11 |
|  | 2.1 | The Study Area |  | 4 |
|  | 2.2 | Current Situation, Proposed Intervention and Need for the Subproject |  | 6 |
|  | 2.3 | Envisaged Subprojects Activities and Implementation Process |  | 8 |
|  | 2.4 | ## Category of the Subproject |  | 10 |
|  | 2.5 | ## Subproject Schedule |  | 10 |
| 3.0 |  | BASELINE ANALYSIS OF ENVIRONMENTAL CONDITION |  | 12-19 |
|  | 3.1 | Physical Environment |  | 12 |
|  | 3.2 | Biotic Environment |  | 18 |
|  | 3.3 | Socio-economic and Socio-cultural Environment |  | 18 |
| 4.0 |  | ENVIRONMENTAL SCREENING |  | 20-22 |
| 5.0 |  | SPECIFIC IMPACT, MITIGATION AND ENHANCEMENT MEASURES |  | 23-28 |
|  | 5.1 | Site Clearing, Excavation and Earth Work |  | 23 |
|  | 5.2 | Ecological Impact and Enhancement Work |  | 23 |
|  | 5.3 | Pollution from the Storage of Construction Materials |  | 23 |
|  | 5.4 | Air Quality and Dust |  | 24 |
|  | 5.5 | Noise and Vibration |  | 24 |
|  | 5.6 | Water Quality |  | 25 |
|  | 5.7 | Occupational Health and Safety |  | 25 |
|  | 5.8 | Impacts on Social Environment and Common Property Resources |  | 26 |
|  | 5.9 | Labor Influx and Anticipated Impacts |  | 27 |
|  | 5.10 | Impacts on Traffic Movement |  | 28 |
| 6.0 |  | ENVIRONMENTAL MANAGEMENT PLAN |  | 29-41 |
|  | 6.1 | Access to Information |  | 29 |
|  | 6.2 | Grievance Redress Mechanism |  | 29 |
|  |  | 6.2.1 | Grievance Redress Committee (GRC) | 29 |
|  |  | 6.2.2 | Grievance Resolution Process | 30 |
|  | 6.3 | Institutional Arrangement for Safeguard Compliances |  | 31 |
|  | 6.4 | Capacity Building |  | 32 |
|  | 6.5 | Emergency Response and Disaster Management |  | 32 |
|  | 6.6 | Environmental Management Action Plan |  | 33 |
|  | 6.7 | Environmental Monitoring Plan |  | 38 |
|  | 6.8 | Environmental Management Budget |  | 39 |
|  | 6.9 | Environmental Codes of Practice |  | 40 |
| 7.0 |  | PUBLIC CONSULTATION AND PARTICIPATION |  | 42-44 |
|  | 7.1 | Methodology |  | 42 |
|  | 7.2 | Issues Raised by the Participants |  | 43 |
|  | 7.3 | Feedback, Suggestions and Recommendations of the Participants |  | 43 |
| 8.0 |  | CONCLUSIONS AND RECOMMENDATIONS |  | 45 |
| 9.0 |  | # Reference |  | 46 |
| APPENDIX |  |  |  |  |
|  |  | Appendix I: Environmental Monitoring Checklist |  | 47 |

# LIST OF TABLES

| - Table 3.1.1 | - Major Environmental and Infrastructural Features |  |
| --- | --- | --- |
| - Table 3.1.2 | Temperature, Rainfall and Humidity during 2008-2011 |  |
| - Table 3.1.3 | - Monthly averages of climatic variables at the Mymensingh BMD station, 2001-2012 |  |
| - Table 5.7.1 | - General Requirements for the Workers Health and Safety |  |
| - Table 5.8.1 | - Impacts on Social Environment and Common Property Resources |  |
| - Table 6.2.1.1 | - List of GRC Committee Members |  |
| - Table 6.6.1 | - Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures |  |
| - Table 6.6.2 | - Anticipated Environmental Impacts during Operational Phase and Corresponding Mitigation and Enhancement Measures |  |
| - Table 6.7.1 | - Matrix Table of Monitoring Plan (Visual observation during construction phase) |  |
| - Table 6.8.1 | - Environmental Management Budget in Bill of Quantity |  |

# LIST OF FIGURES

| Figure 2.1.1 | Location Map of the Fulbaria Road |  |
| --- | --- | --- |
| Figure 2.1.2 | Lay-Out Plan of the Fulbaria Road |  |
| Figure 2.1.3 | Topographic Features of Fulbaria Road |  |

# LIST OF PHOTOGRAPHS

| Photographs 2.2.1 | Current Situation of the Fulbaria Road |  |
| --- | --- | --- |
| Photographs 7.1.1 | ### DSM Consultant Team with Pourashava Representatives inspected the proposed road |  |
| Photographs 7.1.2 | ### Consultation Meeting at subproject sites with Pourashava Officials and Consultants Participants |  |

# ABBREVIATIONS

| AP persons (AP’s) | Affected Person |
| --- | --- |
| BDT | Bangladeshi Taka |
| BOQ | Bill of Quantity |
| CC | Cement Concrete |
| CIP | Capital Investment Plan |
| CP | Contingency Planning |
| DSM | Design, Supervision, and Management |
| EA | Environmental Assessment |
| ECR | Environmental Conservation Rules |
| EMP | Environmental Management Plan |
| EPP | Emergency Preparedness Planning |
| ES | Environmental Screening |
| FGD | Focal Group Discussion |
| GoB | Government of Bangladesh |
| GRC | Grievance Redress Committee |
| GRM | Grievance Redress Mechanism |
| GRP | Grievance Redress Procedure |
| LGED | Local Government Engineering Department |
| MGSP | Municipal Governance and Services Project |
| PD | Project Director |
| PMU | Project Management Unit |
| RCC | Reinforcement Cement Concrete |
| RP | Relevant Reports |
| TL | Team Leader |
| WB | World Bank |
| XEN | Executive Engineer |

# 1 INTRODUCTION

## 1.1 Subproject Background

Trishal Pourashava is located in the core of Trishal Upazila under Mymensingh District. Trishal Pourashava was established in the year of 1998. Currently, Trishal is A type Pourashava. It has an area of 15.49 sq.km.  The present population of Trishal Pourashava is about 34,747. The Pourashava has 44.50 km pucca road and 40.50 km earthen road. It has 7.75 km pucca drain and 4.20 km earthen drain (Source: (At a glance Trishal Pourashava).

In Trishal Pourashava, the current infrastructure situation is in vulnerable state. Drains are mostly clogged that cannot properly drain out waste water during heavy rains. Natural drainage systems have either been filled up or occupied by land grabbers creating water logging during monsoon. Traffic volume is increasing day by day with the increase in population and demand. The substandard road network cannot cope with the growing demand; consequently, traffic congestion becomes a common problem. Hence, planned development of road and drain network is essential to meet the emergent demand.

This subproject includes: BC and RCC Road with RCC Drain, Box Culvert, Road Protection Wall and Street Light at Fulbaria Road. The significant features of the subproject are mentioned below:

| Name of the Subproject : | 1) Rehabilitation of Fulbaria Road from Gohata Bazar to End of Pourashava Boundary with Link Dhanikhola Road (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m), Total Length = 3210 m under Trishal Pourashava. 2) Construction of RCC Drain at Fulbari Road from Gohata Bazar to End of Pourashava Boundary under Trishal Pourashava (Ch. 0+00 to 0+437 m). 3) Construction of 2 nos of Box Culvert (2.00 m* 2.00 m) at Ch. 1215 m & Ch. 2670 m on Fulbaria Road from Gohata Bazar to End of Pourashava. 4) Supplying, fitting & fixing of Street Lights from Gohata Bazar to End of Pourashava under Trishal Pourashava (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m), Total Length = 3210 m |
| --- | --- |
| Package No. : | MGSP/TRI/2017- 2018/W-07 |
| District Name : | Mymensingh |
| ULB Name : | Trishal Pourashava |
| Jurisdiction Area : | Ward number 1,2 and 3 |
| Structural Design Option : | BC and RCC Road, RCC Drain, Box Culvert, Road Protection Wall and Street Light |
| Beneficiary Population : | About 11,895 as per information of Trishal Pourashava |
| Tribal People : | No tribal people settlement found in the subproject area |
| Land Acquisition : | No land acquisition is required |
| Estimated Cost : | 57.278 million BDT |
| Subproject Duration : | 12 Months |
| Tentative Start Date : | October 14, 2018 |
| Tentative Completion Date : | October 13, 2019 |

## Objectives of the Study

The general objective of this study is to determine the major environmental impacts succeeding from execution of the subproject and to recommend mitigation measures to avoid or reduce adverse environmental impacts and to enhance positive impacts. The specific objectives include:

- To assess the existing environmental conditions of the subproject site and its influence area;

- To identify and assess impacts resulting from the subproject during its construction phase and operation phase;

- To develop a environmental management plan with recommendations for mitigating impacts and enhance positive impacts;

- To summarize environmental monitoring requirements.

## Scope and Methodology of the Study

The subproject appraisal is included environmental screening for all the subproject components. According to that screening, environmental assessment is required to fulfill the regulatory requirement for BC and RCC Road, RCC Drain, Box Culvert, Road Protection Wall and Street Light. Hence, this environmental assessment is prepared to meet the regularity requirement of this subproject. The study methodology covered the following activities:

- Desktop Study;

- Field Investigations and Data Collection;

- Data Analysis and Report Writing.

Desktop Study

The desktop study involved:

- Initial meetings with client, stakeholders to discuss the proposed subproject;

- Collection and review of the baseline data, maps, reports and other relevant information;

- Review of the relevant existing legislation, regulation and policies;

- Understand the anticipated technical processes that may affect the environment.

Field Investigation and Data Collection

A team of the consultants made a field investigation to the proposed site. Field investigations involved mainly site walks at the subproject areas for identification of the environmental components to be affected due to the subproject intervention.

The following key tasks were performed during the field visit:

- Taking photographs of the significant aspects to assist in describing the baseline environmental conditions of the subproject area;

- Interviews with representatives of the relevant Pourashava officials and beneficiary and affected people within the subproject influence zone;

- Obtaining relevant documents from Pourashava and local people within the subproject influence zone;

- Verifying information and data collected during the desktop study and to collect new information.

Data Analysis and Report Writing

The data and information collected from all the sources (literature review, secondary and primary data, public consultation) were analyzed to describe the existing environmental setting of the subproject area, to identify the potential positive and negative impacts of the proposed subproject, as well as to provide preliminary suggestions for mitigation measures. Finally, this environmental assessment report has been prepared.

# SUBPROJECT DESCRIPTION

## The Study Area

The subproject site is situated within the jurisdiction area of the Trishal Pourashava Ward no. 01, 02, and 03. The starting point of Fulbaria Road is located at 24°34'52.26"N (Latitude) and 90°23'24.40"E (Longitude).The end point of this road is located at 24°35'35.53"N (Latitude) and 90°21'53.71"E (Longitude). The location map, lay-out plans and topographic features of the subproject are shown in Figure 2.1.1, Figure 2.1.2, and Figure 2.1.3.

Figure 2.1.1: Location Map of the Fulbaria Road

Figure 2.1.2: Lay-Out Plan of the Fulbaria Road

Figure 2.1.3: Topographic Features of the Fulbaria Road

## Current Situation, Proposed Intervention and Need for the Subproject

At present, Trishal is category A type Pourashava. The subproject is located at wards no 1, 2, and 3 under Trishal Pourashava. From Ch. 0 to 3110 m at Fulbaria Road and Link Dhanikhola Road, the existing BC (Ch. 0-2485 m and Link Ch.0-100 m) and earthen pavement (Ch. 2485-3110 m) will be demolished and replaced by the new RCC (Ch. 0- 75 m) and BC (Ch 75 m – 3110 and Link Ch 0- 100 m) pavement. Presently, the existing road is badly damaged in most of the sections which hamper the normal traffic operation. The existing road is also narrow (average pavement width is 3 m) and the proposed road width 5 m except (Ch.0 – 410 m) that width is 6 m. From the site investigation, it is revealed that damaged and narrow road interrupts normal traffic operation. Hence, widening and road improvement is necessary for the normal traffic operation. On the contrary, some road side ponds and ditch exist, which necessitates road protection work. Road protection work is needed at left side of the road (total 340 m at 8 different Chainage with different lengths and at right side of the road (total 185 m, at Ch. 570 - Ch.590 m, Ch.1550 -  Ch.1590 m, Ch. 2050 – Ch. 2125 m and Ch. 2200 – Ch. 2250 m).

In the subproject area, there is no drainage facility. Hence, storm water creates water logging problem at different sections of the road especially during monsoon. Stagnation of water on the road surface damages the road. As a result, smooth traffic flow is hampered. To improve the situation, RCC U type drain, cross drains and box culverts are needed for drainage facility. Therefore, RCC drain with cover slab at right side of the road (Ch. 0 – 437 m) and 6 cross drains (Ch. 128 m, Ch. 426 m, Ch 832 m, Ch. 2432 m, Ch. 2798 m, and Ch. 3033 m) will be constructed. The storm water through the proposed drain will be discharged primarily in the main drain of the Pourashava and finally in the Sutia River. At Ch. 1215 m and 2670 m, two RCC box culverts with 2.0 *2.0 m size will be constructed to divert storm water from one side to another side.

Furthermore, absent of the street light hampers the normal traffic operation at night time. Hence, fixing of the street light is also required for improved transport facilities. The street light will be fixed at single side of the road (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m) at 30m interval. The existing condition and inspection of the proposed site is further elaborated in the following Photographs 2.2.1

Photographs 2.2.1: Current Situation of the Fulbaria Road

## Envisaged Subproject Activities and Implementation Process

The general activities of the subproject includes: construction of the semi-pucca site office, construction of the labor shed and relocation of the electrical poles for access road.

Specific activities for RCC Drain:

- Earth work in excavation of the foundation;

- Pumping and bailing out of water as per requirement;

- Sand filling for the preparing foundation bed;

- Laying polythene sheet;

- Plain cement concrete work in foundation;

- Manufacturing CC blocks;

- Fabrication of the ribbed or deformed bar;

- Reinforced cement concrete work.

Specific activities for RCC Box Culvert:

- Demolition of the existing culvert;

- Making earthen ring/cross bundh of required height and width to prevent water from entering in the working area;

- Earth work in excavation of foundation of structures by Long Boom hydraulic excavator;

- Pumping and bailing out of water from the interior of any foundation enclosure of work site;

- Single layer brick flat soling in foundation;

- Plain cement concrete work in foundation;

- Back filling behind wing walls and with selected granular material;

- Brick work as a parapet wall on the culverts;

- Manufacturing and supplying plain cement concrete blocks;

- Laying single layer CC blocks of different sizes on slope of embankment;

- Construction of temporary diversion road.

Specific activities for BC Road:

- Site clearing work;

- Preparation of the hard bed by scarifying and loosening of the existing top;

- Earth work in box cutting;

- Sand filling on the road bed;

- Brick on end edging;

- Compacted aggregates and sub-base course;

- Providing prime coat;

- Lying pre-mixed dense bituminous surfacing.

Specific activities for RCC Road:

- Dismantling of the damaged sub-base and base course works;

- Earth work in box cutting;

- sand filling on the road bed;

- sand and brick chips mix in the sub-base;

- Mechanical compaction;

- Polythene sheet laying work;

- Fabrication of the ribbed or deformed bar reinforcement;

- Reinforced cement concrete work.

Specific activities for Road Protection Wall:

- Earth work in excavation

- Earth filling,

- Polythene sheet laying work,

- Cement concrete work,

- Brick masonry work,

- Fabrication of the ribbed or deformed bar,

- Reinforced cement concrete work,

- Pile driving work.

Specific activities for Street Light:

- Assembling, fitting, fixing, installation,

- Testing and commissioning of the GI pole;

- Fitting and fixing energy meter board;

- Earthling the electrical installation with GI pipe;

- Fixing insulator,

- Erection of tubular pole;

- Fixing of the overhead conductor;

- Fixing of the wire rack;

- Fixing and installation of the switch board;

- Fixing of the water tight street light with aluminum reflector.

The materials to be used for the key activities: bricks, brick chips, stone chips, reinforcement bar, admixture, bearing pad, sand, cement, concrete, paint, laying pre-mixed dense bituminous surfacing etc.

The major equipment to be used for the implementation of the subproject: long-boom hydraulic excavator, brick breaking machine, pile rig for boring, concrete mixer machine, vibrator machine, MS sheet, steel cutter, steel shutter, steel drum rollers, pneumatic multiple tire roller, dump truck, water tanker, pump, generator and measuring device.

## Category of the Subproject

For BC and RCC Road, RCC Drain, Box Culvert and Road Protection Work

- According to ECR 1997: Green □    Orange□ A    Orange B □√ Red □    Not Listed □

- According to WB classification:  Category B □√    Category C □

The overall anticipated adverse impact due to the subproject intervention is limited and site specific. Considering the environmental impact which is anticipated as insignificant, this subproject can be considered as Orange B as per ECR-97.  According to the WB classification, it can be classified as Category B.

For Street Light

- According to ECR 1997	: Green □√    Orange A □  Orange B □ Red □      Not Listed □

- According to WB classification: 	Category B □	             Category C  □ √

Considering the environmental impacts, the street light can be considered as Green category as per ECR-97.  According to the WB classification, it can be classified as Category C.

## Subproject Schedule

The tentative schedule of construction of the subproject is:

(a)  Subproject Duration (months)  	:12 months

(b)  Tentative Start Date    		: 14 October 2018

(c) Tentative Completion Date  		: 13 October 2019

The daily construction hours will normally include regular working time. However, daily working hours may vary based on the on-site condition. The detailed work program will be prepared by the contractor with the assistance of the PIU-Pourashava. Then it will be shared with the PMU, LGED and DSM consultants. In addition, the detailed work program will also be shared with the Bank as per requirement.

# Baseline ANALYSIS OF THE ENVIRONMENTAL CONDITION

## Physical Environment

### Important Environmental and Infrastructural Features

During site visit the environmental and infrastructural features within the subproject area were collected. Conditional survey has also been conducted for designing the subproject. Hence, the survey data is also used for preparing the report. The environmental and infrastructural features listed within four directions of the existing bridges and culvert, two sides of approach roads in Table 3.1.1.

Table 3.1.1: Major Environmental and Infrastructural Features

| Chainage (m) | Left | Right | Major Environmental and Infrastructural Features |
| --- | --- | --- | --- |
| Trishal Gohata Bazar to end point of Pourashava Boundary (Ch. 0 to 3110 m) |  |  |  |
| 0-100 | √ |  | Bismillah Food and confectionary,open space, trees ( koroi, mehagany), pond |
|  |  | √ | Various shops with semi pucca structures, electric poles, house boundary,trees( koroi, betel-nut, mehagany), semipucca houses |
| 100-200 | √ |  | Pond, trees( koroi, coconut, mehagany, date, sajina, mango, betel nut, black berry), kakasia, fence, electric pole, houses with various structures, open space |
|  |  | √ | Houses with various structures, trees( koroi, coconut, mehagany, date, betel nut, jackfruit), electric pole, boundary wall, pond, tin fence |
| 200-300 | √ |  | Residential houses, trees( koroi, coconut, mehagany, date, sajina, mango, betel nut, black berry), kalkasia, bunch of banana, fence, open space, ditch |
|  |  | √ | Houses with various structures, trees( koroi, coconut, mehagany, date, betel nut, jackfruit), boundary wall, tin fence, temporary shop |
| 300-400 | √ |  | Houses with structures, boundary wall, trees( betel nut, coconut), kalkasia, pond |
|  |  | √ | Residential houses with various structures,electric pole, shops, trees( mango, coconut, akashi) |
| 400-500 | √ |  | Ponds, trees(koroi, mango), rice mill, electric pole |
|  |  | √ | Houses, pond, trees( mehagany, coconut, koroi, mango,jackfruit, eucaliptus), open space, bunch of banana,houses |
| 500-600 | √ |  | Trees( date, mango, palm), bamboo bushes, grave yard, house,pond |
|  |  | √ | Bunch of banana, trees( coconut, date, mango, sajina, betel nut, mehagany, shimul), agricultural field |
| 600-700 | √ |  | Pond, trees( palm, boroi, mehagany, shishu, date, dumur), electric poles, different houses, house boundary |
|  |  | √ | Agricultural field, electric poles, tin shed houses |
| 700-800 | √ |  | Trees(black berry, mehagany, shishu, date, dumur), Mosque,agricultural field |
|  |  | √ | Agricultural field, date tree |
| 800-900 | √ |  | Date tree, agricultural field, trees( shimul, dumur, mehagany) |
|  |  | √ | Agricultural field |
| 900-1000 | √ |  | Poultry farm, agricultural field, trees( mehagany,koroi) |
|  |  | √ | Agricultural field |
| 1000-1100 | √ |  | Agricultural field, date tree |
|  |  | √ | Date tree, agricultural field |
| 1100-1200 | √ |  | Agricultural field, date tree |
|  |  | √ | Date tree, agricultural field |
| 1200-1300 | √ |  | Agricultural field, date tree, electric pole, pond |
|  |  | √ | Agricultural field, trees ( koroi, date, mango), electric pole, pond |
| 1300-1400 | √ |  | Pond, tree(jica, palm, date) |
|  |  | √ | Pond, electric pole |
| 1400-1500 | √ |  | Pond, tree(jica, palm, date) |
|  |  | √ | Pond, electric pole |
| 1500-1600 | √ |  | Pond, tree(mango, date) |
|  |  | √ | Pond, electric pole |
| 1600-1700 | √ |  | Trees( mango, date, coconut), pond, electric pole |
|  |  | √ | Pond, trees( palm, date, coconut, papaya), electric pole |
| 1700-1800 | √ |  | Trees( palm, mango, date, boroi), bunch of banana,pond, agricultural field, electric pole |
|  |  | √ | Pond, trees( palm, date, coconut, mehagany, jackfruit, sajina), bunch of banana, electric pole, agricultural field |
| 1800-1900 | √ |  | Agricultural field, trees( date, koroi, jackfruit) |
|  |  | √ | Agricultural field, trees( date, koroi, jackfruit), electric pole |
| 1900-2000 | √ |  | Agricultural field, bunch of banana, trees(betel nut,jackfruit, mango), electric pole |
|  |  | √ | Pond,electric pole, trees( mehagany, koroi,banyon) |
| 2000-2100 | √ |  | Pond, agricultural field,trees(mehagany,akashi, jackfruit) |
|  |  | √ | Ponds, agricultural field,trees(mehagany,date, koroi, jackfruit,boroi, betel nut), electric pole |
| 2100-2200 | √ |  | Pond, agricultural field,trees(mehagany,akashi, jackfruit, palm) |
|  |  | √ | Ponds, agricultural field,trees(mehagany,date, koroi, jackfruit,boroi, betel nut), electric pole |
| 2200-2300 | √ |  | Tree garden, agricultural field,tree(jackfruit,date) |
|  |  | √ | Open space, electric pole, agricultural field, trees |
| 2300-2400 | √ |  | Open Space, electric pole, trees(koroi, date), houses |
|  |  | √ | Electric pole, agricultural field, trees( mango, koroi, jackfruit, date), houses |
| 2400-2500 | √ |  | Mosque, connecting with Nazrul museum road , pond, trees( jackfruit, betel nut, mango, mehagany, koroi, date), electric pole, bamboo bushes |
|  |  | √ | Tree garden, houses, bunch of banana, trees(coconut, mehagany, date, jack fruit) |
| 2500-2600 | √ |  | Pond, electric pole, house |
|  |  | √ | Residential houses, trees( jackfruit, betel nut, mango, mehagany, koroi, date) |
| 2600-2700 | √ |  | Residential houses, pond, trees(mango, koroi), temporary shop, agricultural field |
|  |  | √ | Electric pole, pond, bamboo bushes, trees( jackfruit, betel nut, mango), houses, bunch of banana |
| 2700-2800 | √ |  | Agricultural field, trees, electric pole |
|  |  | √ | Agricultural field, trees |
| 2800-2900 | √ |  | Trees(mango, jackfruit, date, koroi, mehagany, coconut), bamboo bushes, agricultural field |
|  |  | √ | Ditch, bamboo bushes, trees(jackfruit, mehagany), bamboo bushes |
| 2900-3000 | √ |  | Agricultural field, trees( date, koroi) |
|  |  | √ | Agricultural field, trees( date, koroi) |
| 3000-3110 | √ |  | Agricultural field, trees( date, koroi) |
|  |  | √ | Agricultural field, trees( date, koroi) |
| Link-1( Ch. 0-100 ) |  |  |  |
| 0-100 | √ |  | Homestead area with various structures, trees( betel nut, coconut) |
|  |  | √ | Houses, electric pole, open space, trees( coconut, mahogany) |

** Note: There are 29 no. electrical poles to be re-located for the implementation of the subproject.

### Geology, Topography and Soils

Topographically, Trishal Pourashava is a plain land. The plain land mainly consists of fluvio-deltaic sediments deposited by the Ganges, the Brahmaputra and the Meghna River Systems. Geomorphologically, Trishal Pourashava falls under the Floodplains of the Old Brahmaputra and Jamuna Rivers having distinguishing character. Surface geology is consisted of sedimentary rocks originated by riverine processes. Eleven general soil types exist in the area of which deep red brown terrace, shallow red brown terrace soils and Acid Basin Clays are the major ones. They are strongly acidic in reaction with low status of organic matter, low moisture holding capacity and low fertility level. According to BNBC, Bangladesh has three seismic zones with moderate and low seismic activity. The Trishal Pourashava area falls in Zone I, i.e. high intensity seismic zone of the country.

Figure 3.1.1: Location of Trishal in Earthquake Zone Map of Bangladesh

### Climate and Meteorology

The climate of the subproject area can be described as Tropical Monsoon. It is characterized by warm, humid summers and cool, dry winters. From November to March, it is dry and cool while from April to May it is extremely hot during the pre-monsoon season. From June to October, the monsoon season is warm, cloudy, and wet. The warmest month is April, the coolest is January, the wettest is July and the driest is January.

The minimum temperature was 10.8°C in 2011 and maximum temperature was 34.2°C in 2008. The minimum humidity was 66.3% in 2010 and the maximum humidity was 80 % in 2009. The peak rainfall months in Trishal are June, and July. About 90% of the total annual rainfall occurs in the period from May to October. The highest annual rainfall during the last four years (2008–11) was 2235 mm in 2008, and the lowest was 1653 mm in 2009.

According to the statistics of wind data from the Bangladesh Meteorological Department Climate Division, wind direction changes by month. Nevertheless, the northwest, south, and northeast winds are predominant. The average wind speed as recorded at Mymensingh Met Station is in the range of 2 – 7.8 knots/ hr. But in the winter months it remains in the range of 1-4.5 knots/hour. The recent meteorological data (from year 2013 to 2016) is not in hand now. Hence, available meteorological data for the year 2008 -2011 and average climatic variables for the year 2001-2012 are used here for having idea about the meteorological condition.

Table 3.1.2: Temperature, rainfall, humidity during 2008-2011

| Years | Temperature (centigrade) |  | Rainfall (millimeter) | Humidity (%) |
| --- | --- | --- | --- | --- |
|  | Maximum | Minimum |  |  |
| 2008 | 34.2 | 12.5 | 2235 | 79.0 |
| 2009 | 33.0 | 13.2 | 1653 | 80.0 |
| 2010 | 32.4 | 11.7 | 1811 | 66.3 |
| 2011 | 32.2 | 10.8 | 2153 | 79.3 |

Table 3.1.3: Monthly averages of climatic variables at the Mymensingh BMD station,

2001-2012

| Month | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rainfall (mm) | 6 | 11 | 36 | 147 | 284 | 420 | 419 | 341 | 244 | 190 | 16 | 6 |
| Mean Temp (°C) | 17.2 | 20.9 | 24.5 | 26.8 | 27.8 | 28.3 | 28.6 | 28.9 | 28.5 | 26.8 | 22.9 | 18.9 |
| Max Temp (°C) | 22.2 | 25.2 | 28.6 | 30.7 | 31.8 | 31.7 | 31.6 | 31.9 | 31.5 | 30.6 | 27.0 | 23.1 |
| Min Temp (°C) | 8.2 | 11.0 | 14.6 | 19.0 | 20.2 | 22.3 | 24.4 | 24.3 | 23.9 | 19.5 | 13.8 | 9.8 |
| Humidity (%) | 82 | 75 | 75 | 80 | 81 | 86 | 87 | 86 | 86 | 85 | 83 | 83 |
| Sunshine (Hours) | 5.3 | 7.2 | 6.8 | 7.0 | 6.5 | 3.7 | 4.0 | 4.7 | 4.5 | 6.3 | 7.2 | 5.7 |
| Solar Radiation (Cal/cm2/min) | 120 | 152 | 178 | 193 | 188 | 140 | 155 | 162 | 157 | 150 | 146 | 132 |
| Evaporation (mm/d) | 1.9 | 2.8 | 3.9 | 4.6 | 4.5 | 3.8 | 3.7 | 3.9 | 3.7 | 3.3 | 2.7 | 2.0 |

Source: Bangladesh Meteorological Department

The construction works can be influenced by the climatic condition and meteorological components like humidity, temperature, sudden rainfall, and wind speed. High winds can enhance quick spreading of the generated dust. It is very risky working under rain and in high winds because the possibility of getting injury increases. Furthermore, work under high temperature and excess humidity may create health hazards like dehydration problem.

### Water Resources (Surface Water, Ground Water and Rain Water Harvesting)

In the subproject area, there are some khal, ponds and ditches. Ground water is available in plenty and water table does not go beyond suction limit throughout the year. The water of shallow layer contains lot of iron, while water of deep layer does not contain excessive iron. Arsenic concentration is not reported as higher level in both deep and shallow aquifer. Local people typically use deep tube-well water for drinking and other domestic purposes. Salinity problems are not visible. Rain water harvesting system is not common in and around the subproject area.

During construction period, water will be used both for the construction work and domestic purposes. Therefore, ensuring the water requirements is one of the key issues.

Flooding, Water Logging and Drainage Pattern

Trishal Pourashava is free from external flooding. However, Trishal Pourashava is subjected to internal water logging problem due to delay discharge of the storm water during monsoon season. In the subproject area, delay discharge of the storm water causes water logging problem. As per information by the local people the intensity of water logging problem is increasing due to siltation of natural canal and lack of functional and adequate drainage system. However, a flood zoning map and location of the Trishal Pourashava provided here in Figure 3.1.2.

Figure 3.1.2: Location of Trishal Pourashava in Flood Zone Map of Bangladesh

Air Quality and Dust

As there are no major industries in Trishal, the main sources of air pollution are vehicles movement and non-point sources such as open burning and black smoke emission from vehicles. During dry season the windblown dust also degrades the air quality.

### Noise Level

Noise level data is not available. The major causes for noise in the subproject area are the vehicle movement (motor cycles and auto-rickshaws), playing of loud-speaker and mass people gathering (for advertising of products and political, social and religious aspects) and local market. This is a common experience of the peri-urban population that noise poses a threat to the ill / physically weak people health and nerves.

Solid Waste Management

The local people normally dump their solid wastes in the road side drains, open space or roads adjacent to their houses, low lying areas, in khals and ditches. Therefore, improper solid waste disposal by the community people creates severe public health hazards and degrading the quality of living environment. Currently, the vacant place at Naodhar beside Dhaka-Mymensingh Highway is used as a Pourashava designated dumping site. Basically, it is not structured solid waste management system in this area.

## Biotic Environment

### Flora and Fauna

In the subproject area, the floral species includes: Koroi, Mehagany, Betel-nut, Mango, Coconut, Date, Sajina, Black-berry, Jackfruit, Akashi, Eucaliptus, Palm, Shimul, Boroi, Shishu, Dumur, Jica, Boroi, Banyan, Papaya etc. The common local birds such as Crow, Sparrow, Bulbuli, Tuntuni, Shalik etc. were found during the site visit. There are no wild animals and endangered faunal species in the subproject area.

## Socio-economic and Socio-cultural Environment

Land Use, Status of Housing and Built-up Infrastructure

Like other Pourashava in Bangladesh, the land use pattern in the subproject area consists of mixed land-use pattern. The major roadside built-up infrastructures includes bismillah food and confectionary, various shops with semi pucca structures, houses with various structures, house boundary, residential houses, Mosque. In addition, ponds, ditch, common planted trees, bunch of banana, shrubs, bushes, agricultural field, low land and open space were found during site visit.

Beneficiary Population

The subproject is situated within the jurisdiction of the Ward number 01, 02 and 03. Considering the ward population about 11,895 people will benefit directly and many others indirectly.

Education

In the Pourashava area, literacy among the population is about 61.0%. This is higher than the national average (51.8%). The literacy rate among males (62.4%) is more than females (59.6%). (Ref: Population and Housing Census 2011).

Tribal Communities

There is no indigenous or tribal people settlement in the subproject area. Hence, no measures needed for the indigenous peoples’ safeguard.

### Land Acquisition and Resettlement

Private land acquisition is needed for road widening work. The land owners are agreed to donate their land for the development works. In this regard, an agreement has been signed between land owners and Pourashava Authority (Agreement copies have been attached in the subproject documentation). However, in case of any objection from the affected persons, DSM will investigate the actual situation. Then, measures will be taken as per rules.

Principal Livelihoods and Economic Activities

The subproject area is inhabited by the mixed occupational people. In the Pourashava area, major income comes from business, enterprises, small trades, private sector jobs, and government jobs in the town.

Cultural Heritage and Protected Areas

During site visit it was observed that there is no protected and important cultural or historical site in the subproject influence area.

Social Conflicts and Development Activities

The local people welcome the subproject and there is no conflicts between the local communities regarding subproject implementation.

# ENVIRONMENTAL SCREENING

Environmental Screening Checklist, as adopted in Appendix C of the Environmental Management Framework (EMF) of the MGSP, was administered for identifying the impacts and their extents.

The screening data and information for the BC and RCC Road, RCC Drain, RCC Box Culvert and RCC Road Protection Work have been formulated and are shown in below.

2)  Potential environmental impact during construction phase:

(a)  Ecological impacts:

Felling of trees  		Significant  □√   	Moderate  □   	Minor  □   Number of trees

297

- Clearing of vegetation  	Significant  □  	Moderate  □√   	Minor □

- Potential impact on species of

aquatic (i.e., water) environment  Significant □    Moderate □  Minor □√

Major trees to be felled down are: Koroi, Mehagany, Betel-nut, Mango, Coconut, Date, Sajina, Black-berry,Jackfruit, Akashi, Eucaliptus, Palm, Shimul, Boroi, Shishu, Dumur, Jica, Boroi, Banyan,Papaya etc. There is no tree to be felled down bring religious and heritage importance. However, the trees to be felled down have moderate economic value because most of them are moderate to small size in terms of length and diameter. Vegetation coverage on both sides of the road is not similar in terms of the intensity. So clearing of the road side vegetation is considered as moderate. In addition, anticipated impact on species of aquatic environment is minor because except throwing of the wastes materials in to the ponds and ditches, the subproject does not have any impact on the aquatic environment. Considering the number of trees and vegetation to be felled down, the ecological impact is considered as moderate.

- Physicochemical impacts:

- Noise pollution  			Significant □  	Moderate □√	Insignificant  □

- Air pollution  				Significant □  	Moderate □√	Insignificant  □

- Drainage congestion  			Very likely □  	Likely       □   	Unlikely         □√

- Water pollution  			Significant □  	Moderate □	Insignificant  □√

- Pollution from solid/ construction wastes  Significant □  	Moderate □√	Insignificant  □

- Water logging  				Significant □  	Moderate □√	Insignificant  □

The subproject will have impacts on noise and air quality during construction phase due to mobilization of the equipment, vehicles movement for the transportation of the materials, using of hydraulic excavator, brick breaking machine, mechanical compaction machine, paver, asphalt plant, concrete mixer machine, vibrator machine etc. The construction work will be performed section wise. Hence, it is anticipated that the overall impacts on noise and air quality will be moderate, temporary and limited within the subproject boundary. There are road side residents, and religious institution. Hence, anticipated impacts on noise and air quality will be significant for this particular section if measures do not take properly. The generated dust during construction activities may disturb the local people during construction period. The generated construction wastes, un-suitable materials and solid wastes may degrade the adjacent pond water if not properly collected and disposed and thrown in to the pond and ditch. Improper collection and disposal of the generated wastes materials also may degrade the quality of the surrounding environment and degrade the aesthetic value. The anticipated water logging and drainage congestion during construction period is minor because space is available at most sections of the road for drain-out the storm water. In addition, provision will be there for pumping out the storm water if required.

- Socio-economic impacts:

- Traffic congestion 			Very likely □  	Likely       □ √   	Unlikely     □

- Health and safety 			Significant □  	Moderate □√	Insignificant  □

- Impact on archaeological and historical   Significant □  	Moderate □  	Insignificant □√

- Employment generation 		Significant □ 	Moderate □ √  	Insignificant  □

Firstly, the proposed drain will be constructed.  The traffic congestion due to drain construction is comparatively less because it does not require closing of the entire road. The pedestrians, cycle rider, biker can use the road during drain construction. The local people will use existing connecting and diversion road during road construction work. Furthermore, to minimize traffic disturbances, construction work will be performed section wise. Hence, the subproject activities will not have any severe impact on the local traffic system due to diversify of the traffic in the connecting and other roads. The construction work will follow simple procedure with commonly used equipments. Hence, anticipated impact on health and safety is moderate. The construction work adjacent to the residents and religion institutes may cause safety risks to the public especially for school going kids. Therefore, construction activities adjacent to the Mosque should be performed within very short time considering safety issues/site fencing. There is no archaeological and historical site will be affected due to the subproject activities. The subproject has positive impact by generating work opportunities for the local people and supplying of the construction materials, equipments, food and other necessary stuffs to the campsite.

3)  Potential environmental impact during operational phase:

(d)  Ecological impacts:

- Potential impact on species of aquatic     Significant □         Moderate □√         Minor  □

(i.e., water) environment

The anticipated impact on the aquatic environment is insignificant due to operation of the road. Though, the drain is designed for storm water only. However, the storm water may carry washed-out materials, pollutants from any sources that may disturb the aquatic environment of the outfall. Hence, the anticipated impact on the aquatic environment due to operation of the drain is considered as moderate.

(e)  Physicochemical impacts:

- Potential air quality                 Improvement □       No-improvement      □          Deterioration  □√

- Noise level                              Improvement □       No-improvement     □           Deterioration  □√

- Drainage congestion    	         Improvement □√     Minor Improvement □           No Impact      □

- Risk of Water pollution            Significant     □        Moderate 	       □√         Minor              □

- Pollution from solid waste        Improvement □       No-improvement     □           Minor             □√

During operation phase due to road component, increased traffic volume may degrade the air quality and noise level due to black smoke emission and un-due use of hydraulic horn from vehicles. The new drain will minimize drainage congestion and water logging problem. The storm water to be discharged through the drain may degrade the water quality of the outfall if carry pollutants from any sources. The new covered drain will minimize throwing of the wastes materials in to the drain and spreading of the bad odor.

(f)  Socio-economic impacts:

- Traffic                                 Improvement □√	No-improvement □	Adverse □

- Safety 			    Improvement □√  	No-improvement □	Adverse □

- Employment generation     Significant     □	Moderate  □√		Minor    □

After completion, this road will improve the transport facilities. By providing uniform pavement, it will also enhance traffic safety and will enhance traffic operation. The new road will minimize the frequency of the maintenance for first three to five years after completion. After completion of the drain, the drain will enhance the existing drainage network.  The new drain will reduce water logging problem. By reducing environmental hazards caused by water logging problem, it will provide better environment to the community people.

3)  Summary of the Possible Environmental Impacts of the Subproject:

The ecological impact due to the subproject implementation is mainly due to tree felling. However, the ecological imbalance to be caused due to cut down of the trees will be compensated by the plantation of the trees.

The overall impact due to implementation of the subproject is not severe. The physicochemical components will be disturbed during the construction phase. However, the anticipated impacts will be mainly within the subproject boundary. The wastes generated due to the construction activities should be properly collected and disposed in a designated site. There is also some adverse impact at operational stage. Due to improved road, motorized traffic volume may increase. Thus, air quality and noise level may be deteriorated due to vehicles emission and horns by the vehicles. The storm water to be discharged through the drain may degrade the water quality of the outfall and adjacent water bodies if carry pollutants from any sources.

Safety concern is an important issue for both the construction and operation phases that should be considered properly. The socio-economic aspect mainly includes health and safety and employment generation. This subproject will have positive impacts in terms of the generation of the employment and business activities by supplying construction materials and equipment.

# Specific impact, mitigation AND enhancement measures

The likely impacts of the subproject are mainly caused by the activities required for the implementation of the subproject, and materials, resources and equipment to be used to perform the activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

The likely impacts of the subproject are mainly caused by the activities required for the implementation of the subproject, materials, resources and equipment to be used to perform the activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

## Dismantle Work, Excavation Work and Earth Work

The drain construction work consists of earth excavation, earth filling, back filling and cutting and removal of the unsuitable materials. These works lead dust blowing, improper disposal of the wastes, noise and vibration which may disturb the local people.

Mitigation Measures

- Careful operation of the hydraulic excavator if used for excavation and earth filling to avoid any undue disturbances to the roadside residents;

- Cover the exposed earth with mulch fabric;

- Quick disposal of the wastes to the selected dumping site at Naodhar beside Dhaka-Mymensingh Highway.

## Tree Felling and Ecological Impact

The subproject has adverse impacts on the ecological components due to felled down of the

297 numbers of trees and clearing of the roadside vegetation and bushes.

Mitigation Measures

- 450 nos. of trees will be planted to compensate the felled down trees-preferably local fruits, flowers, medicinal and ornamental trees- Mango, Jackfruit, Jam, Kathbadam, Chalta, Krisnachura, Bokul, Jarul, Polash, Kadom, Shimul, Neem, Arjun, Amloki, Horitoki, Bohera, Mahogany, Palm Tree, Chambal, Rain Tree, Shil koroi, Satim, Sishu (Preferably at both sides of the Fulbaria Road where space is available).

## Pollution from the Construction Materials

Dumping of the construction spoils, including accidental leakage of the oil, grease, and fuel in equipment yards is a significant hazard. Surface water and soil quality might be polluted from these contaminants. Even the people to be engaged for the construction activities might endanger the physical and human habitats of the area.

Mitigation Measures

- Safe transport, storage, and disposal of the construction materials, and the equipment have to be carried out in order to avoid the accidental spillage and loss;

- Fuels, lubricants, and other hazardous materials should store over raised platforms and  not directly on the ground;

- The playgrounds of the educational institutions should not use as a stockyard or work camp.

## Air Quality and Dust

During construction phase, air pollutants will be emitted from the equipment and construction vehicles are expected to remain low. Local residents in the vicinity of the work sites will be temporarily disturbed by the limited dust pollution. The overall impacts, however, are expected to remain low.

Mitigation Measures

- Water should be sprayed at the work site for dust control. Ensure sprinkle and cover stockpiles of loose materials (e.g., fine aggregates).

- Maintain adequate moisture content of soil and sand for transportation, compaction and  handling;

- Avoid use of dust generating equipment (which produce significant amount of particulate matter) far from the local residents;

- Ensure that all project vehicles are in good operating condition.

## Noise and Vibration

Noise and vibration caused by the equipment and movement of the construction vehicles may temporarily disturb nearby residents. In this subproject, sensitive areas like roadside houses, educational institutes and shops are likely to be affected from the noise, though the impacts are anticipated to be limited.

Mitigation Measures

- Transportation of the construction materials have to be carried during the scheduled times, and mainly during the day;

- If needed, all powered mechanical equipment and machineries will be fitted with noise abating gear such as mufflers for effective sound reduction.

## Water Quality

The water quality may deteriorate if the construction materials, construction wastes, effluent from work camps, and food waste are dumped in the roadside water body and low land.

Mitigation Measures

- Proper construction management including waste management as well as training of the operators and other workers should provide to avoid pollution of the water bodies;

- Construction waste will dispose properly (not in the ditch, water bodies or lowland), for which contractor will be responsible.

## Occupational Health and Safety

The most important risks associated with the construction activities are listed below:

- Exposure to the sunlight- workers are being exposed to the sun for long hours;

- Exposure to the high temperature, and humidity for a long time resulting in dehydration;

- Contact with the hazardous substances and wastes pose risks of the infections and diseases;

- Risk of the poor air quality due to the dust;

- Risk of the collision (traffic);

- Risks from head loads for carrying soil, construction materials and construction equipment;

- Risks of the using of the machineries in motion;

- Risk associated to the sudden bad weather working conditions.

General Requirements for the Workers’ Health and Safety

The key salient features of the general requirements for the workers’ health and safety stated are presented in Table 5.7.1.

Table 5.7.1:  General Requirements for the Workers Health and Safety

| Issues | Requirements |
| --- | --- |
| Health and Hygiene | - Cleanliness at the site premises and workers living places and at the Labor Shed; - Arrangement of the proper ventilation and temperature at the Labor Shed; - Protection against dust and furnace by using of the nose masks and covering of the head and body; - Proper disposal of the wastes and effluents; - Introduce waste bins for the solid waste management system. |
| Safety and First Aid Box | - Using of the personal protective equipment (helmet, gloves, goggles, nose mask, safety boots); - Precautions during work on or near machineries in motion; - Head loads are prohibited; - First aid facilities should be provided and maintained; - The first aid kit should include adhesive bandages, regular strength pain medication, gauze, and low grade disinfectant. |
| Compensation for Accidents at Work | - Contractors will bear medical treatment costs. If any sever accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation-(the amount of the compensation should be fixed considering the type of accidents). |
| Dust and Fumes | - For any dust, fumes, or other impurities likely to be injurious to the workers, effective measures shall be taken to prevent their accumulation and its inhalation by the workers. |
| Overcrowding | - No labor room should be overcrowded. |
| Latrines and Urinals | - Sufficient latrines shall be provided; - Latrines shall be maintained in clean and sanitary condition; - Latrines shall be adequately lighted and ventilated. |
| Disposal of Wastes and Effluents | - Proper disposal system for the solid waste and effluent is required; - Waste bins must be provided by the contractor at labor shed. |

## Impacts on Social Environment and Common Property Resources

Through comprehensive study, it is revealed that impacts are expected not to be severe and to be largely manageable. The following Table 5.8.1 presents impacts on socio-economic environment and common property resources.

Table 5.8.1: Impacts on Social Environment and Common Property Resources

| Social Components | Impacts on IECs | Impact Significance |
| --- | --- | --- |
| Community Perception | The local community people welcome this subproject and there is no visible objection from them. | Significant (+ve) |
| Employment and Business Opportunity | Community feels happy because the construction works will create work opportunity for the local people for the skilled and non-skilled labor. The subproject will create business opportunity for the equipment and materials suppliers’. | Significant (+ve) |
| Community Order and Security | This subproject activity does not create any severe security problems to the local community and community people. | Minor (-ve) |
| Possible damage to existing infrastructure and facilities | Degradation of the existing road infrastructure by the construction equipment/vehicles used in this subproject. | Minor (-ve) |
| New infrastructure and facilities | Construction of the new drains will improve the drainage facilities in the subproject area. | Moderate (+ve) |
| Labor Habitat | Most of the labors will stay at the Labor sheds which will have impacts on the environment relates to the generation of the solid wastes, effluent, and water consumption. | Moderate (-ve) |
| Health Care | Workers may suffer from the dehydration problems, respiratory problem, and other health hazards. | Minor (-ve) |
| Accident | In case of road accidents by the vehicles to be used for the transportation may have serious negative impact. | Significant (-ve) |

Mitigation Measures

- Conduct dissemination with the local community about the subproject details;

- Continue liaison with the community leaders in order to maintain the community support;

- Engage local contractor and local people as much as possible for positive perception of the local community;

- Follow traffic rules to avoid any accidents;

- Transportation and mobilization of the equipments and construction materials avoiding peak hours and scheduled time;

- Ensure first aid facilities and effective use of personal protective equipments where applicable.

## Labor Influx and Anticipated Impacts

The labor force and associated goods and services required for the construction of infrastructure civil works under this subproject cannot be fully supplied locally. The migration to and temporary settlement of laborers in the subproject, referred to as labor influx, carries an array of potentially positive and negative impacts in terms of demands on public infrastructure, utilities, housing and sustainable resource management and the strain on social dynamics.

Labor influx effects on host communities include positive impacts such as:

- The subproject activities will generate work opportunities for the local people and supplying of the construction materials, equipment, food and other necessary stuffs to the campsite;

- Improved infrastructure and public service access and availability whereby subproject investment catalyzes larger allocation of resources to a region, stimulating the development or expansion of infrastructure and public services.

Critical negative social risks include:

- Increase in criminal activity and alcohol and drug abuse, domestic violence, political attachment and violence, smuggling and robbery etc;

- Increase in gender-based violence, including eve teasing, sexual harassment etc;

- Increases in communicable diseases, including respiratory problems, diarrheal diseases, vector-borne diseases (e.g., malaria), and sexually transmitted infections (e.g., HIV/AIDS, syphilis, gonorrhea, hepatitis B);

- Conflicts arising from increased demand on existing infrastructure, services, and utilities, including transportation, health, education, water and sanitation, waste management, public utilities and community, religious, and recreational facilities and loss of land for access routes.

The general environmental impacts of labor influx include pressure on the natural resources such as using of the water, electricity, other fuel for cooking, loss of land for the labor establishment, depletion of the water supply, sewage and waste water generation, degradation of the air quality, waste generation,  increased demands on the local energy and resources and noise pollution effects. The number of local and migrated people involved in the subproject activities can be only identified in the construction phase. Hence, these specific impacts will be quantified during construction work and environmental assessment report will be modified accordingly. However, the following safeguard measures are recommended to avoid any risk of labor influx:

- Inform local people about the subproject activities;

- Liaison with the community leaders in order to get community support;

- Engage local people as much as possible to minimize workers from outsiders;

- Monitor workers attitude and behavioral matter;

- Monitor the workers movement for avoiding any unexpected social activities (robbery, crime, political attachment and conflicts, drugs abuse);

- Inform and use local administration to get support if needed;

- Inform local utilities service providers (such as for new electricity connection REB or any other department);

- Ensure effective use of natural resources such as water, electricity, fuel, wood etc.

## Impacts on Traffic Movement

The subproject activities will interrupt free movement of the pedestrians and non-motorized vehicles during construction phase. Since the volume of the motorized traffic is not huge for this road. Hence, the subproject activities do not have any severe impact on the motorized vehicles. However, the subproject activities have negative impact on the local traffic during construction phase due to inadequate connecting and diversion roads.

During construction phase, interruption of the traffic movement and impact on the local traffic system due to the subproject activities will be monitored closely. Then separate traffic management plan will be provided if required. However, the following safeguard measures are recommended to minimize the impacts associated to the traffic movement:

- Inform local people about the subproject activities;

- Inspire local people to use connecting and diversion roads;

- Ensure schedule deliveries of material/ equipment during off-peak hours;

- Place traffic sign/cautionary sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption;

- The place of construction works should be fenced off with fences if required and should be isolated from general public access and marked with signs to ensure safe movement.

# Environmental Management Plan (EMP)

The purpose of the Environmental Management Plan (EMP) is to ensure that the activities are undertaken in a responsible and non-detrimental manner. The EMP will guide the environmentally sound construction of the subproject and ensure efficient lines of communication between the PMU (LGED), PIU (Trishal Pourashava), DSM, and the contractors.

## Access to Information

The environmental assessment report should be translated into Bengali and disseminated locally. The copies of the report (both in English and Bengali) will be sent to all the concerned personnel responsible for subproject implementation. It will also be made available to the public. The final assessment report will also be uploaded in the LGED website and the World Bank website after approval.

## Grievance Redress Mechanism

The project-specific Grievance Redress Mechanism (GRM) has been established at Trishal Pourashava to receive, evaluate, and facilitate the solution of APs concerns, complaints, and grievances concerning the social and environmental performance of the subproject. The GRM aimed to provide a time-bound and transparent mechanism to voice and resolve social and environmental concerns linked to the subproject.

The grievance mechanism is related to resolve the risks and adverse impacts of the subproject. It addresses APs’ concerns and complaints promptly, using an understandable and transparent process that is also gender responsive, and culturally appropriate. It is readily accessible to all the segments of the affected people at no costs and without retribution. The mechanism should not impede access to the country’s judicial or administrative remedies. The affected people will be appropriately informed about the mechanism.

LGED has its own Grievance Redress Procedure (GRP), which it operates to address any dissatisfaction and complaints by the local people regarding its activities. This procedure is being applied to address any complaints or grievances through negotiations with the community leaders and representatives of APs during implementation of the MGSP.

### 6.2.1 Grievance Redress Committee (GRC)

The discussions and negotiations will be conducted by the Project Implementation Unit (PIU) of Trishal Pourashava, and will be involved the APs and Grievance Redress Committee (GRC) headed by the Pourashava Mayor. With the facilitation of DSM, the Pourashava Mayor nominated the GRC members and included representative from Government Agencies, local NGO, and Civil Society. The GRC has been formed and established at Trishal Pourashava. A complaints box will be provided in the Pourashava Office. The grievance response focal point is available at Trishal Pourashava for instant response to an aggrieved person. The Focal Point will collect the written complaints or suggestions from the box, and produce them to the GRC for hearing and resolution. The GRC members are as follows:

### Table 6.2.1.1: Grievance Redress Committee (GRC) Member List

| No. | GRC Member Name | GRC Designation | Self-Position |
| --- | --- | --- | --- |
| 1 | Md. A.B.M Anisuzzaman | Chairman | Mayor, Trishal Pourashava |
| 2 | Subbroto Tarafdar | Member Secretary | Executive Engineer, Trishal Pourashava |
| 3 | Md. Mozaharul Haque | Member | Upazila Project Implementation officer, Trishal Pourashava |
| 4 | Md. Rafiqul Islam | Member | Head Teacher, Trishal Nazrul Girls High School, Trishal, Mymensingh |
| 5 | Md. Zahirul Islam | Member | Executive Director, Sheba, Trishal Pourashava |
| 6 | ANM Shova Mia Akonda | Member | Civil Society Representative |
| 7 | Female Ward Councilor | Member | Related Female Ward Councilor (Reserved Seat) |
| 8 | Ward Councilor | Member | Related Ward Councilor |
| 9 | Md. Nazrul Islam | Focal Point | Secretary Trishal Pourashava |

### 6.2.2 Grievance Resolution Process

All complaints and suggestions will be received formally in the Pourashava Office by the GRC Member Secretary. A sample Grievance Redress Form has been prepared and sent to Trishal Pourashava prior to the implementation of the subproject.

An intake register will be maintained at the office of the Member Secretary. Member Secretary will record the details of the grievances in the intake register for documentation and ensure impartiality, fairness, and transparency. The intake register will have data and information columns including (i) Case no., (ii) Date of receipt, (iii) name, type of complaint, grievance, (iv) father’s name, husband’s name, (v) sex, (vi) complete address of the person raising the complaint, grievance, (vii) main objection (loss of land, property, or entitlement), (viii) detailed complaint story, (ix) expectation with documentary evidence and previous records of similar grievances, etc.

No GRC members are allowed to contact the aggrieved persons in advance. Rather, the concerned persons are informed to attend the formal hearings at an appointed date. The GRC committee will sit for hearing the complaints of the aggrieved persons. The GRC will record salient points presented by the aggrieved person and will examine documentary evidence submitted during informal hearings. A resolution register will be maintained by the Member Secretary at the Trishal Pourashava Office. The resolution register will contain (i) serial no., (ii) case no., (iii) name of complaint, (iv) complaint story and expectation, (v) date of hearing, (vi) date of field investigation (if any), (vii) results of hearing and field investigation, (viii) decision of GRC, (ix) progress (pending, solved) and (x) agreement or commitments. Closing register will keep records such as, (i) serial no., (ii) case no., (iii) name of complaint, (iv) decision and response to complaints, (v) mode and medium of communication, (vi) date of closing, (vi) confirmation of complainant's satisfaction, and (vii) management actions to avoid recurrence.

The GRC will decide within 30 days of receiving a complaint. There will also be an appeals procedure where, if a person is dissatisfied with the ruling of the GRC, he or she or a representative may attend their next meeting to present the case again. The committee will then reconsider the case in private, after which their decision is final. If the appellant is still not satisfied, he or she has the right to take the case to the public courts. LGED and Trishal Pourashava should also publish the outcome of cases on public notice boards. All costs involved in resolving the complaints (meetings, consultations, communication, and information dissemination) will be borne by Trishal Pourashava.

Based on consensus, this procedure will help to resolve issues or conflicts amicably and quickly, saving the aggrieved persons from having to resort to expensive, time consuming legal action. The procedure will however, not pre-empt a person’s right to go to the courts of law.

## Institutional Arrangement for the Safeguard Compliances

In the institutional arrangement procedure, Project Director, and Team Leader/ Deputy Team Leader will directly involve. The PD, TL would be supported by DSM Environmental Safeguard Specialist and Social Management Expert. The Pourashava Officials, especially engineers, would be responsible for supporting the construction supervision with the help of DSM consultants. The civil works contractors will implement the environmental mitigation measures.

The PMU (LGED), with the help of Environmental Safeguard Specialist and Social Management Expert, will submit the monthly and quarterly progress reports on Environmental and Social Compliances to World Bank.

PD-MGSP

TL/DTL-DSM

Environmental Safeguard Specialist-DSM

Social Management Specialist-DSM

Senior Municipal Engineer-DSM

Junior Environmental Specialist-DSM

Assistant Municipal Engineer-DSM

Social Management Officer-DSM

Pourashava Representative/ Focal Point/ Asst. Engineer

Contractor

Figure 6.3.1: Environmental and Social Management Team (Tentative)

## Capacity Building

A demonstration training program has already been conducted by PMU (LGED) and DSM Consultants at Comilla City Corporation on 16 May 2016. Similar capacity building workshop will also be conducted by PMU and DSM Consultants in the Narayanganj region for the capacity building. This workshop will be conducted by the PMU (LGED) and DSM Consultants. The participants from the Trishal Pourashava will attend this training program. Under this training program, PMU and DSM will organize an introductory course for the training and preparing the participants on: (i) Environmental Screening, (ii) EMP Implementation, including environmental monitoring requirements related to mitigation measures; and (iii) taking immediate action to remedy unexpected adverse impacts or ineffective mitigation measures found during the course of the implementation. The contractor should be also included in the training program to enhance the Environmental awareness and orientation among the workers before deploying to the work sites in order to achieve the expected standards. A detailed training manual has already been developed by the Environmental Safeguard Specialist and Social Management Specialist prior to the demonstration training program in Comilla Region. Hence, the existing manual will be reviewed and modification will be done as per requirement.

## Emergency Response and Disaster Management

Disaster management can be defined as the organization and management of
resources and responsibilities for dealing with all humanitarian aspects of emergencies, in particular the preparedness, response, and recovery to lessen the impact of disasters. Emergency Preparedness Planning (EPP) and Contingency Planning (CP) are the processes of disaster management plan for developing strategies, arrangements, and procedures to address the humanitarian needs of those adversely affected by the crisis. There are four main types of disasters, namely: Natural disasters, Environmental Emergencies, Complex Emergencies, and Pandemic Emergencies.

For MGSP activities, Trishal Pourashava would identify the immediate needs, prioritize the tasks, and identify resource requirements to address the humanitarian needs of those adversely affected by the crisis.

## Environmental Management Action Plan

The activity wise anticipated environmental impacts and corresponding mitigation measures have been outlined in Table 6.6.1 and Table 6.6.2.

Table 6.6.1: Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | - Generation of sewage and solid waste may cause water/ environmental pollution | - Construction of sanitary latrine considering 15 persons for one toilet at the labor shed and separate toilet for male and female; - Erection of “no litter” sign, provision of waste bins/cans (introduce separate waste bins for organic and inorganic wastes); - Ensure wastes (solid wastes and other forms of the waste) disposal at the dumping site at Naodhar beside Dhaka-Mymensingh Highway. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well being of the local people. |  |
| General construction works | - Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water, if needed; - Ensure provision for pumping of congested water, if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces regularly to reduce dust generation; - Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and handling; - Ensure contractor sprinkle and cover stockpiles of loose materials (e.g., fine aggregates); |  |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Place traffic sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption; - Inform the local people about the subproject activities. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment producing excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles. |  |
|  | - Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes into adjacent water bodies, low lands and soil. |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 450 nos. of timber, medicinal and beatification trees will be replanted to compensate the felling trees. |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accidents; - Provide PPEs (personal protective equipment) and ensure using of the PPEs by the workers. |  |
|  | - Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil and adjacent pond; - Collection and proper disposal of the spills. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures, such as tree plantation, and cautionary signs. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the BC road |  |  |  |
| Dismantle work/ Excavation/ Earth work/ | - Generation of solid and construction wastes due to the dismantle works; - Generation of loose soil due to the earth excavation work. | - Cover exposed earth works with fabric; - Disposal of soil and construction wastes at the dumping site at Naodhar beside Dhaka-Mymensingh Highway. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accident due to dismantle of the existing road | - Carefully operation of the hammer. |  |
| ISG (Improved Sub grade)/ Sand filling | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Setting up and operation of asphalt plant | - Possible degradation of air quality by the suspended particles and increase noise level from asphalt plant affecting nearby settlements; | - Locate plant away from residential settlements; - Consider use of emulsified bitumen. |  |
|  | - Possible water pollution by bitumen and solvents; | - Avoid spills and proper collection and disposal of the generated spills. |  |
|  | - Possible preparation of the bitumen in open air and using of charcoal and wood as fuel | - Strictly prohibit bitumen preparation in the open air and use of charcoal and wood as fuel. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the RCC road, RCC U type drain and cross drain, box culvert and road protection wall |  |  |  |
| Excavation and site clearing & grubbing work | - Generation of loose and clay soil due to the earth excavation work | - Disposal of soil and construction wastes at the dumping site at Naodhar beside Dhaka-Mymensingh Highway. - Cover exposed earth works with fabric. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accidents | - Carefully handle of the hydraulic excavator. |  |
|  | - Air pollution | - Regular maintenance of the equipment. |  |
| Precast RCC pile driving work | - Accident due to careless driving of pile. | - Carefully operate the pile driving equipment - Ensure the use of PPEs. |  |
| Sand filling/Back filling for RCC Road, RCC Drain, Box Culvert, and Road Protection Wall | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Cutting & welding of the reinforcement for RCC Road, RCC Drain, Box Culvert, and Road Protection Wall | - Noise Pollution | - Avoid using of rod cutter and wielding machine at night; |  |
|  | - Health and Safety | - Ensure use of the PPEs; - Availability and access to first-aid equipment and medical supplies. |  |
| RCC (reinforcement concrete) work for RCC Road, RCC Drain, Box Culvert, and Road Protection Wall | - Air pollution due to black emission | - Regular maintenance of the concrete mixer and vibrator machine. |  |
|  | - Noise pollution | - Avoid operation of the concrete mixer and vibrator machine at night; |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the street light |  |  |  |
| Setting up the pole and electrical connection | - Potential health and safety risks; | - Inform the local authority to switch off power during connection; - Ensure use of the PPEs. |  |
| Source of electricity and equipment | - Reduce of resource i.e. use of electricity; | - Provision of renewable energy (solar panel electrification) and use of environmental friendly equipment (LED bulb rather than CFL bulb). |  |

Table 6.6.2: Anticipated environmental Impacts during operation phase and corresponding mitigation and enhancement measures

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Operation of the road | - Increase traffic speed and accidents; - Increased traffic congestion due to movement of increased number of vehicles. | - Better traffic management. | Monitoring by Pourashava |
|  | - Increased air and noise pollution affecting surrounding areas. | - Better traffic management; - Conduct awareness program to minimize undue use of hydraulic horns. |  |
| Operation of RCC drain | - Pollution of downstream water body due to disposal of polluted water from the drain. | - Ensure installation of septic tank by the household people in all establishment; - Stop connecting sanitation facilities to storm drain directly. |  |
|  | - Possible backflow of water due to blockage in the drain. | - Proper maintenance and cleaning of the drain and outfall on regular basis. |  |
|  | - Possible degradation of the water quality and minimize blockage. | - Raising awareness among the beneficiaries, "Do not through solid waste into the water body and road side area’’. |  |
| Operation of RCC culvert | - Ecological impacts including destruction of aquatic habitat | - Preservation of aquatic habitats by restricting movements of people/equipment into them and preventing entry of sediments into these water body |  |
| Operation and maintenance for street light | - Accidents due to collapse of the arms, electric bulbs and pole | - Monthly checking and maintenance of the arms, switch box, electric bulbs if needed; - Provision of automatic shut-down the switch, lamps during thunder storm and other natural disasters. |  |

## 6.7 Environmental Monitoring Plan

Environmental Monitoring Plan for this subproject will help to evaluate the extent and severity of environmental impacts against the predicted impact and the performance of environmental protection measures. The following Table 6.7.1 has been prepared for the key environmental indicators.

Table 6.7.1: Matrix Table of Monitoring Plan (Visual observation during construction phase)

| Monitored Parameter/ Issues | Monitoring Method/ Key Aspects | Location of Monitoring | Period & Monitoring Frequency |
| --- | --- | --- | --- |
| Safety orientation and training of workers | Frequency of training & orientation of workers for safety | Subproject site | - Once in a month - Reporting: Once in a month |
| Personal Protective Equipment | Ensure every single person involved in the activities wear and use safety equipment | Subproject site | - Daily - Reporting: Once in a month |
| Worker’s health | Monitoring process of worker’s health | Subproject site | - Daily - Reporting: Once in a month |
| Sanitation & drinking water facility to the workers | Availability of safe drinking water and sanitation to the workers | Subproject site | - Daily - Reporting: Once in a month |
| Incident record and reporting | Documented record of all incident, accident, its remedial process | Subproject site | - Daily - Reporting: Once in a month |
| Site security/ Fencing at the site | Isolation of site from general access by fencing, restriction of the un-authorized entry in the site. | Subproject site | - Daily - Reporting: Once in a month |
| Bulletin/ announcement boards/ prohibition signs | Visible in good condition or not | Subproject site | - Daily - Reporting: Once in a month |
| Equipment /vehicles | Switched-off diesel engines when not in use; Search any possible leakage; Fuelling. | Subproject site | - Daily - Reporting: Once in a month |
| Dust | Dust is visible or not | Subproject site | - Daily - Reporting: Once in a month |
| Oily waste generation and disposal | Quantity of oily waste, storage and disposal | Subproject site | - Daily - Reporting: Once in a week |
| Solid waste generation | Quantity of solid wastes and disposal | Subproject site | - Daily - Reporting: Once in a month |
| Gender equity | No discrimination regarding payment | Subproject site | - Daily - Reporting: Once in a month |
| Child labor | No child will be engaged in the activities | Subproject site | - Daily - Reporting: Once in a month |
| Handling of hazardous materials | Fuelling, storage, operation | Subproject site | - Daily - Reporting: Once in a month |

In addition, a comprehensive Environmental Compliance Monitoring checklist has been enclosed in Appendix-I.

## 6.8 Cost of Environmental Enhancement Works in BOQ

Considering the environmental impacts and their mitigation measures for this subproject, several items are included in the BOQ to address these issues. The estimated cost for implementing the EMP is elaborated in Table 6.8.1.

Table 6.8.1: Environmental Management Budget in Bill of Quantity

| Item No. | Description of the Works | Unit Rate (BDT) | Qty (Number) | Amount (BDT) |
| --- | --- | --- | --- | --- |
| eme-1 | a) Dust suppression measures by water spraying throughout the construction period in and around the subproject sites, uncovered aggregates and loose materials such as stockpiles of the sand, excavated earth etc. | Lumpsum |  | 10,000.00 |
|  | b) Dust suppression measures for BC road by water sprinkling (it includes water sprinkling for overall BC road construction works that includes-ISG, WBM, ASG and sub-grade) | Lumpsum |  | 15,000.00 |
| eme-5 | Prevention of the spillage and leakage of the polluting materials (Detailed procedure will be given in the EMP) | Lumpsum |  | 5,000.00 |
| eme-6 | Campsite wastes disposal facility during the construction period (collection, transportation and dumping of the wastes at designated dumping site at Naodhar beside Dhaka-Mymensingh Highway): 2 nos. (1 no. for the organic wastes and 1 no. for the inorganic wastes disposal facility) | 20,000.00 | 2.00 | 40,000.00 |
| eme-7 | Campsite water supply facilities: Preferably 1 no. of tube well at the labor campsite (Depending on the site condition, DSM consultant will assist the contractor for selecting the option) | 20,000.00 | 1.00 | 20,000.00 |
| eme-8 | Campsite sanitation facilities: 2 nos. of the toilets preferably sanitary toilets at the labor campsite (1 no. for women and 1 no. for men) | 20,000.00 | 2.00 | 40,000.00 |
| eme-9 | a) Providing safety gear packages like hand gloves, spectacles for eye protection, ear plug, helmets, masks, visible jacket, safety shoes for at least 35 persons (25 workers and 10 visitors) | 3,000.00 | 35.00 | 105,000.00 |
|  | b) One first aid box with necessary accessories (contractor is responsible for providing necessary medicines, saline as per requirement during construction period) | 2,500.00 | 1.00 | 2,500.00 |
| eme-11 | Tree plantation to compensate the felled down trees and enhance the ecological condition in the subproject area- preferably local fruits, flowers, medicinal and ornamental trees- Mango, Jackfruit, Jam, Kathbadam, Chalta, Krisnachura, Bokul, Jarul, Polash, Kadom, Shimul, Neem, Arjun, Amloki, Horitoki, Bohera, Mahogany, Palm Tree, Chambal, Rain Tree, Shil koroi, Satim, Sishu (including protection, fencing and conservation during project defect liability period): Preferably at both sides of the Fulbaria Road where space is available -450 nos. of the trees (fencing as per LGED rate schedule 5.26.14) (Tree plantation and fencing detailed will be given in the EMP. Contractors will also be instructed by the consultant and PIU prior to the tree plantation work) | 750.00 | 450.00 | 337,500.00 |
| eme-14 | Cautionary signs-5 nos. (Detailed specifications will be given in the EMP) | 2,500.00 | 5.00 | 12,500.00 |
|  | Total |  |  | 587,500.00 |

After approval to revise the cost estimate has lengthy complex procedure. Hence, as per project EMF, PMU suggestion and experience from other LGED projects, adequate budget has been allocated for the environmental management for the mitigation and enhancement measures. The subproject activities have temporary and localized negative impact on noise level during construction phase due to movement of the subproject vehicles and equipments. Hence, budget includes analytical monitoring for noise level. For solid waste management adequate waste bins will be installed. The budget for labor shed, and site office construction is included in the civil works items. Therefore, it is not included in the environmental budget. It should be noted that the contractor will be paid as per actual work done.

## 6.9 Environmental Codes of Practice

This section identifies and specifies environmental management guidelines and practices to be followed by the contractor for sustainable management of all environmental issues. The Contractor shall carry out the subproject related activities as specified in contract agreement. Trishal Pourashava shall ensure that contractor take due responsibility to mitigate those negative impacts. Environmental awareness creation, particularly about the direct construction impacts and for the health, pollution and safety issues will be Contractor’s responsibility. Clauses that may be incorporated in the tender documents are:

- ECoP-1 (Overall Environmental Protection): Contractor shall take all steps to protect environment and avoid causing all types of public nuisances during implementation;

- ECoP-2 (Labor shed Management): Contractor shall maintain the work camp and construction sites in clean and tidy conditions and shall ensure standard facilities;

- ECop-3 (Workforce Environment): Contractor shall engage local people as much as possible where applicable and ensure prohibition of the child labor (less than 18 years) and aged labor (more than 65 years) in heavy works;

- ECoP-4 (Waste Management): Contractor shall be responsible for the safe transportation and disposal of the wastes generated due to the subproject activities;

- ECoP- 5 (Workers Health and Safety): Contractor shall be responsible for providing personal protective equipments and first aid facilities as per requirements;

- ECoP-6 (Compensation for Accidents):  Contractor shall bear medical treatment costs for any accidents. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation- (the amount of the compensation should be fixed considering the type of accidents);

- ECoP-7 (Implementation of the Mitigation Measures): Contractor shall responsible for the implementation of the mitigation measures mentioned in the EMP;

- ECoP-8 (Spill Prevention, Fuels and Hazardous Substances Management): Contractor shall take preventive measures for spill prevention and fuels and hazardous substances management;

- ECoP-9 (Restoration of the Facilities): The contractor on completion of the contract shall remove the equipment, surplus materials, and rubbish and temporary structures of all types and shall leave sites in clean condition to the satisfaction of Pourashava and local people.

# 7   PUBLIC Consultation and participation

- 7.1 Methodology

In the context of preparing the Environmental Assessment (EA), participatory public consultation was conducted for this subproject. The Pourashava Mayor, Officials, Engineers and local individuals as well as LGED and Consultant participants participated. Informal Focus Group Discussions (FGD) and a formal CIP were conducted involving the participants. In addition, walk-through informal group consultations and individual interviews were also held. The local communities were informed about the subproject intervention including their benefits. Suggestions made by the participants were listed and incorporated in the EMP accordingly.

### Photographs 7.1.1: DSM Consultant Team with Pourashava Representatives inspected the proposed road

Photographs 7.1.2: Consultation Meeting at subproject sites with Pourashava Officials and Consultants Participants

- 7.2 Issues Raised by the Participants

The participants raised the issues related to the infrastructure development of Trishal Pourashava. The anticipated environmental and social impacts due to construction works have also been raised and discussed. The participants also discussed about the potential benefit from the subproject.

- 7.3 Feedback, Suggestions, and Recommendations of the Participants

The participants were presented with feedback, suggestions, and recommendations listed below:

- The FGD results confirmed that an improved communication network is needed for future development of Trishal Pourashava.;

- Most of the participants expressed that the number of subproject that have been selected for each financial year is insufficient;

- The participants stated that the public water supply facilities, sanitation facilities, and access road are not adequate;

- The participants stated that construction works should be scheduled properly;

- Local people also showed expectation for the increased opportunities for employment for unskilled or semi-skilled labor in the construction work;

- Local people expressed that construction of new bridges will assist for the development of the peripheral road network in the Trishal Pourashava.

# 8   Conclusions and Recommendations

This study assists to understand the initial environmental impacts due to the subproject as well as to formulate the applicable mitigation and monitoring plans. Based on the environmental assessment, all possible environment aspects have been adequately assessed and necessary mitigation measures have been formulated to minimize the adverse impacts.

The overall conclusion is that if the mitigation, compensation, and enhancement measures are entirely implemented, there will be no significant negative environmental impacts as a result of location, design, construction, and or operation of the proposed subproject.

The conclusions of the environmental assessment can be summarized as follows:

- The short-term negative impacts that may come such as air quality, noise, solid waste, occupational health and safety will be minimized through the mitigation measures;

- The subproject will create employment for the workforce who live in the vicinity of the construction site and will provide them a short-term economic gain.

A few key recommendations are outlined below:

- All mitigation, compensation, and enhancement measures proposed in this report should be followed by the concern authorities for implementing the subproject;

- The environmental management and monitoring plan proposed in this report also needs to be followed;

- Regular monitoring by Pourashava, PMU (LGED) and DSM consultants is needed;

- The contractor will ensure using of the PPEs by the workers where applicable;

- The contractor is responsible for the proper disposal of the generated wastes materials from the subproject sites and campsite;

- The EMP shall be made binding on contractor operating the site, and will be included in the contractual clauses;

- Due to changing circumstances during the construction phase if any, there might be minor adaptation needed for environmental, health and safety issues.

# 9 REFERENCE

### Trishal Pourashava Master Plan: 2011-2031 Urban Area Plan

### Trishal Pourashava at a glance

### Environmental Assessment Volume 1: Overall Environmental Assessment [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) and Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh.

### Environmental Assessment Volume 2: Environmental Management Framework (EMF) [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) and Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh.

Appendix

Appendix 1: Environmental Monitoring Checklist

Local Government Engineering Department

Municipal Governance Services Project (MGSP)

Environmental Compliance Monitoring Form

Part A: General Subproject Information

| Subproject Name | 1) Rehabilitation of Fulbaria Road from Gohata Bazar to End of Pourashava Boundary with Link Dhanikhola Road (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m), Total Length = 3210 m under Trishal Pourashava. 2) Construction of RCC Drain at Fulbari Road from Gohata Bazar to End of Pourashava Boundary under Trishal Pourashava (Ch. 0+00 to 0+437 m). 3) Construction of 2 nos of Box Culvert (2.00 m* 2.00 m) at Ch. 1215 m & Ch. 2670 m on Fulbaria Road from Gohata Bazar to End of Pourashava. 4) Supplying, fitting & fixing of Street Lights from Gohata Bazar to End of Pourashava under Trishal Pourashava (Ch. 0+00 to 3+110 m & 0+0 to 0+ 100 m), Total Length = 3210 m |
| --- | --- |
| Package No. | MGSP/TRI/2017-18/W-07 |
| ULB Name | Trishal Pourashava |
| Approved Estimated Cost in BDT |  |
| Contract Amount in BDT |  |
| Contractor Name |  |
| Date of Commencement |  |
| Target Date of Completion |  |
| Physical Progress (%) |  |
| Financial Progress (%) |  |
| Person Responsible (PIU) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Person Responsible (DSM) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Form Completed by (Name, Designation &Phone) |  |
| Overall Environmental Description of the Subproject |  |

Part B: Design, Preparation, and Legal Requirements

| Environmental Concerns (PMU, PIU & DSM) (Name, Designation & Phone) | PMU- |  |
| --- | --- | --- |
|  | PIU- |  |
|  | DSM- |  |
| Subproject Category | DoE-BD- | WB- |
| Environmental Clearance Received? | Yes | No |
| EA Required? | Yes | No |
| EA Prepared and Delivered? | Yes | No |
| EMP Prepared & Delivered Separately? | Yes | No |
| Items and Cost of EMP Implementation Included in the Contract? | Yes | No |
| EMP Included in the Procurement Documents? | Yes | No |
| Inspection Schedule/Last Inspection/Monitoring by PMU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by PIU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by DSM Environmental Safeguard Specialist/Jr. Environmental Specialist | Date- |  |
|  | Key findings- |  |

Part C: Key Environmental Impacts

| The Subproject Results in any of the following Impacts? | Yes | No | NA | If yes, is the impact (give observation) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Significant | Moderate | Minor |
| Felling of the trees |  |  |  |  |  |  |
| Clearing of the vegetation that increase the risk of increased soil degradation or erosion |  |  |  |  |  |  |
| Disturbance of the terrestrial and or aquatic specifies |  |  |  |  |  |  |
| Noise pollution |  |  |  |  |  |  |
| Air pollution |  |  |  |  |  |  |
| Adverse effects on the quantity or quality of the surface water or groundwater |  |  |  |  |  |  |
| Production or increase the production of the solid waste |  |  |  |  |  |  |
| Drainage congestion |  |  |  |  |  |  |
| Water logging that increases the risk of the water related diseases |  |  |  |  |  |  |
| Traffic congestion |  |  |  |  |  |  |
| Public safety |  |  |  |  |  |  |

Part D: Work Place Environment and Gender Equity

| The Subproject Results in any of the following Impacts? | Yes | No | NA | Observations |
| --- | --- | --- | --- | --- |
| Does the contractor pay to the workers regularly? |  |  |  |  |
| Is there any discrepancy between the male and female workers regarding the wages or salary for the same works? |  |  |  |  |
| Is the contractor complying with the GOB labor law concerning the hiring of the workers? |  |  |  |  |
| Does the contractor engage women labors and does the project have suitable works for them? |  |  |  |  |
| Does the contractor engage child labor (less than 18 years) and aged people (more than 65 years old)? |  |  |  |  |
| Does the contractor force to the workers for the completion of the works? |  |  |  |  |
| Do the workers involve with the political activities, crime, drugs addiction and other forms of unwanted activities? |  |  |  |  |
| Are construction camps adequately equipped with water supply, sanitary toilets, washing facilities and facilities for waste collection and storage? |  |  |  |  |
| Has separate sanitation facilities been provided for women at work camps and the construction site? |  |  |  |  |

Part E: Potential Impacts, Mitigation Measures, and Monitoring Indicator Mentioned in the EMP

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | - Generation of sewage and solid waste may cause water/ environmental pollution | - Construction of sanitary latrine considering 15 persons for one toilet at the labor shed and separate toilet for male and female; - Erection of “no litter” sign, provision of waste bins/cans (introduce separate waste bins for organic and inorganic wastes); - Ensure wastes (solid wastes and other forms of the waste) disposal at the dumping site at Naodhar beside Dhaka-Mymensingh Highway. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well being of the local people. |  |
| General construction works | - Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water, if needed; - Ensure provision for pumping of congested water, if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces regularly to reduce dust generation; - Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and handling; - Ensure contractor sprinkle and cover stockpiles of loose materials (e.g., fine aggregates); |  |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Place traffic sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption; - Inform the local people about the subproject activities. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment producing excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles. |  |
|  | - Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes into adjacent water bodies, low lands and soil. |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 450 nos. of timber, medicinal and beatification trees will be replanted to compensate the felling trees. |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accidents; - Provide PPEs (personal protective equipment) and ensure using of the PPEs by the workers. |  |
|  | - Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil and adjacent pond; - Collection and proper disposal of the spills. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures, such as tree plantation, and cautionary signs. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the BC road |  |  |  |
| Dismantle work/ Excavation/ Earth work/ | - Generation of solid and construction wastes due to the dismantle works; - Generation of loose soil due to the earth excavation work. | - Cover exposed earth works with fabric; - Disposal of soil and construction wastes at the dumping site at Naodhar beside Dhaka-Mymensingh Highway. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accident due to dismantle of the existing road | - Carefully operation of the hammer. |  |
| ISG (Improved Sub grade)/ Sand filling | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Setting up and operation of asphalt plant | - Possible degradation of air quality by the suspended particles and increase noise level from asphalt plant affecting nearby settlements; | - Locate plant away from residential settlements; - Consider use of emulsified bitumen. |  |
|  | - Possible water pollution by bitumen and solvents; | - Avoid spills and proper collection and disposal of the generated spills. |  |
|  | - Possible preparation of the bitumen in open air and using of charcoal and wood as fuel | - Strictly prohibit bitumen preparation in the open air and use of charcoal and wood as fuel. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the RCC road, RCC U type drain and cross drain, box culvert and road protection wall |  |  |  |
| Excavation and site clearing & grubbing work | - Generation of loose and clay soil due to the earth excavation work | - Disposal of soil and construction wastes at the dumping site at Naodhar beside Dhaka-Mymensingh Highway. - Cover exposed earth works with fabric. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accidents | - Carefully handle of the hydraulic excavator. |  |
|  | - Air pollution | - Regular maintenance of the equipment. |  |
| Precast RCC pile driving work | - Accident due to careless driving of pile. | - Carefully operate the pile driving equipment - Ensure the use of PPEs. |  |
| Sand filling/Back filling for RCC Road, RCC Drain, Box Culvert, and Road Protection Wall | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Cutting & welding of the reinforcement for RCC Road, RCC Drain, Box Culvert, and Road Protection Wall | - Noise Pollution | - Avoid using of rod cutter and wielding machine at night; |  |
|  | - Health and Safety | - Ensure use of the PPEs; - Availability and access to first-aid equipment and medical supplies. |  |
| RCC (reinforcement concrete) work for RCC Road, RCC Drain, Box Culvert, and Road Protection Wall | - Air pollution due to black emission | - Regular maintenance of the concrete mixer and vibrator machine. |  |
|  | - Noise pollution | - Avoid operation of the concrete mixer and vibrator machine at night; |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the street light |  |  |  |
| Setting up the pole and electrical connection | - Potential health and safety risks; | - Inform the local authority to switch off power during connection; - Ensure use of the PPEs. |  |
| Source of electricity and equipment | - Reduce of resource i.e. use of electricity; | - Provision of renewable energy (solar panel electrification) and use of environmental friendly equipment (LED bulb rather than CFL bulb). |  |

Prepared by-                                                                                         Signature-                                                                                              Date-

Copies to 1.  PIU

2. SME

3. AME

4. Jr. ES