| GOVERNMENT OF THE PEOPLE’S REPUBLIC OF BANGLADESH |  |
| --- | --- |
| Local Government Engineering Department (LGED) Local Government Division Ministry of Local Government, Rural Development and Cooperatives |  |
| # Environmental Assessment (EA) REPORT Name of the Subproject: Construction of BC Road from DC Office Police Box to Atong via Pakar Matha (Ch. 0 + 00 to 4+ 192 m) and from Palong School to Kanar Bazaar (Ch. 0 + 00 to 2130 m) with Link Road ( Ch. 0+ 00 to 0+748 m. Package No: MGSP/SHA/2017-18/W-06 Shariatpur Pourashava, Shariatpur Municipal Governance and Services Project (MGSP) Design, Supervision, and Management (DSM) Consultancy Services |  |
| Joint Venture of Hifab International AB, Sweden AQUA Consultant and Associates Ltd., Bangladesh |  |

TABLE OF CONTENTS

|  |  |  |  | Page |
| --- | --- | --- | --- | --- |
| 1.0 |  | INTRODUCTION |  | 1-3 |
|  | 1.1 | Subproject Background |  | 1 |
|  | 1.2 | Objectives of the Study |  | 2 |
|  | 1.3 | Scope and Methodology of the Study |  | 2 |
| 2.0 |  | SUBPROJECT DESCRIPTION |  | 4-11 |
|  | 2.1 | The Study Area |  | 4 |
|  | 2.2 | Current Situation, Proposed Intervention and Need for the Subproject |  | 8 |
|  | 2.3 | Envisaged Subprojects Activities and Implementation Process |  | 10 |
|  | 2.4 | ## Category of the Subproject |  | 11 |
|  | 2.5 | ## Subproject Schedule |  | 11 |
| 3.0 |  | BASELINE ANALYSIS OF ENVIRONMENTAL CONDITION |  | 12-21 |
|  | 3.1 | Physical Environment |  | 12 |
|  | 3.2 | Biotic Environment |  | 20 |
|  | 3.3 | Socio-economic and Socio-cultural Environment |  | 20 |
| 4.0 |  | ENVIRONMENTAL SCREENING |  | 22-24 |
| 5.0 |  | SPECIFIC IMPACT, MITIGATION AND ENHANCEMENT MEASURES |  | 25-31 |
|  | 5.1 | Site Clearing, Excavation and Earth Work |  | 25 |
|  | 5.2 | Ecological Impact and Enhancement Work |  | 25 |
|  | 5.3 | Pollution from the Storage of Construction Materials |  | 26 |
|  | 5.4 | Air Quality and Dust |  | 26 |
|  | 5.5 | Noise and Vibration |  | 27 |
|  | 5.6 | Water Quality |  | 27 |
|  | 5.7 | Occupational Health and Safety |  | 27 |
|  | 5.8 | Impacts on Social Environment and Common Property Resources |  | 29 |
|  | 5.9 | Labor Influx and Anticipated Impacts |  | 30 |
|  | 5.10 | Impacts on Traffic Movement |  | 31 |
| 6.0 |  | ENVIRONMENTAL MANAGEMENT PLAN |  | 32-44 |
|  | 6.1 | Access to Information |  | 32 |
|  | 6.2 | Grievance Redress Mechanism |  | 32 |
|  |  | 6.2.1 | Grievance Redress Committee (GRC) | 32 |
|  |  | 6.2.2 | Grievance Resolution Process | 33 |
|  | 6.3 | Institutional Arrangement for Safeguard Compliances |  | 34 |
|  | 6.4 | Capacity Building |  | 35 |
|  | 6.5 | Emergency Response and Disaster Management |  | 35 |
|  | 6.6 | Environmental Management Action Plan |  | 36 |
|  | 6.7 | Environmental Monitoring Plan |  | 41 |
|  | 6.8 | Environmental Management Budget |  | 42 |
|  | 6.9 | Environmental Codes of Practice |  | 43 |
| 7.0 |  | PUBLIC CONSULTATION AND PARTICIPATION |  | 45-47 |
|  | 7.1 | Methodology |  | 45 |
|  | 7.2 | Issues Raised by the Participants |  | 46 |
|  | 7.3 | Feedback, Suggestions and Recommendations of the Participants |  | 46 |
| 8.0 |  | CONCLUSIONS AND RECOMMENDATIONS |  | 48 |
| 9.0 |  | # Reference |  | 49 |
| APPENDIX |  |  |  |  |
|  |  | Appendix I: Environmental Monitoring Checklist |  | 50 |

# LIST OF TABLES

| - Table 3.1.1 | - Major Environmental and Infrastructural Features |  |
| --- | --- | --- |
| - Table 3.1.2 | Temperature, Rainfall and Humidity during 2008-2011 |  |
| - Table 5.7.1 | - General Requirements for the Workers Health and Safety |  |
| - Table 5.8.1 | - Impacts on Social Environment and Common Property Resources |  |
| - Table 6.2.1.1 | - List of GRC Committee Members |  |
| - Table 6.6.1 | - Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures |  |
| - Table 6.6.2 | - Anticipated Environmental Impacts during Operational Phase and Corresponding Mitigation and Enhancement Measures |  |
| - Table 6.7.1 | - Matrix Table of Monitoring Plan (Visual observation during construction phase) |  |
| - Table 6.8.1 | - Environmental Management Budget in Bill of Quantity |  |

# LIST OF FIGURES

| Figure 2.1.1 | Location Map of the DC Office Road |  |
| --- | --- | --- |
| Figure 2.1.2 | Location Map of the Kanar Bazaar Road |  |
| Figure 2.1.3 | Lay-Out Plan of the DC Office Road |  |
| Figure 2.1.4 | Lay-Out Plan of Kanar Bazaar Road |  |
| Figure 2.1.5 | Topographic Features of DC Office Road |  |
| Figure 2.1.6 | Topographic Features of the Kanar Bazaar Road |  |
| Figure 3.1.1 | Location of Shariatpur in Earthquake Zone Map of Bangladesh |  |
| Figure 3.1.2 | Location of Shariatpur Pourashava in Flood Zone Map of Bangladesh |  |

# LIST OF PHOTOGRAPHS

| Photographs 2.2.1 | Current Situation of the DC Office Road |  |
| --- | --- | --- |
| Photographs 2.2.2 | Current Situation of the Kanar Bazaar Road |  |
| Photographs 7.1.1 | ### DSM Consultant Team with Pourashava Representatives inspected the proposed roads |  |
| Photographs 7.1.2 | ### Consultation Meeting at subproject sites with Pourashava Officials and Consultants Participants |  |

# ABBREVIATIONS

| AP persons (AP’s) | Affected Person |
| --- | --- |
| BDT | Bangladeshi Taka |
| BOQ | Bill of Quantity |
| CC | Cement Concrete |
| CIP | Capital Investment Plan |
| CP | Contingency Planning |
| DSM | Design, Supervision, and Management |
| EA | Environmental Assessment |
| ECR | Environmental Conservation Rules |
| EMP | Environmental Management Plan |
| EPP | Emergency Preparedness Planning |
| ES | Environmental Screening |
| FGD | Focal Group Discussion |
| GoB | Government of Bangladesh |
| GRC | Grievance Redress Committee |
| GRM | Grievance Redress Mechanism |
| GRP | Grievance Redress Procedure |
| LGED | Local Government Engineering Department |
| MGSP | Municipal Governance and Services Project |
| PD | Project Director |
| PMU | Project Management Unit |
| RCC | Reinforcement Cement Concrete |
| RP | Relevant Reports |
| TL | Team Leader |
| WB | World Bank |
| XEN | Executive Engineer |

# 1 INTRODUCTION

## 1.1 Subproject Background

Shariatpur Pourashava is situated at Shariatpur District. It is established as a Pourashava in the year of 1985. This is A type Pourashava with present population of 49535 (BBS 2011). The Pourashava covers an area of 24.75 sq. km. The Pourashava has Poura Pucca Road 45 km, Poura Earthen Road 60 km. (Source: Shariatpur Pourashava)

Urbanization is considered to be closely and positively associated process, with complex and mutually reinforcing links. In sustained economic growth, it is essential to develop urban areas. Urban centers are the places of agglomeration of economic activities, important hubs of production, processing, innovation and employment. In recent days, there has been a trend of growing importance of urban areas. This subproject emphasizes on the planning to address the problems and to guide future planning of Shariatpur Pourashava.

This subproject includes the following components: BC Road and Road Protection Wall. The significant features of the subproject are mentioned below:

| Name of the Subproject: | (1) Construction of Road starting from DC Office Police Box to Atong via Pakar Matha (Ch. 0 + 00 to 4+ 192) by BC under Shariatpur Pourashava, Shariatpur; (2) Construction of Bituminous Carpeting Road from Palong School to Kanar Bazaar (Ch. 0 + 00 to 2130 m), Link Road (Ch. 0+ 00 to 0+748 m) under Shariatpur Pourashava, Shariatpur. |
| --- | --- |
| Package No.: | MGSP/SHA/2017-18/W-06 |
| District Name: | Shariatpur |
| ULB Name: | Shariatpur Pourashava |
| Jurisdiction Area: | # Ward number 01, 02, 04, 05, and 06 |
| Structural Design Option: | # BC Road and Road Protection Wall |
| Wards Population: | # About 26, 780 as per information by the Pourashava |
| Tribal People: | # No tribal people settlement found in the subproject area |
| Land Acquisition: | # No private land acquisition is needed |
| Estimated Cost: | # 92.286 Million BDT |
| Subproject Duration: | # 12 Months |
| Tentative Start Date: | # 16 September 2018 |
| # Tentative Completion Date | # 15 September 2019 |

## Objectives of the Study

The general objective of this study is to determine the major environmental impacts succeeding from execution of the subproject and to recommend mitigation measures to avoid or reduce adverse environmental impacts and to enhance positive impacts. The specific objectives include:

- To assess the existing environmental conditions of the subproject site and its influence area;

- To identify and assess impacts resulting from the subproject during its construction phase and operation phase;

- To develop a environmental management plan with recommendations for mitigating impacts and enhance positive impacts;

- To summarize environmental monitoring requirements.

## Scope and Methodology of the Study

The subproject appraisal is included environmental screening for all the subproject components. According to that screening, environmental assessment is required to fulfill the regulatory requirement for BC Road and Road Protection Wall. Hence, this environmental assessment is prepared to meet the regularity requirement of this subproject. The study methodology covered the following activities:

- Desktop Study;

- Field Investigations and Data Collection;

- Data Analysis and Report Writing.

Desktop Study

The desktop study involved:

- Initial meetings with client, stakeholders to discuss the proposed subproject;

- Collection and review of the baseline data, maps, reports and other relevant information;

- Review of the relevant existing legislation, regulation and policies;

- Understand the anticipated technical processes that may affect the environment.

Field Investigation and Data Collection

A team of the consultants made a field investigation to the proposed site. Field investigations involved mainly site walks at the subproject areas for identification of the environmental components to be affected due to the subproject intervention.

The following key tasks were performed during the field visit:

- Taking photographs of the significant aspects to assist in describing the baseline environmental conditions of the subproject area;

- Interviews with representatives of the relevant Pourashava officials and beneficiary and affected people within the subproject influence zone;

- Obtaining relevant documents from Pourashava and local people within the subproject influence zone;

- Verifying information and data collected during the desktop study and to collect new information.

Data Analysis and Report Writing

The data and information collected from all the sources (literature review, secondary and primary data, public consultation) were analyzed to describe the existing environmental setting of the subproject area, to identify the potential positive and negative impacts of the proposed subproject, as well as to provide preliminary suggestions for mitigation measures. Finally, this environmental assessment report has been prepared.

# SUBPROJECT DESCRIPTION

## The Study Area

The subproject site is situated within the jurisdiction area of the Ward no. 01, 02, 04, 05, and 06 of Shariatpur Pourashava. The starting point of DC Office Road is located at 23°12'19.95"N (Latitude) and 90°20'43.30"E (Longitude). The end point of this road is located at 23°11'48.71"N (Latitude) and 90°22'45.47"E (Longitude). The starting point of Kanar Bazaar Road is located at 23°13'14.05"N (Latitude) and 90°24'0.87"E (Longitude).The end point of this road is located at 23°12'57.26"N (Latitude) and 90°22'32.46"E (Longitude). The location map, lay-out plans, and topographic features of the subproject are shown in Figure 2.1.1, Figure 2.1.2, Figure 2.1.3, Figure 2.1.4, Figure 2.1.5, and Figure 2.1.6.

Figure 2.1.1: Location Map of the DC Office Road

Figure 2.1.2: Location Map of the Kanar Bazaar Road

Figure 2.1.3: Lay-Out Plan of the DC Office Road

Figure 2.1.4: Lay-Out Plan of the Kanar Bazaar Road

Figure 2.1.5: Topographic Features of the DC Office Road

Figure 2.1.6: Topographical Features of the of the Kanar Bazaar Road

## Current Situation, Proposed Intervention and Need for the Subproject

At DC Office Road from Ch. 0 to 4192 m, the existing BC pavement will be demolished and replaced by the new BC pavement. Presently, the existing road is partially damaged in different sections which hamper the normal traffic operation. The existing road is also narrow (average pavement width is 4 m (Ch.0 - 830 m) and 3 m (Ch 830 m – Ch. 4192 m). From the site investigation, it is revealed that damaged and narrow road interrupts normal traffic operation. Hence, widening and road improvement is necessary. On the contrary, some road side ditches, ponds and khal exist, which necessitates road protection work. Road protection work is needed at left side of the road (total 187 m at Ch. 1150 - Ch.1200 m, Ch.1720 -  Ch.1745 m, Ch. 1820 - Ch. 1860 m, Ch. 3450 – Ch. 3470 m and Ch. 3750 – Ch. 3782 m) with different lengths and at right side of the road (total 385 m, at 7 different Chainage with different lengths).

At Kanar Bazaar Road from Ch. 0 to 2130 m and link Ch. 0 to 748 m, the existing BC pavement will be demolished and replaced by the new BC pavement. Presently, the existing road is partially damaged in different sections which hamper the normal traffic operation. The existing road is also narrow (average pavement width is 3 m). From the site investigation, it is revealed that damaged and narrow road interrupts normal traffic operation. Hence, widening and road improvement is necessary for the normal traffic operation. On the contrary, some road side ditches, ponds and khal exist, which necessitates road protection work. Road protection work is needed at left side of the road (total 370 m, at 9 different Chainage with different lengths) and at right side of the road (total 155 m, at Ch. 680 - Ch.700 m, Ch.930 - Ch.980 m, Ch. 1605 - Ch. 1650 m, Ch. 820 – Ch. 850 m and Ch. 1110 – Ch. 1120 m). The existing condition subproject road site is further elaborated in the following Photographs 2.2.1 and 2.2.2.

Photographs 2.2.1: Current Situation of the DC Office Road

Photographs 2.2.2: Current Situation of the Kanar Bazaar Road

## Envisaged Subproject Activities and Implementation Process

The general activities of the subproject includes: construction of the semi-pucca site office, construction of the labor shed and relocation of the electrical poles for access road.

Specific activities for BC Road:

- Site clearing work;

- Preparation of the hard bed by scarifying and loosening of the existing top;

- Earth work in box cutting;

- Sand filling on the road bed;

- Brick on end edging;

- Compacted aggregates and sub-base course;

- Providing prime coat;

- Lying pre-mixed dense bituminous surfacing.

Specific activities for Road Protection Wall:

- Earth work in excavation;

- Earth filling;

- Polythene sheet laying work;

- Cement concrete work;

- Brick masonry work;

- Fabrication of the ribbed or deformed bar;

- Reinforced cement concrete work;

- Pile driving work.

The materials to be used for the key activities: bricks, brick chips, stone chips, reinforcement bar, admixture, bearing pad, sand, cement, concrete, paint, laying pre-mixed dense bituminous surfacing etc.

The major equipment to be used for the implementation of the subproject: long-boom hydraulic excavator, brick breaking machine, pile rig for boring, concrete mixer machine, vibrator machine, MS sheet, steel cutter, steel shutter, steel drum rollers, pneumatic multiple tire roller, dump truck, water tanker, pump, generator and measuring device.

## Category of the Subproject

For BC Road and RCC Road Protection Work

- According to ECR 1997: Green □    Orange□ A    Orange B □√ Red □    Not Listed □

- According to WB classification:  Category B □√    Category C □

The overall anticipated adverse impact due to the subproject intervention is limited and site specific. Considering the environmental impact which is anticipated as insignificant, this subproject can be considered as Orange B as per ECR-97. According to the WB classification, it can be classified as Category B.

## Subproject Schedule

The tentative schedule of construction of the subproject is:

(a)  Subproject Duration (months)  	: 12 months

(b)  Tentative Start Date    		: 16 September 2018

(c) Tentative Completion Date  		: 15 September 2019

The daily construction hours will normally include regular working time. However, daily working hours may vary based on the on-site condition. The detailed work program will be prepared by the contractor with the assistance of the PIU-Pourashava. Then it will be shared with the PMU, LGED and DSM consultants. In addition, the detailed work program will also be shared with the Bank as per requirement.

# Baseline ANALYSIS OF THE ENVIRONMENTAL CONDITION

## Physical Environment

### Important Environmental and Infrastructural Features

During site visit the environmental and infrastructural features within the subproject area were collected. Conditional survey has also been conducted for designing the subproject. Hence, the survey data is also used for preparing the report. The environmental and infrastructural features listed within four directions of the existing bridges and culvert, two sides of approach roads in Table 3.1.1.

Table 3.1.1: Major Environmental and Infrastructural Features

| Chainage (m) | Left | Right | Major Environmental and Infrastructural Features |
| --- | --- | --- | --- |
| DC Office Police Box to Atong (Ch. 0 to 4192 m) |  |  |  |
| 0-100 | √ |  | Shops with various structures, trees, Survey Station, District Judge Office, Advocate Office |
|  |  | √ | Shops with various structures, electric poles, Circuit House, PWD Office, trees, Advocate Office |
| 100-200 | √ |  | Nagarik Adhikar Kamity Karjalay, shops, trees, electric pole |
|  |  | √ | Hazi Farida Mazid Hafezia Madrasa, khal, trees, electric poles, shops |
| 200-300 | √ |  | Judge Court Jame Mosque, boundary wall, electric poles, trees |
|  |  | √ | Advocate office, Police Superintend Office, electric poles, khal, trees |
| 300-400 | √ |  | Sabbir and Kaosar Enterprise, trees, various shops, tin fence, Hindu Marriage Registry Office |
|  |  | √ | LGED office, trees, khal, electric poles, shops |
| 400-500 | √ |  | Shops with various structures, trees, electric poles |
|  |  | √ | Trees, khal, temporary shops, electric poles |
| 500-600 | √ |  | Shops with various structures, trees, electric poles |
|  |  | √ | Trees, khal, temporary shops, electric poles |
| 600-700 | √ |  | Tin fence, electric poles, trees, houses, bunch of banana |
|  |  | √ | District Parishad Office, trees, electric poles , khal, boundary wall |
| 700-800 | √ |  | Palong Bazar Road, shops, electric poles, khal, houses, trees |
|  |  | √ | Hugli Road, shops, electric poles, khal, agricultural field, trees |
| 800-900 | √ |  | Residential houses, trees, electric pole |
|  |  | √ | Open field, houses, electric pole, trees |
| 900-1000 | √ |  | Homestead area, trees, bushes, connecting road, electric pole, open space |
|  |  | √ | Open field, houses, electric pole, trees |
| 1000-1100 | √ |  | Trees, bamboo bushes, bushes, houses, bunch of banana |
|  |  | √ | Trees , agricultural field, bushes |
| 1100-1200 | √ |  | Homestead area with various houses, pond, trees, bunch of banana, bamboo bushes |
|  |  | √ | Dakhin Baluchara Jame Mosque, trees, low land, residential houses, bunch of banana, bamboo bushes, electric pole |
| 1200-1300 | √ |  | Homestead area, trees, bunch of banana, bamboo bushes |
|  |  | √ | Houses, trees, bunch of banana, bamboo bushes, open field |
| 1300-1400 | √ |  | Homestead area, trees, bunch of banana, bamboo bushes |
|  |  | √ | Houses, trees, bunch of banana, bamboo bushes, open field |
| 1400-1500 | √ |  | House of Rothindra Kanto Ghatok Chowdhury, pond, common planted trees, abandoned structure of Rothindra Kanto Ghatok Chowdhury, houses, house fence |
|  |  | √ | Residential houses, house fence, open field, trees |
| 1500-1600 | √ |  | Trees, houses, electric pole |
|  |  | √ | Agriculture field, bamboo bushes, trees |
| 1600-1700 | √ |  | Pond, electric pole, Trees, houses |
|  |  | √ | Agriculture field, bamboo bushes, connecting Khagdi Bazar Road, trees |
| 1700-1800 | √ |  | Pond, electric pole, trees, houses, shop |
|  |  | √ | Agriculture field, bamboo bushes, trees, temporary shop |
| 1800-1900 | √ |  | Trees, bamboo bushes, houses |
|  |  | √ | Pond, electric pole, trees, bunch of banana, |
| 1900-2000 | √ |  | Trees, bamboo bushes, houses |
|  |  | √ | Pond, electric pole, trees, bunch of banana, |
| 2000-2100 | √ |  | Trees, bunch of banana, electric pole, houses, tin fence, khal |
|  |  | √ | Khal, trees, electric pole, open field, houses |
| 2100-2200 | √ |  | Boundary wall, residential houses, trees, bushes, electric pole |
|  |  | √ | Trees, pond, electric pole, residential houses, bushes |
| 2200-2300 | √ |  | House garden, boundary wall, trees, residential houses, ponds |
|  |  | √ | Trees, shrubs, pond, electric pole, open space, tree garden |
| 2300-2400 | √ |  | Ponds, trees, shrubs, bunch of banana, open space |
|  |  | √ | Trees, shrubs, bunch of banana, open space |
| 2400-2500 | √ |  | Fence of tin, trees, residential houses, electric pole |
|  |  | √ | Residential houses, trees, temporary shops, tin fence, electric pole |
| 2500-2600 | √ |  | Trees, open space, residential houses, bunch of banana, bushes |
|  |  | √ | Trees, bunch of banana, bushes, electric pole |
| 2600-2700 | √ |  | Bamboo fence, trees, residential houses, electric pole |
|  |  | √ | Residential houses, trees, temporary shops, tin fence, electric pole |
| 2700-2800 | √ |  | Tree garden, electric pole, houses, shrubs |
|  |  | √ | Trees, khal, shrubs, open field |
| 2800-2900 | √ |  | Trees, electric pole, houses, shrubs |
|  |  | √ | Trees, khal, shrubs, open field |
| 2900-3000 | √ |  | Khal, trees, bushes |
|  |  | √ | Open field, khal, trees, bushes |
| 3000-3100 | √ |  | House boundary, residential houses, tree, pond |
|  |  | √ | Sharps, trees, low land, agricultural land, pond |
| 3100-3200 | √ |  | Boundary wall, electric pole, tin fence, houses, trees, bushes |
|  |  | √ | Houses, trees, bushes, lowland, open field |
| 3200-3300 | √ |  | Houses, Mosque, Graveyard, trees, electric pole |
|  |  | √ | Trees, bushes, pond, lowland, open field |
| 3300-3400 | √ |  | Houses, tin fence of house, Graveyard, pond, trees, electric pole |
|  |  | √ | Residential houses, trees, bushes, pond, lowland, open field |
| 3400-3500 | √ |  | Hafezia Madrasa, pond, trees, fence, electric pole, connecting Uttar Atong road |
|  |  | √ | Semi pucca stationary shops, trees, bushes |
| 3500-3600 | √ |  | Semi pucca stationary shops, bushes, trees |
|  |  | √ | Atong Forkania Madrasa, Atong BM Yusuf Ali High School, school boundary, bushes, trees |
| 3600-3700 | √ |  | Pond, shrubs, trees, residential houses, bamboo fence, shops, bunch of banana, electric pole |
|  |  | √ | Houses with various structures, bamboo fence, bunch of banana, shops, connecting earth road |
| 3700-3800 | √ |  | Trees, bunch of banana, agricultural field |
|  |  | √ | Trees, bunch of banana, khal, agricultural field |
| 3800-3900 | √ |  | Trees, bunch of banana, agricultural field |
|  |  | √ | Trees, bunch of banana, khal, agricultural field |
| 3900-4000 | √ |  | Trees, bunch of banana, agricultural field |
|  |  | √ | Trees, bunch of banana, khal, agricultural field |
| 4000-4100 | √ |  | Trees, bunch of banana, agricultural field |
|  |  | √ | Trees, bunch of banana, khal, agricultural field |
| 4100-4192 | √ |  | Trees, bunch of banana, agricultural field |
|  |  | √ | Trees, bunch of banana, khal, agricultural field |
| Palong School to Kanar Bazaar (Ch. 0 to 2130 m) |  |  |  |
| 0-100 | √ |  | Various shops, houses, trees, bunch of banana, pond, tin fence |
|  |  | √ | School, open space, khal, bunch of banana |
| 100-200 | √ |  | Pond, trees, bamboo bushes, electric poles |
|  |  | √ | Houses, trees, bunch of banana, fence |
| 200-300 | √ |  | Electric pole, pond, trees, bunch of banana, houses |
|  |  | √ | Residential house, Mosque, trees, bamboo bushes, houses |
| 300-400 | √ |  | Tin shed houses, trees, bushes, bunch of banana |
|  |  | √ | Houses, trees, ditch, bunch of banana, fence |
| 400-500 | √ |  | Tin shed structures, trees, bunch of banana |
|  |  | √ | Houses, electric poles, trees |
| 500-600 | √ |  | Residential houses, pond, house boundary, trees |
|  |  | √ | Tin shed houses, electric pole, khal, trees, pond, bunch of banana |
| 600-700 | √ |  | Residential houses, pond, house boundary, trees |
|  |  | √ | Houses, pond, trees, bunch of banana |
| 700-800 | √ |  | Houses, trees, pond, bunch of banana, fence |
|  |  | √ | Tin shed house, trees, bunch of banana |
| 800-900 | √ |  | Houses, pond, trees, bunch of banana, open space |
|  |  | √ | Homestead area, pond, trees, bunch of banana, open space |
| 900-1000 | √ |  | Trees , tin shed houses, electric poles, pond, bunch of banana, agricultural field |
|  |  | √ | Trees, bunch of banana, bushes, tin shed houses, agricultural field, electric poles |
| 1000-1100 | √ |  | Trees, houses, electric poles, pond, bunch of banana, agricultural field |
|  |  | √ | Trees, ponds, houses, bunch of banana, bushes, agricultural field, electric poles |
| 1100-1200 | √ |  | Bunch of banana, trees, pond, houses |
|  |  | √ | Agricultural field, houses, electric poles |
| 1200-1300 | √ |  | Bunch of banana, trees, pond, open field |
|  |  | √ | Bunch of banana, khal, trees, agricultural field, electric poles |
| 1300-1400 | √ |  | Bunch of banana, trees, pond, open field |
|  |  | √ | Bunch of banana, khal, trees, agricultural field, electric poles |
| 1400-1500 | √ |  | Bunch of banana, trees, open field |
|  |  | √ | Tin shed structure, bunch of banana, khal, trees, agricultural field, electric poles |
| 1500-1600 | √ |  | Houses, bamboo bushes, bunch of banana, trees electric poles |
|  |  | √ | Bamboo bushes, bunch of banana, pond, trees, agricultural field, electric poles |
| 1600-1700 | √ |  | Houses, bamboo bushes, bunch of banana, khal, trees electric poles |
|  |  | √ | Bamboo bushes, bunch of banana, pond, trees, agricultural field, electric poles |
| 1700-1800 | √ |  | Tin shed residential houses, trees |
|  |  | √ | Houses, Mosque, trees, pond, electric poles |
| 1800-1900 | √ |  | Trees, houses, bamboo bushes, bunch of banana, electrical poles, agricultural field |
|  |  | √ | Residential houses, common planted trees, bunch of banana, bamboo bushes, electric poles |
| 1900-2000 | √ |  | Trees, bamboo bushes, electrical poles, agricultural field |
|  |  | √ | Tin shed houses, Mosque, trees, bushes, bunch of banana |
| 2000-2130 | √ |  | Trees, bamboo bushes, electrical poles, agricultural field |
|  |  | √ | Khal, trees, electric poles |
| Link Road Ch. (0 to 748 m) |  |  |  |
| 0-100 | √ |  | Trees, bamboo bushes, electrical poles, agricultural field |
|  |  | √ | Khal, trees, electric poles |
| 100 - 200 | √ |  | Trees, bamboo bushes, electrical poles, agricultural field |
|  |  | √ | Khal, trees, electric poles |
| 200- 300 | √ |  | Trees, bamboo bushes, electrical poles, agricultural field |
|  |  | √ | Khal, trees, electric poles |
| 300- 400 | √ |  | Trees, bamboo bushes, electrical poles, agricultural field |
|  |  | √ | Khal, trees, electric poles |
| 400-500 | √ |  | Houses, trees, bunch of banana, fence |
|  |  | √ | Tin shed house, trees, bunch of banana |
| 500-600 | √ |  | Houses, trees, bunch of banana, open space |
|  |  | √ | Homestead area, trees, bunch of banana, open space |
| 600-700 | √ |  | Houses, trees, pond, bunch of banana, fence |
|  |  | √ | Tin shed house, trees, bunch of banana |
| 700 -748 | √ |  | Houses, trees, bunch of banana, open space |
|  |  | √ | Homestead area, trees, bunch of banana, open space |

** Note: There are 168 no. electrical poles and 5 no. telephone poles to be re-located for the implementation of the subproject.

### Geology, Topography, and Soils

The topographical feature of the Shariatpur Pourashava area is undulating and low lying area. The elevation of the Pourashava ranges from 1.2 m to 11.0 m PWD. Geology is composed of Tidal Deltaic Floodplain. Shariatpur Pourashava lays the active floodplain of the Ganges and the adjoining meander floodplain. Soil developed in seasonally flooded alluvium containing lime in part or all of the upper 125 cm and with dark grey topsoil. Usually, there are continuous dark grey gleans. Generally, these soils are well oxidized within subsoil pads and below the upper dark layers. The central area of the municipality contains land comprised of soil on artificially slightly raised platform. According to BNBC (2006), Bangladesh has three seismic zones with moderate and low seismic activity. The Pourashava falls in Zone III (Figure 3.1.1).

Figure 3.1.1: Location of Shariatpur in Earthquake Zone Map of Bangladesh

### Climate and Meteorology

The climate of the subproject area can be described as Tropical Monsoon. It is characterized by warm, humid summers and cool, dry winters. From November to March, it is dry and cool while from April to May it is extremely hot during the pre-monsoon season. From June to October, the monsoon season is warm, cloudy, and wet. The warmest month is April, the coolest is January, the wettest is July and the driest is January.

The minimum temperature was 10.9°C in 2010 and maximum temperature was 42.4°C in 2010. The minimum humidity was 65.8% in 2010 and the maximum humidity was 79.8% in 2011.The average annual rainfall is around 1330 mm. The peak rainfall months in Shariatpur are June, and July. About 92.26% of the total annual rainfall occurs in the period from May through October. The highest annual rainfall during the last ten years (2008–11) was 1659 mm in 2008, and the lowest was 890 mm in 2010.

According to the statistics of wind data from the Bangladesh Meteorological Department Climate Division, wind direction changes by month. Nevertheless, the northwest, south, and northeast winds are predominant. The monthly average wind speed as recorded in Shariatpur Met Station is mostly remains in the range of 5 –10.4 knots/ hr. Nevertheless, in the winter months it remains in the range of 3.3-7.4 knots/hour. The highest daily average speed is recorded as 18 knots/ hr in the month of April, 1999 in the north direction.  Maximum monthly speed of wind is found to occur in the period of Mar – Apr reaching a maximum 0f 18.5 knots per hour. Table 3.1.2 shows the key meteorological data from 2008 to 2011.

Table 3.1.2: Temperature, Rainfall and Humidity during 2008-2011

| Years | Temperature(centigrade) |  |  | Rainfall (millimeter) | Humidity (%) |
| --- | --- | --- | --- | --- | --- |
|  | Maximum |  | Minimum |  |  |
| 2008 | 34.2 | 11.6 |  | 1659 | 79.0 |
| 2009 | 35.1 | 13.8 |  | 1169 | 79.0 |
| 2010 | 42.4 | 10.9 |  | 890 | 65.8 |
| 2011 | 23.8 | 13.9 |  | 1603 | 79.8 |

The construction works can be influenced by the climatic condition and meteorological components like humidity, temperature, sudden rainfall, and wind speed. During high winds, there might be possibility of quick spreading of dust generated from the construction activities. It is very risky to work during rain and in high winds because it increases injury risks. Furthermore, work under high temperature and excess humidity is extremely difficult and may create dehydration problem.

### Water Resources (Surface Water, Ground Water and Rain Water Harvesting)

There are khals and ponds in the subproject area. It is the main source of the surface water for the subproject area. This khal influence the hydrology of the municipality. Shariatpur Pourashava belongs to the hydro-geological unit II Holocene Deltaic and Flood Plains. Ground water is available at shallow depth. Groundwater is the main source of potable water in the subproject area. Groundwater is not saline and normally arsenic and iron free. The local people do not use rain water harvesting system.

### Flooding, Water Logging and Drainage Pattern

This area is affected in severe flood events of 1988, 1998, 2004 and 2007. There are surface water bodies such as natural khals, ponds in the subproject area. As per map (Figure 3.1.2), this Pourashava is situated in the normal flood prone area. The subproject area is not generally subjected to water logging problems. During continuous heavy rain, delay discharge of storm water causes water congestion.

Figure 3.1.2: Location of Shariatpur Pourashava in Flood Zone Map of Bangladesh

Air Quality and Dust

The degradation of the air quality is mainly from windblown dust and emission from the vehicles. Improper disposal of the solid wastes also spread bad odor which causes air quality degradation. During construction phase concrete mixer machine, vibrator machine, hydraulic excavator and vehicles used for transportation activities may degrade the air quality.

### Noise Level

Noise is generated mainly from the vehicles movement and hydraulic horn. Noise is also generated due to public gathering. During construction phase concrete mixer machine, vibrator machine, hydraulic excavator and vehicles used for transportation activities may create temporary noise nuisance within the subproject boundary.

Solid Waste Management

There is no structured dumping yard in the Pourashava of its own. The Pourashava people normally dump the collected solid wastes at roadside open spaces. Currently, the vacant place near abandoned Ruma Cinema Hall is used as a dumping ground by the Pourashava. This is simply open dumping to a specific location. In recent times, the municipalities provide services for household waste collection from door to door and dump at the vacant place near Ruma Cinema Hall. The generated wastes due to construction activities will also be dumped at the vacant place near Ruma Cinema Hall.

## Biotic Environment

### Flora and Fauna

In the subproject area, major roadside tree includes: Coconut, Mahogany, Betel-Nut, Mango, Koroi, Bunch of Banana, Bamboo Bushes, Jackfruit, Black Berry, Date, Papaya, Guava, Amra, Banyan, Dumur, Pulm, Boroi, Neem, Akashi, Amra, Chambal and Kadom. There are common local birds such as Crow, Sparrow, Bulbuli, Tuntuni, Shalik etc. were found during the site visit.

## Socio-economic and Socio-cultural Environment

Land Use, Status of Housing and Built-up Infrastructure

The land use pattern in the subproject area consists of mixed land-use pattern. The major roadside built-up infrastructures of DC Office Road include Advocate Office, Survey Station, District Judge Office, Circuit House, PWD Office, Nagarik Adhikar Kamity Karjalay, Hazi Farida Mazid Hafezia Madrasa, Judge Court Jame Mosque, Police Superintend Office, Hindu Marriage Registry Office, LGED Office, District Parishad Office, Palong Bazar Road, Dakhin Baluchara Jame Mosque, House of Rothindra Kanto Ghatok Chowdhury, Hafezia Madrasa, Atong Forkania Madrasa, Atong BM Yusuf Ali High School, residential houses, varieties shops, Mosque and Graveyard. In addition, khal, ponds, common planted trees, bunch of banana, bamboo bushes, shrubs, bushes, agricultural field, low land and open space were found during site visit.

Beneficiary Population

The subproject is situated within the jurisdiction of the ward number 01, 02, 04, 05 and 06. Considering the ward population about 26,780 people will benefit directly and many others indirectly.

Education

In the subproject area, literacy among the population is about 62.5%. This is higher than the national average (51.8%).The literacy rate among males (64.7%) is still more than females (60.2). (Ref: Population and Housing Census 2011).

Tribal Communities

There is no indigenous or tribal people settlement in the subproject area. Hence, no measures needed for the indigenous peoples’ safeguard.

### Land Acquisition

Private land acquisition is not needed. Due to site clearing work there are some roadside structures need to be removed. The Pourashava will clear the site amicably with the owners of these structures prior to the construction work. An agreement has already been signed by the Pourashava with the owners of these structures. However, in case of any objection from the affected persons, DSM will investigate the actual situation. Then, measures will be taken as per rules.

Principal Livelihoods and Economic Activities

The subproject area is inhabited by the mixed occupational people. In the Pourashava area, major income comes from business, enterprises, small trades, private sector jobs, and government jobs in the town.

Cultural Heritage and Protected Areas

There are no protected areas and no important cultural or historical sites identified in the subproject area. The local cultural structures are religious spots such as Mosques and Madrasa.

Social Conflicts and Development Activities

The local people welcome the subproject and there is no conflicts between the local communities regarding subproject implementation.

# ENVIRONMENTAL SCREENING

Environmental Screening (ES) for the subproject has been conducted with the purpose of fulfilling the requirements of Government (GOB) and World Bank (WB) for the preparation of subproject appraisal. Environmental Screening Checklist, as adopted in Appendix C of the Environmental Management Framework (EMF) of the MGSP, was administered for identifying the impacts and their extents.

The screening data and information for the BC Road and RCC Road Protection Work at DC Office Road have been formulated and are shown in below.

1)  Potential environmental impact during construction phase:

(a)  Ecological impacts:

Felling of trees  		Significant  □√  	Moderate  □    	Minor  □   Number of trees

306

- Clearing of vegetation  	Significant  □  	Moderate  □√   	Minor □

- Potential impact on species of

aquatic (i.e., water) environment  Significant □    Moderate □  Minor □√

Major trees to be felled down are: Coconut, Mahogany, Betel-Nut, Mango, Koroi, Bunch of Banana, Bamboo Bushes, Jackfruit, Black Berry, Date, Papaya, Guava, Amra, Banyan, Dumur, Banyan, Pulm, Sajina, Boroi, Neem, Akashi, Amra, and Chambal etc. There is no tree to be felled down bring religious and heritage importance. However, the trees to be felled down are moderate to small size in terms of length and diameter. Vegetation coverage on both sides of the road is not similar in terms of the intensity. So clearing of the road side vegetation is considered as moderate. In addition, anticipated impact on species of aquatic environment is minor because except throwing of the wastes materials in to the khals, ponds and ditches, the subproject does not have any impact on the aquatic environment. Considering the number of trees and vegetation to be felled down, the ecological impact is considered as significant.

- Physicochemical impacts:

- Noise pollution  			Significant □  	Moderate □√	Insignificant  □

- Air pollution  				Significant □  	Moderate □√	Insignificant  □

- Drainage congestion  			Very likely □  	Likely       □√   	Unlikely         □

- Water pollution  			Significant □  	Moderate □	Insignificant  □√

- Pollution from solid/ construction wastes  Significant □  	Moderate □√	Insignificant  □

- Water logging  				Significant □  	Moderate □√	Insignificant  □

The subproject will have impacts on noise and air quality during construction phase due to mobilization of the equipment, vehicles movement for the transportation of the materials, using of hydraulic excavator, brick breaking machine, mechanical compaction machine, paver, asphalt plant, concrete mixer machine, vibrator machine etc. The construction work will be performed section wise. Hence, it is anticipated that the overall impacts on noise and air quality will be moderate, temporary and limited within the subproject boundary. There are road side residents, educational and religious institutions. Hence, anticipated impacts on noise and air quality will be significant for this particular section if measures are not taken properly. The generated dust during construction activities may disturb the local people during construction period. The generated construction wastes, un-suitable materials and solid wastes may degrade the adjacent khal water if not properly collected and disposed and thrown in to the khal, pond and ditch. Improper collection and disposal of the generated waste materials also may degrade the quality of the surrounding environment and degrade the aesthetic value. The anticipated water logging and drainage congestion during construction period is minor because space is available at most sections of the road for drain-out the storm water. In addition, provision will be there for pumping out the storm water if required.

- Socio-economic impacts:

- Traffic congestion 			Very likely □  	Likely       □ √   	Unlikely     □

- Health and safety 			Significant □  	Moderate □√	Insignificant  □

- Impact on archaeological and historical   Significant □  	Moderate □  	Insignificant □√

- Employment generation 		Significant □ 	Moderate □ √  	Insignificant  □

The subproject activities will not have any severe impact on the local traffic system due to diversify of the traffic in the connecting and other roads. The construction work will follow simple procedure with commonly used equipment. Hence, anticipated impact on health and safety is moderate. The construction work adjacent to the educational institutes may cause safety risks to the public especially for school going kids. Therefore, construction activities adjacent to the educational institutes should be performed within very short time considering safety issues/site fencing. There is no archaeological and historical site will be affected due to the subproject activities. The subproject has positive impact by generating work opportunities for the local people and supplying of the construction materials, equipment, food and other necessary stuffs to the campsite.

2)  Potential environmental impact during operational phase:

(d)  Ecological impacts:

- Potential impact on species of aquatic     Significant □         Moderate □         Minor  □√

(i.e., water) environment

In operation phase, there is no likely adverse impact on the ecological environment. On the other hand, roadside tree plantation will enhance the ecological condition in the subproject area.

(e)  Physicochemical impacts:

- Potential air quality                 Improvement □       No-improvement      □          Deterioration  □√

- Noise level                              Improvement □       No-improvement     □           Deterioration  □√

- Drainage congestion    	         Improvement □       Minor Improvement □           No Impact      □√

- Risk of Water pollution            Significant     □        Moderate 	       □√         Minor              □

- Pollution from solid waste        Improvement □       No-improvement     □          Minor              □√

During operation phase, improved road may increase traffic volume which may degrade the air quality and noise level due to black smoke emission and un-due use of hydraulic horn from the vehicles. During operation phase, there is no likely impact on the water pollution and drainage congestion.

(f)  Socio-economic impacts:

- Traffic                                 Improvement □√	No-improvement □	Adverse □

- Safety 			    Improvement □√  	No-improvement □	Adverse □

- Employment generation     Significant     □	Moderate  □√		Minor    □

After completion, the subproject roads will improve the transport facilities. By providing uniform pavement, it will also enhance traffic safety. The new subproject will minimize the frequency of the maintenance for first three to five years after completion.

3)  Summary of the Possible Environmental Impacts of the Subproject:

The ecological impact due to the subproject implementation is mainly due to tree felling. However, the ecological imbalance to be caused due to cut down of the trees will be compensated by the plantation of the trees.

The overall impact due to implementation of the subproject is not severe. The physicochemical components will be disturbed during the construction phase. However, the anticipated impacts will be mainly within the subproject boundary. The wastes generated due to the construction activities should be properly collected and disposed in a designated site. There is also some adverse impact at operational stage. Due to improved road, motorized traffic volume may increase. Thus, air quality and noise level may be deteriorated due to vehicles emission and horns by the vehicles.

Safety concern is an important issue for both the construction and operation phases that should be considered properly. The socio-economic aspect mainly includes health and safety and employment generation. This subproject will have positive impacts in terms of the generation of the employment and business activities by supplying construction materials and equipment.

# Specific impact, mitigation AND enhancement measures

The likely impacts of the subproject are mainly caused by the activities required for the implementation of the subproject, and materials, resources and equipment to be used to perform the activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

The likely impacts of the subproject are mainly caused by the activities required for the implementation of the subproject, materials, resources and equipment to be used to perform the activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

  - Site Clearing, Dismantle, Excavation and Earth Work

The subproject works includes earth excavation and earth filling. These works lead dust blowing, improper disposal of the wastes, noise and vibration which may disturb the local people.

Mitigation Measures

- Proper care will be taken by the contractor during site clearing work, excavation work, earth work and disposal work to avoid any undue disturbances to the nearby people;

- Cover exposed earth works and other wastes materials with fabric;

- Disposal of soil and construction wastes to designated dump site at the vacant place near abandoned Ruma Cinema Hall;

- Avoid loss of the topsoil for the earth work.

  - Ecological Impact and Enhancement Work

There are 306 nos. of planted and naturally grown trees will be felled down and vegetation will be cleared due to implementation of this subproject.

Mitigation Measures

- Depending on the space availability, 750 nos. of the trees will be planted to compensate the felled down trees and ecological disturbance and also for beautification work- preferably local fruits, flowers, medicinal and ornamental trees-Mango, Jam, Jackfruit, Kathbadam, Amra, Bel, Krisnachura, Jarul, Shimul, Polash, Sonalu, Bokul, Kadom, Neem, Amloki, Horitoki, Bohera, Arjun, Mahogany, Koroi, Koi Babla, Palm Tree, Shishu, Rain Tree at both sides of the DC Office Police Box to Burirhat Bypass Road and Palong School to Kanar Bazar where adequate of space is available.

- Planting many trees will enhance the ecological condition of the area after their successful growth.

  - Pollution from the Storage of Construction Materials

Dumping of the construction spoils, including accidental leakage of the oil, grease, and fuel in equipment yards is a significant hazard. Storm water may carry these substances during rain. Thus, both surface and groundwater might be polluted from these contaminants. Air pollution and dust may affect the nearby settlement which is generated from fine aggregate such as sand and loose dry soil.

Mitigation Measures

- Safe transport, storage, and disposal provisions for the construction materials, and the equipment have to be carried out in order to avoid the accidental spillage and loss;

- Maintain adequate moisture content of soil and sand during transportation, compaction and handling;

- Carry the materials especially loose soil and sand with adequate cover;

- Fuels, lubricants, and other hazardous materials should store over raised platforms and  not directly on the ground;

- Disposal of un-used soils and construction wastes designated dump site at the vacant place near abandoned Ruma Cinema Hall;

- The playgrounds of the educational institutions should not use as a stockyard or work campsite.

  - Air Quality and Dust

During the construction phase, air pollutants will be emitted from the equipment and construction vehicles are expected to remain low. There are few residential houses located near subproject site. Hence, it will have very limited impacts in the nearby residential area. Limited dust generation and black smoke emission from the equipment and construction vehicles may temporarily disturb the workers. The overall impacts, however, are expected to remain very low.

Mitigation Measures

- When dust is visible water should be sprayed to control the dust;

- Maintain adequate moisture content of sand for handling, transportation, and compaction;

- Avoid use of dust generating equipment which produce significant amount of particulate matter far from the local residents;

- Ensure that all subproject vehicles are in good operating condition.

  - Noise and Vibration

Dumping of the construction spoils, including accidental leakage of the oil, grease, and fuel in equipment yards is a significant hazard. Storm water may carry these substances during rain. Thus, both surface and groundwater might be polluted from these contaminants. Air pollution and dust may affect the nearby settlement which is generated from fine aggregate such as sand and loose dry soil.

Mitigation Measures

- Safe transport, storage, and disposal provisions for the construction materials, and the equipment have to be carried out in order to avoid the accidental spillage and loss;

- Maintain adequate moisture content of soil and sand during transportation, compaction and handling;

- Carry the materials especially loose soil and sand with adequate cover;

- Fuels, lubricants, and other hazardous materials should store over raised platforms and  not directly on the ground;

- Disposal of un-used soils and construction wastes designated dump site at the vacant place near abandoned Ruma Cinema Hall;

- The playgrounds of the educational institutions should not use as a stockyard or work campsite.

  - Water Quality

There are some sources of the water pollution have been found at the subproject area. The water quality may deteriorate if the construction materials, sand, construction wastes, effluent from the work camps, spills, and food wastes are dumped in the low land, ponds and khal.

Mitigation Measures

- Proper construction management including waste management as well as training of the operators and other workers should provide to avoid pollution of the water bodies;

- Construction wastes will dispose properly (not in water bodies or lowland), for which contractor will be responsible;

- Construction wastes will dispose properly at the designated dumping site the vacant place near abandoned Ruma Cinema Hall, for which contractor will be responsible.

  - Occupational Health and Safety

The most important risks associated with the construction activities are listed below:

- Exposure to the sunlight- workers are being exposed to the sun for long hours;

- Exposure to the high temperature, and humidity for a long time resulting in dehydration;

- Contact with the hazardous substances and wastes pose risks of  the infections and diseases;

- Risk of the poor air quality due to dust;

- Risk of the collision (traffic);

- Risks from the head loads for carrying materials;

- Risks of using of the machinery in motion such as rod cutter;

- Risk associated to the sudden bad weather working conditions;

- Risk associated due to haphazard work by the workers.

General Requirements for the Workers’ Health and Safety

The key salient features of the general requirements for the workers’ health and safety stated is presented in Table 5.7.1.

Table 5.7.1:  General Requirements for the Workers Health and Safety

| Issues | Requirements |
| --- | --- |
| Health and Hygiene | - Cleanliness at the site premises and workers living places and at the Labor Shed; - Arrangement of the proper ventilation and temperature at the Labor Shed; - Protection against dust and furnace by using of the nose masks and covering of the head and body; - Proper disposal of the wastes and effluents; - Provision of the adequate latrines and separate toilets for the women; - Sufficient dustbins for the solid waste management system. - The labor shed shall be with the facilities like; mosquito nets, cooking arrangement, water supply, waste bins, lighting etc. - Potable water supply for drinking and other purposes |
| Safety and First Aid Box | - Using of the personal protective equipment (helmet, gloves, ear plug, goggles, nose mask, safety boots); - Precautions during work on or near machineries in motion; - Head loads are prohibited; - First aid facilities should be provided and maintained; - Ensure first aid box with the necessary medicine at the labor shed; - The first aid kit should include adhesive bandages, regular strength pain medication, gauze, and low grade disinfectant. |
| Compensation for Accidents at Work | - Contractors will bear medical treatment costs. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation-(the amount of the compensation should be fixed considering the type of accidents). |
| Dust and Fumes | - For any dust, fumes, or other impurities likely to be injurious to the workers, effective measures shall be taken to prevent their accumulation and its inhalation by the workers. |
| Overcrowding | - No labor room should be overcrowded. |
| Latrines and Urinals | - Sufficient latrines shall be provided; - Latrines shall be maintained in clean and sanitary condition; - Latrines shall be adequately lighted and ventilated. |
| Disposal of Wastes and Effluents | - Proper disposal system for the solid waste and effluent is required; - Waste bins must be provided by the contractor at labor shed. |

  - Impacts on Social Environment and Common Property Resources

Through comprehensive study, it is revealed that impacts are expected not to be severe and to be largely manageable. The following Table 5.8.1 presents impacts on socio-economic environment and common property resources.

Table 5.8.1: Impacts on Social Environment and Common Property Resources

| Social Components | Impacts on IECs | Impact Significance |
| --- | --- | --- |
| Community Perception | The local community people welcome this subproject and there is no visible objection from them. | Significant (+ve) |
| Employment and Business Opportunity | Community feels happy because generally the local contractor will be engaged for the construction works which will create work opportunity for the skilled and non-skilled labor. The subproject will create business opportunity for the equipment and materials suppliers’. | Significant (+ve) |
| Community Order and Security | This subproject activity does not create any severe security problems to the local community and community people. | Minor (-ve) |
| Possible damage to existing infrastructure and facilities | Degradation of the existing roads by the construction equipment/vehicles used in this subproject. | Minor (-ve) |
| New infrastructure and facilities | Construction of new roads will enhance municipality infrastructure facilities. | Significant (+ve) |
| Labor Habitat | Most of the labors will stay at the Labor shed which will have impacts on the environment relates to the generation of the solid wastes, effluent, and water consumption. | Moderate (-ve) |
| Health Care | Workers may suffer from the dehydration problems, and other health hazards. | Minor (-ve) |
| Accident | In case of road accidents by the vehicles to be used for the transportation may have serious negative impact. | Significant (-ve) |

Mitigation Measures

- Conduct dissemination with the local community about the subproject details;

- Continue liaison with the community leaders in order to maintain the community support;

- Engage local contractor and local people as much as possible for positive perception of the local community;

- Follow traffic rules to avoid any accidents;

- Transportation and mobilization of the equipments and construction materials avoiding peak hours and scheduled time;

- Ensure first aid facilities and effective use of personal protective equipments where applicable.

  - Labor Influx and Anticipated Impacts

The labor force and associated goods and services required for the construction of infrastructure civil works under this subproject cannot be fully supplied locally. The migration to and temporary settlement of laborers in the subproject, referred to as labor influx, carries an array of potentially positive and negative impacts in terms of demands on public infrastructure, utilities, housing and sustainable resource management and the strain on social dynamics.

Labor influx effects on host communities include positive impacts such as:

- The subproject activities will generate work opportunities for the local people and supplying of the construction materials, equipment, food and other necessary stuffs to the campsite;

- Improved infrastructure and public service access and availability whereby subproject investment catalyzes larger allocation of resources to a region, stimulating the development or expansion of infrastructure and public services.

Critical negative social risks include:

- Increase in criminal activity and alcohol and drug abuse, domestic violence, political attachment and violence, smuggling and robbery etc;

- Increase in gender-based violence, including eve teasing, sexual harassment etc;

- Increases in communicable diseases, including respiratory problems, diarrheal diseases, vector-borne diseases (e.g., malaria), and sexually transmitted infections (e.g., HIV/AIDS, syphilis, gonorrhea, hepatitis B);

- Conflicts arising from increased demand on existing infrastructure, services, and utilities, including transportation, health, education, water and sanitation, waste management, public utilities and community, religious, and recreational facilities and loss of land for access routes.

The general environmental impacts of labor influx include pressure on the natural resources such as using of the water, electricity, other fuel for cooking, loss of land for the labor establishment, depletion of the water supply, sewage and waste water generation, degradation of the air quality, waste generation,  increased demands on the local energy and resources and noise pollution effects. The following safeguard measures are recommended to avoid any risk of labor influx:

- Inform local people about the subproject activities;

- Liaison with the community leaders in order to get community support;

- Engage local people as much as possible to minimize workers from outsiders;

- Monitor workers attitude and behavioral matter;

- Monitor the workers movement for avoiding any unexpected social activities (robbery, crime, political attachment and conflicts, drugs abuse);

- Inform and use local administration to get support if needed;

- Inform local utilities service providers (such as for new electricity connection REB or any other department);

- Ensure effective use of natural resources such as water, electricity, fuel, wood etc.

  - Impacts on Traffic Movement

The subproject activities will interrupt free movement of the pedestrians and non-motorized vehicles during construction phase. However, the subproject activities will not have any severe impact on the local traffic system due to diversify of the traffic in the connecting and other roads.

During construction phase, interruption of the traffic movement and impact on the local traffic system due to the subproject activities will be monitored closely. Then separate traffic management plan will be provided if required. However, the following safeguard measures are recommended to minimize the impacts associated to the traffic movement:

- Inform local people about the subproject activities;

- Inspire local people to use connecting and diversion roads;

- Ensure schedule deliveries of material/ equipment during off-peak hours;

- Place traffic sign/cautionary sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption;

- The place of construction works should be fenced off with fences if required and should be isolated from general public access and marked with signs to ensure safe movement.

# Environmental Management Plan (EMP)

The purpose of the Environmental Management Plan (EMP) is to ensure that the activities are undertaken in a responsible and non-detrimental manner. The EMP will guide the environmentally sound construction of the subproject and ensure efficient lines of communication between the Project Management Unit (PMU, LGED), Project Implementation Unit (PIU) of Shariatpur Pourashava, DSM, and the contractors.

## Access to Information

The environmental assessment report should be translated into Bengali and disseminated locally. The copies of the report (both in English and Bengali) will be sent to all the concerned personnel responsible for subproject implementation. It will also be made available to the public. The final assessment report will also be uploaded in the LGED website and the World Bank website after approval.

## Grievance Redress Mechanism

The project-specific Grievance Redress Mechanism (GRM) has been established by the PIU of Shariatpur Pourashava to receive, evaluate, and facilitate the solution of APs concerns, complaints and grievances concerning the social and environmental performance of the subproject. The GRM is aimed to provide a time-bound and transparent mechanism to voice and resolve social and environmental concerns linked to the subproject.

The grievance mechanism is related to resolve the risks and adverse impacts of the subproject. It addresses APs’ concerns and complaints promptly, using an understandable and transparent process that is also gender responsive, and culturally appropriate. It is readily accessible to all segments of the affected people at no costs and without retribution. The mechanism should not impede access to the country’s judicial or administrative remedies. The affected people will be appropriately informed about the mechanism.

LGED has its own Grievance Redress Procedure (GRP), which it operates to address any dissatisfaction and complaints by the local people regarding its activities. This procedure is being applied to address any complaints or grievances through negotiations with the community leaders and representatives of the APs during implementation of the MGSP.

### Grievance Redress Committee (GRC)

The discussions and negotiations will be conducted by the PIU of Shariatpur Pourashava and will be involved the APs and Grievance Redress Committee (GRC) headed by The Mayor. With the facilitation of DSM, the Mayor nominated the GRC members and included representative from the Government Agencies, local NGO, and Civil Society. The GRC has been formed and established at Shariatpur Pourashava. The grievance box has already been delivered to the Pourashava to receive complaints. The grievance response focal point is available at Pourashava for instant response to an aggrieved person. It will receive written complaints or suggestions, and produce them to the GRC for hearing and resolution. The GRC includes the following key members.

Table 6.2.1.1: List of GRC committee members

| No | GRC Member Name | GRC Designation | Self Position |
| --- | --- | --- | --- |
| 1 | Rafiqul Islam | Chairman | Mayor, Shariatpur Pourashava |
| 2 | Laxmi Kanta Haldar | Member Secretary | Executive Engineer, Shariatpur Pourashava |
| 3 | Shohag Hawladar | Member | Assistant Commissioner, Shariatpur |
| 4 | Alamgir Hossain Khan | Member | Assistant Teacher, Palong Tulashar Govt. High School, Shariatpur |
| 5 | Sayed Bakaullah | Member | Director, Polli Unnayan Foundation |
| 6 | Alamgir Hossain Bepari | Member | Civil Society Representative |
| 7 | Shafiqul Islam | Focal Point | Assistant Engineer, Shariatpur Pourashava |

### Grievance Resolution Process

All complaints and suggestions will be received formally in the Pourashava Office by the GRC Member Secretary. A sample Grievance Redress Form has been prepared and sent to the GRC and Shariatpur Pourashava prior to the implementation of the subproject.

An intake register has been maintained at the office of the Member Secretary of Shariatpur Pourashava. Member Secretary takes the details of the grievances in the intake register for documentation and ensures impartiality, fairness, and transparency. The intake register has data and information columns including (i) Case no., (ii) Date of receipt, (iii) name, type of complaint, grievance, (iv) father’s name, husband’s name, (v) sex, (vi) complete address of the person raising the complaint, grievance, (vii) main objection (loss of land, property, or entitlement), (viii) detailed complaint story and (ix) expectation with documentary evidence and previous records of similar grievances, etc.

No GRC members are allowed to contact the aggrieved persons in advance. Rather, the concerned persons are informed to attend the formal hearings at an appointed date. The GRC committee will sit for hearing the complaints of the aggrieved persons. The GRC will record salient points presented by the aggrieved person and will examine documentary evidence submitted during informal hearings. A resolution register will be maintained by the Member Secretary at the Shariatpur Pourashava Office. The resolution register will contain (i) serial no., (ii) case no., (iii) name of complaint, (iv) complaint story and expectation, (v) date of hearing, (vi) date of field investigation (if any), (vii) results of hearing and field investigation, (viii) decision of GRC, (ix) progress (pending, solved) and (x) agreement or commitments. Closing register will keep records such as, (i) serial no., (ii) case no., (iii) name of complaint, (iv) decision and response to complaints, (v) mode and medium of communication, (vi) date of closing, (vi) confirmation of complainant’s satisfaction and (vii) management actions to avoid recurrence.

The GRC will decide within 30 days of receiving a complaint. There will also be an appeals procedure where, if a person is dissatisfied with the ruling of the GRC, he or she or a representative may attend their next meeting to present the case again. The committee will then reconsider the case in private, after which their decision is final. If the appellant is still not satisfied, then GRC will refer the complaint with the minutes of the hearings to the PD-LGED for further review. If the case at this level is again found unacceptable by the aggrieved person/s, PD, LGED will advise the Pourashava to drop the subproject. Shariatpur Pourashava should also publish the outcome of cases on public notice boards. All costs involved in resolving the complaints (meetings, consultations, communication, and information dissemination) will be borne by the Pourashava.

Based on consensus, this procedure will help to resolve issues or conflicts amicably and quickly, saving the aggrieved persons from having to resort to expensive, time consuming legal action. The procedure will however, not pre-empt a person’s right to go to the courts of law.

## Institutional Arrangement for Environmental Safeguard Compliance

In the institutional arrangement procedure, Project Director, Team Leader/Deputy Team Leader will directly involve. The PD and TL/DTL would be supported by DSM Environmental Safeguard Specialist and Social Management Specialist. The Pourashava Officials, especially engineers, would be responsible for supporting the construction supervision with the facilitation of DSM consultants. The civil works contractors will implement the environmental mitigation measures.

The PMU, with the help of Environmental Safeguard Specialist and Social Safeguard Specialist will submit the monthly monitoring reports on Environmental and Social Compliances to the World Bank.

PD-MGSP

TL/DTL-DSM

Environmental Safeguard Specialist-DSM

Social Management Specialist-DSM

Senior Municipal Engineer-DSM

Junior Environmental Specialist-DSM

Assistant Municipal Engineer-DSM

Social Management Officer-DSM

Pourashava Representative/ Focal Point/ Engineer

Contractor

Figure 6.3.1: Environmental and Social Management Team (Tentative)

## Capacity Building

A series of training program has already been conducted by the PMU and DSM to build the capability of the PMU, PIUs and DSM field staffs. Under this training program, PMU and DSM will organize an introductory course for training the participants on: (i) Environmental Screening, (ii) EMP Implementation, including environmental monitoring requirements related to mitigation measures; and (iii) taking immediate action to remedy unexpected adverse impacts or ineffective mitigation measures found during the course of the implementation. A separate training program for the contractor has also been conducted to enhance the Environmental awareness. A detailed training manual has already been developed by the Environmental Safeguard Specialist. Hence, the existing manual will be reviewed and modification will be done as per requirement for the next training program.

## Emergency Response and Disaster Management

Disaster management can be defined as the organization and management of
resources and responsibilities for dealing with all humanitarian aspects of emergencies, in particular the preparedness, response and recovery to lessen the impact of disasters. Emergency Preparedness Planning (EPP) and Contingency Planning (CP) are the processes of disaster management plan for developing strategies, arrangements, and procedures to address the humanitarian needs of those adversely affected by the crisis. There are four main types of disasters, namely: Natural disasters, Environmental Emergencies, Complex Emergencies, and Pandemic Emergencies.

For MGSP activities, Shariatpur Pourashava would identify the immediate needs, prioritize the tasks, and identify resource requirements to address the humanitarian needs of those adversely affected by the crisis.

## Environmental Management Action Plan

The activity wise anticipated environmental impacts and corresponding mitigation measures have been outlined in Table 6.6.1 and Table 6.6.2.

Table 6.6.1: Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | - Generation of sewage and solid waste may cause water/ environmental pollution | - Construction of sanitary latrine considering 15 persons for one toilet at the labor shed and separate toilet for male and female; - Ensure provision of waste bins (introduce separate waste bins for organic and inorganic wastes); - Ensure wastes (solid wastes and other forms of the waste) disposal at the vacant place near abandoned Ruma Cinema Hall. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well being of the local people. |  |
| General construction works | - Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces regularly to reduce dust generation; - Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and handling; - Ensure contractor sprinkle and cover stockpiles of loose materials (e.g., fine aggregates); | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Place traffic sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption; - Inform the local people about the subproject activities. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment producing excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles. |  |
|  | - Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes into adjacent water bodies, low lands and soil. |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 750 nos. of timber, medicinal and beatification trees will be replanted to compensate the felling trees. |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accidents; - Provide PPEs (personal protective equipment) and ensure using of the PPEs by the workers. |  |
|  | - Spills and leaks of oil, toxic chemicals | - Refueling and fuel tank should not be placed on the ground directly; - Proper handling of lubricating oil and fuel so that it does not fall on the soil; Place plane sheet/dry wood and polythene beneath the fuel tank, diesel engine used for vibrator machine, concrete mixer machine etc. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures, such as tree plantation, and cautionary signs. - In cautionary signs-text should be written in Bengali and English; - In cautionary signs-front size should be big so that it can be visible from at least 200m distance; - Cautionary signs should be placed at strategic points such as road intersection. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the BC road |  |  |  |
| Dismantle work/ Excavation/ Earth work/ | - Generation of solid and construction wastes due to the dismantle works; - Generation of loose soil due to the earth excavation work. | - Cover exposed earth works with fabric; - Disposal of soil and construction wastes at the vacant place near abandoned Ruma Cinema Hall. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accident due to dismantle of the existing road | - Carefully operation of the hammer. |  |
| ISG (Improved Sub grade)/ Sand filling | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Setting up and operation of asphalt plant | - Possible degradation of air quality by the suspended particles and increase noise level from asphalt plant affecting nearby settlements; | - Locate plant away from residential settlements; - Consider use of emulsified bitumen. |  |
|  | - Possible water pollution by bitumen and solvents; | - Avoid spills and proper collection and disposal of the generated spills. |  |
|  | - Possible preparation of the bitumen in open air and using of charcoal and wood as fuel | - Strictly prohibit bitumen preparation in the open air and use of charcoal and wood as fuel. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for RCC road protection wall |  |  |  |
| Excavation and site clearing & grubbing work | - Generation of loose and clay soil due to the earth excavation work | - Disposal of soil and construction wastes at the vacant place near abandoned Ruma Cinema Hall. - Cover exposed earth works with fabric. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accidents | - Carefully handle of the hydraulic excavator. |  |
|  | - Air pollution | - Regular maintenance of the equipment. |  |
| Precast RCC pile driving work | - Accident due to careless driving of pile. | - Carefully operate the pile driving equipment - Ensure the use of PPEs. |  |
| Sand filling/Back filling for Road Protection Wall | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Cutting & welding of the reinforcement for Road Protection Wall | - Noise Pollution | - Avoid using of rod cutter and wielding machine at night; |  |
|  | - Health and Safety | - Ensure use of the PPEs; - Availability and access to first-aid equipment and medical supplies. |  |
| RCC (reinforcement concrete) work for Road Protection Wall | - Air pollution due to black emission | - Regular maintenance of the concrete mixer and vibrator machine. |  |
|  | - Noise pollution | - Avoid operation of the concrete mixer and vibrator machine at night. |  |

Table 6.6.2: Anticipated environmental Impacts during operation phase and corresponding mitigation and enhancement measures

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Operation of the road | - Increase traffic speed and accidents; - Increased traffic congestion due to movement of increased number of vehicles. | - Better traffic management. | Monitoring by Pourashava |
|  | - Increased air and noise pollution affecting surrounding areas. | - Better traffic management; - Conduct awareness program to minimize undue use of hydraulic horns. |  |

## 6.7 Environmental Monitoring Plan

Environmental Monitoring Plan for this subproject will help to evaluate the extent and severity of environmental impacts against the predicted impact and the performance of environmental protection measures. The following Table 6.7.1 has been prepared for the key environmental indicators.

Table 6.7.1: Matrix Table of Monitoring Plan (Visual observation during construction phase)

| Monitored Parameter/ Issues | Monitoring Method/ Key Aspects | Location of Monitoring | Period & Monitoring Frequency |
| --- | --- | --- | --- |
| Safety orientation and training of workers | Frequency of training & orientation of workers for safety | Subproject site | - Once in a month - Reporting: Once in a month |
| Personal Protective Equipment | Ensure every single person involved in the activities wear and use safety equipment | Subproject site | - Daily - Reporting: Once in a month |
| Worker’s health | Monitoring process of worker’s health | Subproject site | - Daily - Reporting: Once in a month |
| Sanitation & drinking water facility to the workers | Availability of safe drinking water and sanitation to the workers | Subproject site | - Daily - Reporting: Once in a month |
| Incident record and reporting | Documented record of all incident, accident and its remedial process | Subproject site | - Daily - Reporting: Once in a month |
| Site security/ Fencing at the site | Isolation of site from general access by fencing, restriction of the un-authorized entry in the site. | Subproject site | - Daily - Reporting: Once in a month |
| Bulletin/ announcement boards/ prohibition signs | Visible in good condition or not | Subproject site | - Daily - Reporting: Once in a month |
| Equipment /vehicles | Switched-off diesel engines when not in use; Search any possible leakage; Fuelling. | Subproject site | - Daily - Reporting: Once in a month |
| Dust | Dust is visible or not | Subproject site | - Daily - Reporting: Once in a month |
| Oily waste generation and disposal | Quantity of oily waste, storage and disposal | Subproject site | - Daily - Reporting: Once in a week |
| Solid waste generation | Quantity of solid wastes and disposal | Subproject site | - Daily - Reporting: Once in a month |
| Gender equity | No discrimination regarding payment | Subproject site | - Daily - Reporting: Once in a month |
| Child labor | No child will be engaged in the activities | Subproject site | - Daily - Reporting: Once in a month |
| Handling of hazardous materials | Fuelling, storage, operation | Subproject site | - Daily - Reporting: Once in a month |

In addition, a comprehensive Environmental Compliance Monitoring checklist has been enclosed in Appendix-I.

## 6.8 Environmental Management Budget

Considering the environmental impacts and their mitigation measures for this subproject, several items are included in the BOQ to address these issues. The estimated cost for implementing the EMP is elaborated in Table 6.8.1.

Table 6.8.1: Environmental Management Budget in Bill of Quantity

| Item No. | Description of the Works | Unit Rate (BDT) | Qty (Number) | Amount (BDT) |
| --- | --- | --- | --- | --- |
| eme-1 | a) Dust suppression measures by water spraying throughout the construction period in and around the subproject sites, uncovered aggregates and loose materials such as stockpiles of the sand, excavated earth etc. | Lump sum |  | 10,000.00 |
|  | b) Dust suppression measures for BC road by water sprinkling (it includes water sprinkling for overall BC road construction works that includes-ISG, WBM, ASG and sub-grade) | Lump sum |  | 30,000.00 |
| eme-5 | Prevention of the spillage and leakage of the polluting materials | Lump sum |  | 5,000.00 |
| eme-6 | Campsite wastes disposal facility during the construction period (collection, transportation and dumping of the wastes at the vacant place near abandoned Ruma Cinema Hall): 2 nos. (1 no. for the organic wastes and 1 no. for the inorganic wastes disposal facility) | 20,000.00 | 2.00 | 40,000.00 |
| eme-7 | Campsite water supply facilities: Preferably 1 no. of tube well at the labor campsite (Depending on the site condition, DSM consultant will assist the contractor for selecting the option) | 20,000.00 | 1.00 | 20,000.00 |
| eme-8 | Campsite sanitation facilities: 2 nos. of the toilets preferably sanitary toilets at the labor campsite (1 no. for women and 1 no. for men) | 20,000.00 | 2.00 | 40,000.00 |
| eme-9 | a) Providing safety gear packages like hand gloves, spectacles for eye protection, ear plug, helmets, masks, visible jacket, safety shoes for at least 40 persons (30 workers and 10 visitors) | 3,000.00 | 40.00 | 120,000.00 |
|  | b) One first aid box with necessary accessories (contractor is responsible for providing necessary medicines, saline as per requirement during construction period) | 2,500.00 | 1.00 | 2,500.00 |
| eme-11 | Tree plantation to compensate the felled down trees and enhance the ecological condition in the subproject area - preferably local fruits, flowers, medicinal and ornamental trees-Mango, Jam, Jackfruit, Kathbadam, Amra, Bel, Krisnachura, Jarul, Shimul, Polash, Sonalu, Bokul, Kadom, Neem, Amloki, Horitoki, Bohera, Arjun, Mahogany, Koroi, Koi Babla, Palm Tree, Shishu, Rain Tree (including protection, fencing and conservation during project defect liability period): Preferably at both sides of the DC Office Police Box to Burirhat Bypass Road and Palong School to Kanar Bazar where adequate of space is available-750 nos. of the trees | 750.00 | 750.00 | 562,500.00 |
| eme-14 | Cautionary signs-4 nos. | 2,500.00 | 6.00 | 15,000.00 |
|  | Total |  |  | 8,45,000.00 |

After approval to revise the cost estimate has lengthy complex procedure. Hence, as per project EMF, PMU suggestion and experience from other LGED projects, adequate budget has been allocated for the environmental management for the mitigation and enhancement measures. The budget for labor shed and site office construction is included in the civil works items. Therefore, it is not included in the environmental budget. It should be noted that the contractor will be paid as per actual work done.

## 6.9 Environmental Codes of Practice

This section identifies and specifies environmental management guidelines and practices to be followed by the contractor for sustainable management of all environmental issues. The Contractor shall carry out the subproject related activities as specified in contract agreement. Shariatpur Pourashava shall ensure that contractor takes due responsibility to mitigate those negative impacts. Environmental awareness creation, particularly about the direct construction impacts and for the health, pollution and safety issues will be Contractor’s responsibility. Clauses that may be incorporated in the tender documents are:

- ECoP-1 (Overall Environmental Protection): Contractor shall take all steps to protect environment and avoid causing all types of public nuisances during implementation;

- ECoP-2 (Labor shed Management): Contractor shall maintain the work camp and construction sites in clean and tidy conditions and shall ensure standard facilities;

- ECop-3 (Workforce Environment): Contractor shall engage local people as much as possible where applicable and ensure prohibition of the child labor (less than 18 years) and aged labor (more than 65 years) in heavy works;

- ECoP-4 (Waste Management): Contractor shall be responsible for the safe transportation and disposal of the wastes generated due to the subproject activities;

- ECoP- 5 (Workers Health and Safety): Contractor shall be responsible for providing personal protective equipments and first aid facilities as per requirements;

- ECoP-6 (Compensation for Accidents):  Contractor shall bear medical treatment costs for any accidents. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation- (the amount of the compensation should be fixed considering the type of accidents);

- ECoP-7 (Implementation of the Mitigation Measures): Contractor shall responsible for the implementation of the mitigation measures mentioned in the EMP;

- ECoP-8 (Spill Prevention, Fuels and Hazardous Substances Management): Contractor shall take preventive measures for spill prevention and fuels and hazardous substances management;

- ECoP-9 (Restoration of the Facilities): The contractor on completion of the contract shall remove the equipment, surplus materials, and rubbish and temporary structures of all types and shall leave sites in clean condition to the satisfaction of Pourashava and local people.

# 7   PUBLIC Consultation and participation

- 7.1 Methodology

In the context of preparing the Environmental Assessment (EA), participatory public consultation was conducted for this subproject. The Pourashava Mayor, Officials, Engineers and local individuals as well as LGED and Consultant participants participated. Informal Focus Group Discussions (FGD) and a formal CIP were conducted involving the participants. In addition, walk-through informal group consultations and individual interviews were also held. The local communities were informed about the subproject intervention including their benefits. Suggestions made by the participants were listed and incorporated in the EMP accordingly.

### Photographs 7.1.1: DSM Consultant Team with Pourashava Representatives inspected the proposed roads

Photographs 7.1.2: Consultation Meeting at Pourashava with Pourashava Officials and Consultants Participants

- 7.2 Issues Raised by the Participants

The participants raised the issues related to the infrastructure development of Shariatpur Pourashava. The anticipated environmental and social impacts due to construction works have also been raised and discussed. The participants also discussed about the potential benefit from the subproject.

- 7.3 Feedback, Suggestions, and Recommendations of the Participants

The participants were presented with feedback, suggestions, and recommendations listed below:

- The FGD results confirmed that an improved communication network is needed for future development of Shariatpur Pourashava.;

- Most of the participants expressed that the number of subproject that have been selected for each financial year is insufficient;

- The participants stated that the public water supply facilities, sanitation facilities, and access road are not adequate;

- The participants stated that construction works should be scheduled properly;

- Local people also showed expectation for the increased opportunities for employment for unskilled or semi-skilled labor in the construction work;

- Local people expressed that construction of new roads will assist for the development of the peripheral road network in the Shariatpur Pourashava.

# 8   Conclusions and Recommendations

This study assists to understand the initial environmental impacts due to the subproject as well as to formulate the applicable mitigation and monitoring plans. Based on the environmental assessment, all possible environment aspects have been adequately assessed and necessary mitigation measures have been formulated to minimize the adverse impacts.

The overall conclusion is that if the mitigation, compensation, and enhancement measures are entirely implemented, there will be no significant negative environmental impacts as a result of location, design, construction, and or operation of the proposed subproject.

The conclusions of the environmental assessment can be summarized as follows:

- The short-term negative impacts that may come such as air quality, noise, solid waste, occupational health and safety will be minimized through the mitigation measures;

- The subproject will create employment for the workforce who live in the vicinity of the construction site and will provide them a short-term economic gain.

A few key recommendations are outlined below:

- All mitigation, compensation, and enhancement measures proposed in this report should be followed by the concern authorities for implementing the subproject;

- The environmental management and monitoring plan proposed in this report also needs to be followed;

- Regular monitoring by Pourashava, PMU (LGED) and DSM consultants is needed;

- The contractor will ensure using of the PPEs by the workers where applicable;

- The contractor is responsible for the proper disposal of the generated wastes materials from the subproject sites and campsite;

- The EMP shall be made binding on contractor operating the site, and will be included in the contractual clauses.

# 9 REFERENCE

### Shariatpur Pourashava Master Plan: 2011-2031 Urban Area Plan

### Shariatpur Pourashava at a glance

### Environmental Assessment Volume 1: Overall Environmental Assessment [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) and Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh.

### Environmental Assessment Volume 2: Environmental Management Framework (EMF) [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) and Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh.

Appendix

Appendix 1: Environmental Monitoring Checklist

Local Government Engineering Department

Municipal Governance and Services Project (MGSP)

Environmental Compliance Monitoring Form

Part A: General Subproject Information

| Subproject Name | (1)Construction of Road starting from DC Office Police Box to Atong via Pakar Matha (Ch. 0 + 00 to 4+ 192) by BC under Shariatpur Pourashava, Shariatpur. (2) Construction of Bituminous Carpeting Road from Palong School to Kanar Bazaar (Ch. 0 + 00 to 2130 m), Link Road (Ch. 0+ 00 to 0+748 m) under Shariatpur Pourashava, Shariatpur. |
| --- | --- |
| Package No. | MGSP/SHA/2017-18/W-06 |
| ULB Name | Shariatpur Pourashava |
| Approved Estimated Cost in BDT |  |
| Contract Amount in BDT |  |
| Contractor Name |  |
| Date of Commencement |  |
| Target Date of Completion |  |
| Physical Progress (%) |  |
| Financial Progress (%) |  |
| Person Responsible (PIU) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Person Responsible (DSM) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Form Completed by (Name, Designation &Phone) |  |
| Overall Environmental Description of the Subproject |  |

Part B: Design, Preparation, and Legal Requirements

| Environmental Concerns (PMU, PIU & DSM) (Name, Designation & Phone) | PMU- |  |
| --- | --- | --- |
|  | PIU- |  |
|  | DSM- |  |
| Subproject Category | DoE-BD- | WB- |
| Environmental Clearance Received? | Yes | No |
| EA Required? | Yes | No |
| EA Prepared and Delivered? | Yes | No |
| EMP Prepared & Delivered Separately? | Yes | No |
| Items and Cost of EMP Implementation Included in the Contract? | Yes | No |
| EMP Included in the Procurement Documents? | Yes | No |
| Inspection Schedule/Last Inspection/Monitoring by PMU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by PIU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by DSM Environmental Safeguard Specialist/Jr. Environmental Specialist | Date- |  |
|  | Key findings- |  |

Part C: Key Environmental Impacts

| The Subproject Results in any of the following Impacts? | Yes | No | NA | If yes, is the impact (give observation) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Significant | Moderate | Minor |
| Felling of the trees |  |  |  |  |  |  |
| Clearing of the vegetation that increase the risk of increased soil degradation or erosion |  |  |  |  |  |  |
| Disturbance of the terrestrial and or aquatic specifies |  |  |  |  |  |  |
| Noise pollution |  |  |  |  |  |  |
| Air pollution |  |  |  |  |  |  |
| Adverse effects on the quantity or quality of the surface water or groundwater |  |  |  |  |  |  |
| Production or increase the production of the solid waste |  |  |  |  |  |  |
| Drainage congestion |  |  |  |  |  |  |
| Water logging that increases the risk of the water related diseases |  |  |  |  |  |  |
| Traffic congestion |  |  |  |  |  |  |
| Public safety |  |  |  |  |  |  |

Part D: Work Place Environment and Gender Equity

| The Subproject Results in any of the following Impacts? | Yes | No | NA | Observations |
| --- | --- | --- | --- | --- |
| Does the contractor pay to the workers regularly? |  |  |  |  |
| Is there any discrepancy between the male and female workers regarding the wages or salary for the same works? |  |  |  |  |
| Is the contractor complying with the GOB labor law concerning the hiring of the workers? |  |  |  |  |
| Does the contractor engage women labors and does the project have suitable works for them? |  |  |  |  |
| Does the contractor engage child labor (less than 18 years) and aged people (more than 65 years old)? |  |  |  |  |
| Does the contractor force to the workers for the completion of the works? |  |  |  |  |
| Do the workers involve with the political activities, crime, drugs addiction and other forms of unwanted activities? |  |  |  |  |
| Are construction camps adequately equipped with water supply, sanitary toilets, washing facilities and facilities for waste collection and storage? |  |  |  |  |
| Has separate sanitation facilities been provided for women at work camps and the construction site? |  |  |  |  |

Part E: Potential Impacts, Mitigation Measures, and Monitoring Indicator Mentioned in the EMP

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | - Generation of sewage and solid waste may cause water/ environmental pollution | - Construction of sanitary latrine considering 15 persons for one toilet at the labor shed and separate toilet for male and female; - Erection of “no litter” sign, provision of waste bins/cans (introduce separate waste bins for organic and inorganic wastes); - Ensure wastes (solid wastes and other forms of the waste) disposal at the vacant place near abandoned Ruma Cinema Hall. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well being of the local people. |  |
| General construction works | - Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces regularly to reduce dust generation; - Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and handling; - Ensure contractor sprinkle and cover stockpiles of loose materials (e.g., fine aggregates); | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Place traffic sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption; - Inform the local people about the subproject activities. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment producing excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles. |  |
|  | - Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes into adjacent water bodies, low lands and soil. |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 500 nos. of timber, medicinal and beatification trees will be replanted to compensate the felling trees. |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accidents; - Provide PPEs (personal protective equipment) and ensure using of the PPEs by the workers. |  |
|  | - Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil and adjacent pond; - Collection and proper disposal of the spills. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures, such as tree plantation, and cautionary signs. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the BC road |  |  |  |
| Dismantle work/ Excavation/ Earth work/ | - Generation of solid and construction wastes due to the dismantle works; - Generation of loose soil due to the earth excavation work. | - Cover exposed earth works with fabric; - Disposal of soil and construction wastes at the vacant place near abandoned Ruma Cinema Hall. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accident due to dismantle of the existing road | - Carefully operation of the hammer. |  |
| ISG (Improved Sub grade)/ Sand filling | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Setting up and operation of asphalt plant | - Possible degradation of air quality by the suspended particles and increase noise level from asphalt plant affecting nearby settlements; | - Locate plant away from residential settlements; - Consider use of emulsified bitumen. |  |
|  | - Possible water pollution by bitumen and solvents; | - Avoid spills and proper collection and disposal of the generated spills. |  |
|  | - Possible preparation of the bitumen in open air and using of charcoal and wood as fuel | - Strictly prohibit bitumen preparation in the open air and use of charcoal and wood as fuel. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for RCC road protection wall |  |  |  |
| Excavation and site clearing & grubbing work | - Generation of loose and clay soil due to the earth excavation work | - Disposal of soil and construction wastes at the vacant place near abandoned Ruma Cinema Hall. - Cover exposed earth works with fabric. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accidents | - Carefully handle of the hydraulic excavator. |  |
|  | - Air pollution | - Regular maintenance of the equipment. |  |
| Precast RCC pile driving work | - Accident due to careless driving of pile. | - Carefully operate the pile driving equipment - Ensure the use of PPEs. |  |
| Sand filling/Back filling for Road Protection Wall | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Cutting & welding of the reinforcement for Road Protection Wall | - Noise Pollution | - Avoid using of rod cutter and wielding machine at night; |  |
|  | - Health and Safety | - Ensure use of the PPEs; - Availability and access to first-aid equipment and medical supplies. |  |
| RCC (reinforcement concrete) work for Road Protection Wall | - Air pollution due to black emission | - Regular maintenance of the concrete mixer and vibrator machine. |  |
|  | - Noise pollution | - Avoid operation of the concrete mixer and vibrator machine at night; |  |

Prepared by-                                                                                         Signature-                                                                                              Date-

Copies to 1.  PIU

2. SME

3. AME

4. Jr. ES