GOVERNMENT OF THE PEOPLE’S REPUBLIC OF BANGLADESH

Local Government Engineering Department (LGED) Local Government Division Ministry of Local Government, Rural Development, and Cooperatives

Environmental Assessment (EA) REPORT Name of the Subproject: Construction of RCC Drain from Dinajpur Road Moar to khorkhoria River including Installation of Street Light. Package No: MGSP/SAD/2018-2019/W-10 Saidpur Pourashava, Nilphamari Municipal Governance and Services Project (MGSP) Design, Supervision, and Management (DSM) Consultancy Services Joint Venture of Hifab International AB, Sweden AQUA Consultant and Associates Ltd., Bangladesh.

TABLE OF CONTENTS

|  |  |  |  | Page |
| --- | --- | --- | --- | --- |
| 1.0 |  | INTRODUCTION |  | 1-3 |
|  | 1.1 | Subproject Background |  | 1 |
|  | 1.2 | Objectives of the Study |  | 2 |
|  | 1.3 | Scope and Methodology of the Study |  | 2 |
| 2.0 |  | SUBPROJECT DESCRIPTION |  | 4-8 |
|  | 2.1 | The Study Area |  | 4 |
|  | 2.2 | Current Situation, Proposed Intervention and Need for the Subproject |  | 6 |
|  | 2.3 | Justification of Selection of this Subproject |  | 7 |
|  | 2.4 | Envisaged Subprojects Activities and Implementation Process |  | 7 |
|  | 2.5 | Category of the Subproject |  | 8 |
|  | 2.6 | Subproject Schedule |  | 8 |
| 3.0 |  | BASELINE ANALYSIS OF ENVIRONMENTAL CONDITION |  | 9-17 |
|  | 3.1 | Physical Environment |  | 9 |
|  | 3.2 | Biotic Environment |  | 15 |
|  | 3.3 | Socio-economic and Socio-cultural Environment |  | 16 |
| 4.0 |  | ENVIRONMENTAL SCREENING |  | 18-20 |
| 5.0 |  | SPECIFIC IMPACT, MITIGATION AND ENHANCEMENT MEASURES |  | 21-27 |
|  | 5.1 | - Dismantle Work, Excavation Work and Earth Work |  | 21 |
|  | 5.2 | Tree Felling, Clearing of Vegetation and Ecological Impact |  | 21 |
|  | 5.3 | Pollution from Construction Materials |  | 21 |
|  | 5.4 | Air Quality and Dust |  | 22 |
|  | 5.5 | Noise and Vibration |  | 22 |
|  | 5.6 | Water Quality |  | 23 |
|  | 5.7 | Occupational Health and Safety |  | 23 |
|  | 5.8 | Impacts on Social Environment and Common Property Resources |  | 24 |
|  | 5.9 | Labor Influx and Anticipated Impacts |  | 25 |
|  | 5.10 | Impacts on Traffic Movement |  | 26 |
| 6.0 |  | ENVIRONMENTAL MANAGEMENT PLAN |  | 28-39 |
|  | 6.1 | Access to Information |  | 28 |
|  | 6.2 | Grievance Redress Mechanism |  | 28 |
|  |  | 6.2.1 | Grievance Redress Committee (GRC) | 28 |
|  |  | 6.2.2 | Grievance Resolution Process | 29 |
|  | 6.3 | Institutional Arrangement for Safeguard Compliances |  | 30 |
|  | 6.4 | Capacity Building |  | 31 |
|  | 6.5 | Emergency Response and Disaster Management |  | 31 |
|  | 6.6 | Environmental Management Action Plan |  | 32 |
|  | 6.7 | Environmental Monitoring Plan |  | 37 |
|  | 6.8 | Environmental Management Budget |  | 38 |
| 7.0 |  | PUBLIC CONSULTATION AND PARTICIPATION |  | 40-41 |
|  | 7.1 | Methodology |  | 40 |
|  | 7.2 | Issues Raised by the Participants |  | 40 |
|  | 7.3 | Feedback, Suggestions and Recommendations of the Participants |  | 41 |
| 8.0 |  | CONCLUSIONS AND RECOMMENDATIONS |  | 42 |
| APPENDIX |  |  |  |  |
|  |  | Appendix I: Environmental Monitoring Checklist |  | 43 |

- LIST OF TABLES

| Table 3.1.1 | Major Environmental and Infrastructural Features |  |
| --- | --- | --- |
| Ta Table 3.1.2 | Temperature, Rainfall and Humidity during 2008-2011 |  |
| Table 5.7.1 | General Requirements for the Workers Health and Safety |  |
| Table 5.8.1 | Impacts on Social Environment and Common Property Resources |  |
| Table 6.2.1.1 | List of GRC Committee Members |  |
| Table 6.6.1 | Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures |  |
| Table 6.6.2 | Anticipated Environmental Impacts during Operational Phase and Corresponding Mitigation and Enhancement Measures |  |
| Table 6.7.1 | Matrix Table of Monitoring Plan (Visual observation during construction phase) |  |
| Table 6.8.1 | Environmental Management Budget |  |

- LIST OF FIGURES

| Figure 2.1.1 | Layout Plan of the Subproject |  |
| --- | --- | --- |
| Figure 2.1.2 | ### Topographic Feature of the Subproject Site |  |
| Figure 3.1.1 | Location of Saidpur Pourashava in Earthquake Zone Map of Bangladesh |  |
| Figure 3.1.2 | Location of Saidpur Pourashava in Flood Zone Map of Bangladesh |  |
| Figure 6.3.1 | Environmental and Social Management Team (Tentative) |  |

- LIST OF PHOTOGRAPHS

| Photographs 2.2.1 | Present Situation of the Subproject Site |  |
| --- | --- | --- |
| Photographs 2.2.2 | Present Situation of the Khorkhoria River (A) and Backbone Drain (B) Outfall. |  |
| Photographs 7.1.1 | Consultation Meeting at Subproject Site with Stockholder, Local People, Consultant and Pourashava Representatives. |  |

# INTRODUCTION

## Subproject Background

## Saidpur Pourashava was established in the year of 1958. It was upgraded to A-class Pourashava in the year of 1997. The Pourashava covers an area of 53.04 square kilometer. The current population of Saidpur Pourashava is about 1,27,104. At present, Saidpur Pourashava has about 81.54 km drains (katcha and pucca). (Source: Saidpur Pourashava Master Plan). In addition, Saidpur Pourashava has about 127 km road (Bituminous Carpeting Road-97.53km, CC Road-3.60km, brick flat soling-13.70km and non-paved road-12.13 km). Furthermore, Saidpur Pourashava has 4430 light posts. (Source: At a glance of Saidpur Pourashava).

In Saidpur Pourashava, the current infrastructure situation is in vulnerable state. The substandard drainage network cannot cope with the growing demand. Hence, planned development of drainage network with street light is essential to meet the emergent demand. Therefore, this subproject under the Municipal Governance Services Project (MGSP) is actively contributing infrastructures development of the Saidpur Pourashava.

This subproject includes the following components: RCC Drain and Street Light. The significant features of the subproject are mentioned below:

| Name of the Subproject : | (a) Construction of RCC Drain from Dinajpur Road Moar to Dohola Bridge (Ch.0-2000m) including link length 800m (Total Effective Length 4150m, Left & Right side). (b) Supplying, Fitting & Fixing of Street Light from Dinajpur Road to Khorkhoria River Bridge (Total Length 2800m). |
| --- | --- |
| Package No. : | MGSP/SAD/2018-2019/W-10 |
| District Name : | Nilphamari |
| ULB Name : | Saidpur Pourashava |
| Jurisdiction Area : | Ward no. 11 |
| Structural Design Option : | RCC Drain and Street Light |
| Beneficiary Population : | About 50,000 as per information by the Pourashava Officials |
| Tribal People : | No tribal people settlement found in the subproject area |
| Land Acquisition : | Saidpur Pourashava owned. So, private land acquisition is not required |
| - Estimated Cost : | 103.768 million BDT |
| Subproject Duration : | 12 Months |
| Tentative Start Date : | 31 January 2019 |
| Tentative Completion Date : | 30 January 2020 |

## Objectives of the Study

The general objective of this study is to determine the major environmental impacts succeeding from execution of the subproject and to recommend mitigation measures to avoid or reduce adverse environmental impacts and to enhance positive impacts. The specific objectives include:

- To assess the existing environmental conditions of the subproject site and its influence area;

- To identify and assess impacts resulting from the subproject during its construction phase and operation phase;

- To develop an environmental management plan with recommendations for mitigating impacts and enhance positive impacts;

- To summarize environmental monitoring requirements.

## Scope and Methodology of the Study

The subproject appraisal is included environmental screening for all the subproject components. According to that screening, no further environmental assessment is required for the street light. Nevertheless, environmental assessment is required to fulfill the regulatory requirement for RCC Drain. Hence, this report is mainly included RCC Drain. However, in the EMP table mitigation measures for street light are also included. The study methodology comprised the following activities:

Desktop Study

The desktop study involved:

- Initial meetings with client and stakeholders to discuss about the proposed subproject, including subproject activities;

- Collection and review of the baseline data, maps, reports and other relevant information on the existing environmental and social conditions of the subproject area;

- Review of the relevant existing legislation, regulation and policies;

- Understand the anticipated technical processes that may affect the environment.

Field Investigation and Data Collection

A team of the consultants made a field investigation to the proposed site. Field investigations involved mainly site walks within the subproject area and the neighboring areas that may be affected by the subproject. The following key tasks were performed during the field visit:

- Taking photographs of the significant aspects to describe the baseline environmental conditions of the subproject area;

- Interviews with representatives of the Pourashava Officials, beneficiary and affected people within the subproject influence zone;

- Verifying information and data collected during the desktop study and to collect new information that may have been important in the assessment of the impacts and design of the mitigation measures.

Data Analysis and Report Writing

The data and information collected from all the sources (literature review, secondary and primary data, public consultation) were analyzed to describe the existing environmental setting of the subproject area, to identify the potential positive and negative impacts of the proposed subproject, as well as to provide preliminary suggestions for mitigation measures. Finally, this environmental assessment report has been prepared.

# SUBPROJECT DESCRIPTION

## The Study Area

The subproject is situated within the jurisdiction of the ward number 11. The subproject site is located at the existing right of way. The geographic co-ordinate of the subproject site at starting point is Latitude 25°46'44.67"N, Longitude 88°53'47.90"E and at end point is Latitude  25°46'24.74"N and Longitude  88°52'37.73"E.The layout plan and topographic feature of the subproject is shown in Figure 2.1.1 and Figure 2.1.2.

Figure 2.1.1: Layout Plan of the Subproject Site

Figure 2.1.2: Topographic Feature of the Subproject Site

## Current Situation, Proposed Intervention and Need for the Subproject

At present, in the subproject area existing brick drain and earthen drain are discontinuous. Existing drains are also damaged, inappropriate dimension and silted up with different types of household’s waste materials, pit bottles and polythenes. Therefore, storm water cannot properly drain out in the subproject area. Subsequently, waste water stagnant on the existing drain which is the cause of environmental degradation of the subproject area by spreading bad odor from the decomposition of organic waste. In addition, natural surface drainage system has been destroyed in the subproject area due to formation of infrastructures (houses) by filling of low land and wetlands (as per information of local peoples during site visit). Resulting, the existing natural drainage system (low land and wetlands) does not functional. Therefore, during monsoon period, the subproject area become water logged. To overcome this adverse condition, Pourashava has taken initiative to construct RCC drain from Dinajpur Road to Khorkhoria River with footpath (Effective Length- 2215m) at one side of the road. While at other side of the road and at link road proposed drain is being road level cover slab (Effective Length- 1935m). Furthermore, there will be constructed a cross drain at Ch. 1550m to divert water from right side of the drain to left side of the drain. This drain will be connected with Khorkhoria River and Backbone Drain which will be act as outfall.

In addition, the existing street light in different segments of the road is not sufficient which are mostly non-functional. Hence, it interrupts pedestrians and traffic safety at night. Therefore, fixing of the street light is needed which will be fixed at one side of the road from Dinajpur Road to Khorkhoria River Bridge. The existing condition of drain is further elaborated in the following Photographs 2.2.1.

Photographs 2.2.1: Present Situation of the Subproject Site

The existing condition of the outfall is further elaborated by the Photographs 2.2.2.

A

B

B

### Photographs 2.2.2: Present Situation of the Kharkhoria River (A) and Backbone Drain   (B) Outfall.

## Justification of Selection of this Subproject

Based on CIP, Saidpur Pourashava prepared the priority list of the subprojects. The PMU and DSM team have inspected the site as a part of the reconnaissance survey. After site inspection, it is revealed that the anticipated impacts due to the subproject implementation will not be severe. Hence, considering the low environmental and social impacts due to this subproject implementation, this subproject has been selected. The subproject site is mostly located at the existing right of way. In addition, Pourashava is owned the land. Hence, land acquisition is not an issue for subproject implementation. After completion, the subproject has significant benefit to the community people. Hence, considering the benefit that will derive after construction, this subproject has been selected.

## Envisaged Subproject Activities and Implementation Process

The general activities for the subproject includes: construction of the semi-pucca site office, construction of the labor shed, relocation of the electric poles and site clearing works.

Specific activities for RCC Drain

- Earth work in excavation of the foundation;

- Pumping and bailing out of water as per requirement;

- Sand filling for the preparing foundation bed;

- Laying polythene sheet;

- Plain cement concrete work in foundation;

- Manufacturing CC blocks;

- Fabrication of the ribbed or deformed bar;

- Reinforced cement concrete work.

Specific activities for Street Light

- Assembling, fitting, fixing, installation;

- Testing and commissioning of the GI pole;

- Fitting and fixing energy meter board;

- Earthling the electrical installation with GI pipe;

- Fixing insulator, erection of tubular pole;

- Fixing of the overhead conductor;

- Fixing of the wire rack;

- Fixing and installation of the switch board;

- Fixing of the water tight street light with aluminum reflector;

The materials and resources to be used for the key activities: soil in earth work, sand, stone chips, brick chips, reinforcement, diesel, cement, electricity, water, GI pole and energy meter.

The major equipment to be used for the implementation of the subproject: excavator, concrete mixer, diesel engine, steel cutter, steel shutter and mechanical vibrator.

## Category of the Subproject

RCC Drain

- According to ECR 1997: Green □    Orange□  A  Orange B □√  Red □ Not Listed □

- According to WB classification: Category B □√	 Category C □

Considering the environmental impacts, the RCC Drain can be considered as Orange B as per ECR-97.  According to the WB classification, it can be classified as Category B.

Street Light

- According to ECR 1997: Green □√    Orange A □  Orange B □ Red □

- According to WB classification: Category A □ Category B □	     Category C  □ √

Considering the environmental impacts, the street light can be considered as Green category as per ECR-97.  According to the WB classification, it can be classified as Category C.

## Subproject Schedule

(a)  Subproject Duration (months)  	: 12 months

(b)  Tentative Start Date    		: 31 January 2019

(c) Tentative Completion Date  		: 30 January 2020

# Baseline ANALYSIS OF THE ENVIRONMENTAL CONDITION

## Physical Environment

### Important Environmental and Infrastructural Features

The important environmental features have been identified during site visit within influence area of the subproject. Moreover, the conditional survey has been performed. Hence, the survey data is also used for identification of the major environmental features. The detailed environmental features listed considering the 100 m Chainage from Dinajpur Road to Khorkhoria River and Link – Road 800m. The key findings of the site inspection and investigation are given in Table 3.1.1.

Table 3.1.1: Major Environmental and Infrastructural Features

| Chainage (m) | Left | Right | Major Environmental and Infrastructural Features |
| --- | --- | --- | --- |
| Dinajpur Road Moar to Khorkhoria River (Ch. 0-2000m) |  |  |  |
| 0-100 | √ |  | Pucca, Semi-pucca and Tin shed structure of Varieties Shops and Commercial Space, Electric Pole, Agrani Bank, |
|  |  | √ | Pucca, Semi-pucca and Tin shed structure of Varieties Shops and Commercial Space, Islami Bank, Bangladesh Awamiluge Office, Sador Police Fari, Connecting Road Towards Jahrul Islam Road; |
| 100-200 | √ |  | Basic Bank, City Community Hospital, Pucca, Semi-pucca and Tin shed structure of Varieties Shops and Commercial Space, Backbone Central Drain; |
|  |  | √ | Bitul Aman Jame Mosque, Forecast Islami Insurance Co. Ltd. Back Bone Drain, Pucca, Semi-pucca and Tin-shed Residents and Shops, Electric Pole, Trees, Brick Drain; |
| 200-300 | √ |  | Homeo Phatic Hospital College, Pucca, Semi-pucca and Tin-shed Residential and Commercial Structure, Electric Pole, Trees, Connecting Road Towards Kacha Bazar Road, |
|  |  | √ | Mission General Hospital, Pucca, Semi-pucca and Tin-shed Residential and Commercial Structure, Connecting Road Towards Pourashava, Electric Pole, Brick Drain; |
| 300-400 | √ |  | Pucca, Semi-pucca and Tin-shed Residential and Commercial Structure, Electric pole , Trees; |
|  |  | √ | Sonali Bank, Safe Health and Rehabilitation Center, Pucca, Semi-pucca and Tin-shed Residential, Electric Pole, Trees; |
| 400-500 | √ |  | Boundary Wall, Brick Drain, Pucca Semi-pucca and Tin-shed Residents, Electric Pole; |
|  |  | √ | Sundorbon Express Transportation Ltd, Rocket Kuriar Services, Pucca, Semi-pucca and Tin-shed Residents and Shops, Electric Pole, Brick Drain; |
| 500-600 | √ |  | Pucca, Semi-pucca and Tin-shed Residents and Shops, Brick Drain, Boundary Wall, Electric Pole; |
|  |  | √ | Saw Mill, Boundary Wall, Pucca, Semi-pucca and Tin-shed Residents, Trees, Electric Pole, Connecting Road Towards New Babupara, Low Land, Open Space, Brick Drain; |
| 600-700 | √ |  | Saw Mill, Pucca, Semi-pucca and Tin-shed Residents, Trees, Electric Pole, Agricultural Field, Low Land, Boundary Wall; |
|  |  | √ | Boundary Wall, Tress, Pucca, Semi-pucca and Tin-shed Residents, Electric Pole, Agricultural Field, Low Land; |
| 700-800 | √ |  | Pucca, Semi-pucca and Tin-shed Residents, Open Space, Trees, Electric Pole, Brick drain, Agricultural Field, Low Land; |
|  |  | √ | Agricultural Field, Low Land, Boundary Wall, Tress, Pucca, Semi-pucca and Tin-shed Residents; |
| 800-900 | √ |  | Varieties Shops, Workshops, Choyti Jewelers, Salon, Pucca, Semi-pucca and Tin-shed Residents, Trees; |
|  |  | √ | Sony Studio, Ma Moni Tailors, Sukria Tailors, Surjo Medical, Varieties Shops, Trees, Boundary Wall; |
| 900-1000 | √ |  | Pucca , Semi pucca and Tin shed Residents, Low land, Trees, Open space, electric pole; |
|  |  | √ | Boundary Wall, Pucca , Semi pucca and Tin shed Residents and Shops, Trees, Electrical Pole; |
| 1000-1100 | √ |  | Trees, Open space, Boundary wall, Pucca , Semi pucca and Tin shed Residents and Shops; |
|  |  | √ | Semi pucca and Tin shed Residents and Shops, Electric Pole, Saw mill; |
| 1100-1200 | √ |  | Semi pucca and Tin shed Residents, Boundary Wall, Trees, Vegetation Coverage; |
|  |  | √ | Boundary Wall, Trees, Pucca , Semi pucca and Tin shed Residents, Electric Pole; |
| 1200-1300 | √ |  | Boundary Wall, Trees, Pucca , Semi pucca and Tin shed Residents, Electric Pole; |
|  |  | √ | Boundary Wall, Trees, Pucca , Semi pucca and Tin shed Residents, Electric Pole; |
| 1300-1400 | √ |  | Open space, Boundary wall, Pucca , Semi pucca and Tin shed Residents and Shops; |
|  |  | √ | Trees, Pucca , Semi pucca and Tin shed Residents, Electric Pole, Mosque; |
| 1400-1500 | √ |  | Boundary wall, Pucca , Semi pucca and Tin shed Residents and Shops; |
|  |  | √ | Brick drain, Trees, Pucca , Semi pucca and Tin shed Residents, Electric Pole, Mosque; |
| 1500-1600 | √ |  | Semi pucca and Tin shed Residents, Boundary Wall, Trees, Vegetation Coverage; |
|  |  | √ | Boundary Wall, Trees, Pucca , Semi pucca and Tin shed Residents, Electric Pole, Low land; |
| 1600-1700 | √ |  | Pucca, Semi pucca and Tin shed Residents, Trees, Vegetation Coverage, Electric Pole, Low land; |
|  |  | √ | Pucca, Semi pucca and Tin shed Residents, Trees, Vegetation Coverage, Electric Pole, Low land; |
| 1700-1800 | √ |  | Boundary Wall, Vegetation Coverage, Trees, Pucca, Semi-pucca and Tin shed Residents; |
|  |  | √ | Boundary Wall, Vegetation Coverage, Trees, Pucca, Semi-pucca and Tin shed Residents; |
| 1800-1900 | √ |  | Pucca, Semi-pucca and Tin Shed Residents, Boundary Wall, Trees, Vegetation Coverage, Electric Pole; |
|  |  | √ | Trees, Vegetation Coverage, Pucca, Semi-pucca and Tin Shed Residents; |
| 1900-2000 | √ |  | Saidpur Kendrio Shoshan Ghat, Mashers Northern Cold Storage, Stationary Shop, Trees; |
|  |  | √ | Planted Trees, Low land, Pucca, Semi-pucca and Tin shed Residents, Electric Pole; |
| Link Road (Ch.0-800m) |  |  |  |
| 0-100 | √ |  | Boundary wall, Trees, Pucca, Semi Pucca and Tin Residents; |
|  |  | √ | Govt. Saidpur College, Pucca, Semi Pucca and Tin Residents, Electric pole, Trees, Boundary wall; |
| 100-200 | √ |  | Boundary wall, Trees, Pucca, Semi Pucca and Tin Residents; |
|  |  | √ | Shops, Boundary wall, Trees, Pucca, Semi Pucca and Tin Residents; |
| 200-300 | √ |  | Open space, Shops, Boundary wall, Trees, Pucca, Semi Pucca and Tin Residents; |
|  |  | √ | Pucca, Semi Pucca and Tin Residents, Boundary wall, Trees; |
| 300-400 | √ |  | Low land, Boundary wall, Agricultural Field, Trees, Pucca, Semi Pucca and Tin Residents; |
|  |  | √ | Agricultural Field, Boundary wall, Brick Drain, Trees, Pucca, Semi Pucca and Tin Residents; |
| 400-500 | √ |  | Pucca, Semi Pucca and Tin Residents, Brick Drain, Boundary Wall, Trees; |
|  |  | √ | Pucca, Semi Pucca and Tin Residents, Boundary Wall, Electric Pole, Trees; |
| 500-600 | √ |  | Pucca, Semi Pucca and Tin Residents, Boundary Wall, Trees; |
|  |  | √ | Boundary Wall, Electric Pole, Trees, Pucca, Semi Pucca and Tin Residents; |
| 600-700 | √ |  | Stationary Shops, Pucca, Semi Pucca and Tin Residents, Boundary Wall, Trees; |
|  |  | √ | Electric Pole, Brick Drain, Pucca, Semi Pucca and Tin Residents, Boundary Wall, Trees; |
| 700-800 | √ |  | Pucca, Semi Pucca and Tin Residents, Boundary Wall, Trees; |
|  |  | √ | Pucca, Semi Pucca and Tin Residents, Boundary Wall, Trees; |

** Note: There are 64 numbers of the electrical poles at different Chainage to be re-located for the implementation of the subproject.

### Geology, Topography, and Soils

The landscape condition of the subproject area is level upland like other areas of Saidpur. The elevation of land is ranging from 51.57-55.79 m PWD. Geology of Saidpur Pourashava belongs to Gravelly Sand (Alluvial Fan deposited sediments). The geology of the area can be attributed to Quaternary alluvial sequence which is a part of Tista floodplain. This deposition has resulted in a complex array of coarse sand and gravel channel-fills followed by intercalated finer sediment layers of alluvial and deltaic floodplain aquifers. The sedimentary environment has changed from braided streams and mean-daring rivers to floodplains making deposition of granular layers and finer silty sediments. General soil types predominantly include grey floodplain soils and non-calcareous brown floodplain soils. According to BNBC (2006), Bangladesh has three seismic zones with moderate and low seismic activity. The Pourashava falls in Zone II.

The green circle denotes the Saidpur Pourashava

Figure 3.1.1: Location of Saidpur Pourashava in Earthquake Zones Map of Bangladesh

### Climate and Meteorology

The climate of the subproject area can be described as hot, wet and humid tropical climate. It is characterized by warm, humid summers and cool, dry winters. From November to March, it is dry and cool while from April to May it is extremely hot during the pre-monsoon season. From June to October, the monsoon season is warm, cloudy, and wet. The warmest month is April, the coolest is January, the wettest is July and the driest is January.

The minimum temperature was 9.3°C in 2011 and maximum temperature was 33.1°C in 2010. The minimum humidity was 63.4% in 2010 and the maximum humidity was 77.4% in 2011.The average annual rainfall is around 1842 mm. The peak rainfall months in Saidpur are June, and July. About 92% of the total annual rainfall occurs in the period from May through October. The highest annual rainfall during the last ten years (2008–11) was 2140 mm in 2009, and the lowest was 1526 mm in 2010.

According to the statistics of wind data from the Bangladesh Meteorological Department Climate Division, wind direction changes by month. Nevertheless, the northwest, south, and northeast winds are predominant. The monthly average wind speed as recorded in Rangpur Met Station is mostly remaining in the range of 2.3 – 5.3 knots/ hr. But in the winter months it remains in the range of 2.3-4 knots/hour. Table 3.1.2 shows the key meteorological data from 2008 to 2011.

Table 3.1.2: Temperature, rainfall, humidity during 2008-2011

| Years | Temperature(centigrade) |  | Rainfall (millimeter) | Humidity (%) |
| --- | --- | --- | --- | --- |
|  | Maximum | Minimum |  |  |
| 2008 | 32.2 | 10.0 | 1881 | 71.0 |
| 2009 | 33.0 | 12.3 | 2140 | 77.0 |
| 2010 | 33.1 | 10.2 | 1526 | 63.4 |
| 2011 | 21.2 | 9.3 | 1821 | 77.4 |

The construction works can be influenced by the climatic condition and meteorological components like humidity, temperature, sudden rainfall, and wind speed. During high winds, dust generated from the construction activities may spread quickly. It is very risky to work under rain and in high winds because the possibility of risk and accident increases. Furthermore, work under high temperature and excess humidity creates health hazards like dehydration problem.

#### Water Resources(Surface Water, Ground Water, and Rain Water)

In the subproject area, ponds, ditches and seasonal wetlands and Khorkhoria River are the main source of surface water.

Groundwater is the main source of potable water in the subproject area. Deep groundwater is not saline and normally arsenic (<0.01mg/l) and iron free. Local people typically use deep tube-well water for drinking and other domestic purposes. Salinity problems are not visible. Rain water harvesting system is not common in and around the subproject area.

Flooding, Water Logging and Drainage

Saidpur Pourashava is not normally affected by external floods and or flash floods. The drainage system of the Pourashava is not adequate and non-functional. In addition, the maintenance of the existing drainage system is not satisfactory. People throw and dispose wastes in the drains. Therefore, the drain is being filled up. The poor conditions of the drains cause overflows when it rains heavily and spread bad smell and serves as a breeding ground for mosquitoes. According to the flood zoning map of Bangladesh, this area lies in the flood free area.

The green circle denotes the Saidpur Pourashava

Figure 3.1.2: Location of Saidpur Pourashava in Flood Zone Map of Bangladesh

Air Quality and Dust

Air quality data is not available from secondary sources. Air seems to be clean. However, in dry season dust is generated due to vehicles movement and windblown dust causes air pollution. In addition, black smoke emission from the vehicles degrades the quality of the air. There are no remarkable sources of air pollution such as heavy industries observed in the subproject area.

Noise Level

Ambient noise level data is not available. From the site investigation, it is revealed that the main sources of noise are vehicles. Vehicles such as trolleys, pick-up, trucks, motor cycles, mini trucks, and private cars generally move on the road during day and night. These vehicles generate noise in the subproject area. There are 4-5 flights from Dhaka to Saidpur every day. It also temporarily causes noise disturbances to the local people. During construction phase, concrete mixer machine, vibrator machine, asphalt plant and steel cutter may generate noise.

Solid Waste Management

Practically, there is no well-structured solid waste management/disposal system in the Pourashava. The people of the poor community accumulate their solid waste in their own yard, where it decomposes.  There is no proper conservancy set up in the Pourashava yet. From the site investigation, it is revealed that mass people are still dumping wastes in vacant low lands. Therefore, improper solid waste disposal by the community people creates severe public health hazards and environmental degradation to a great extent.

## Biotic Environment

Flora and Fauna

There are few low lands and ponds adjacent to the proposed site. In addition, Kharkhoria River is also sited within the subproject influence area. These low land, pond and Kharkhoria River is the worthy habitat of the aquatic flora and fauna. Water hyacinth is the dominating aquatic floral species in the subproject area.

Terrestrial plants found during survey in and around the subproject area, on homesteads, roadside and agricultural lands have been listed. The subproject area provides the following major species of natural plants including herbs, shrubs, grasses and plants which are important both economically as well as for environmental sustainability of the area. The flora in and around the subproject area are dominated by the fruit plants and ornamental plants.

There is no natural or social forest in and around the subproject area. However, there was found few vegetation along the drain alignment from Ch. 1000-1500m (Approximately). In addition, at both side of the road from Ch. 800-1500m (Approximately) there are few agricultural field where rice are growing abundantly.

The common planted trees includes- Rain Tree (Samanea saman), Supari (Areca catechu), Coconut (Cocos nucifera), Akashi (Acacia auriculiformis), Mango (Mangifera indica), Jackfruits (Artocarpus heterophyllus), Mahogany (Swietenia macrophylla), Tetul (Tamarindus indica) Shisho (Dalbergia sissoo), Goava (Psidium guajava) and krisnochura (Caesalpinae Delonix) etc.

There are common local birds such as Shalik (Acridotheres fuscus), Choroi (Passer domesticus), Bok (Mesophoyx intermedia), Dowel (Copsychus saularis), Crow (Corvus splendens), Bulbuli (Pycnonotus barbatus), and Tuntuni (Orthotomus sutorius) etc. were found during the site visit.

## Socio-economic and Socio-cultural Environment

Land Use, Status of Housing and Built-up Infrastructure

The land use pattern of the subproject area is divers. Most of the area of the proposed site cover the residential area while about first 500m covers the densely commercial activities. Therefore, this 500m is very busy area.

The subproject area and its influence area covers educational institutes, health care institutes (City Community Hospital, Mission General Hospital, Homeo Phatic Hospital College) Religious institutes (Mosque), NGOs(Forecast Islami Insurance Co. Ltd.), Bank Bima (Sonali Bank, Agrani Bank, Islami Bank, Basic Bank), local administrative area (Police Fari), Sundorbon Express Transportation Ltd, Rocket Kuriar Services, pucca, semi pucca and tin shed residents and varieties shops with different structure.

### Beneficiary Population

The subproject site is situated within the jurisdiction of the Ward number no 11 of the Saidpur Pourashava. As per information by the municipality, considering the ward population about 50000 people will benefit directly and many others indirectly.

### Education

In the Pourashava area, literacy among the population is about 54.6%. This is higher than the national average (51.8%). The literacy rate among males (57.4) is still more than females (51.6). (Ref: Population and Housing Census 2011).

### Tribal Communities

There is no indigenous or tribal people settlement live in the subproject area. Therefore, there are no measures needed for indigenous peoples’ safeguard.

### Land Acquisition and Resettlement

No private land acquisition is needed. But proposed drain alignment belong the RHD land. However, Pourashava Authority has taken NOC from the RHD to construct RCC drain at both side of the road. During site visit there is no objection were found from the consultation with the community people, and stockholder. However, in case of any objection from the community people, and stockholder, the DSM will investigate the actual situation. Then, measures will be taken as per rules.

Principal Livelihoods and Economic Activities

The subproject area is now inhabited by the mixed occupational people where major income comes from non-farm activities such as business, and enterprises. Presently, a significant number of people work in small trades, private sector jobs, and government jobs in the town.

Cultural Heritage and Protected Areas

There is no protected area and historical sites identified in the subproject area.

Social Conflicts and Development Activities

There are no visible conflicts between the local communities regarding the subproject implementation. There are some ongoing subprojects under Pourashava funded by government and different donor agencies. However, there is no visible conflict between these donors and agencies.

# ENVIRONMENTAL SCREENING

Environmental Screening Checklist, as adopted in Appendix C of the Environmental Management Framework (EMF) of the MGSP, was administered for identifying the impacts and their extents.

The screening data for the RCC Drain and Street Light have been formulated and are shown in below:

(2)  Potential environmental impact during construction phase:

- Ecological impacts:

10

- Felling of trees  			Significant □	Moderate □   	Minor □√ Number of trees

- Clearing of vegetation  		Significant □	Moderate □  	Minor □√

- Potential impact on species of 	Significant □  	Moderate □  	Minor □√

aquatic (i.e., water) environment

Due to implementation of the subproject, 10 numbers of planted trees will be cut down. Major trees to be cut down are: Mango, Shajina, Jackfruits and Eucalyptus etc. Most of the trees to be cut down are small size in term of length and diameter. There are few natural grown vegetation from Ch.1000-1500m. Since, the intensity of the vegetation coverage is not uniform. Hence, clearing of the vegetation is considered as minor. Anticipated impact on the species of aquatic environment is also minor because except throwing of the waste materials and accidental leakage and spillage of the oil and grease from subproject vehicles and equipment into the existing water bodies, the subproject activities do not have any direct impact on the aquatic environment.

- Physicochemical impacts:

- Noise and Air pollution        		Significant □     Moderate □√ 	Insignificant □

- Drainage congestion         		Very likely □     Likely □ 		Unlikely □ √

- Water pollution         			Significant □     Moderate □√	Insignificant □

- Pollution from solid/ construction wastes Significant □    Moderate □√	Insignificant □

- Water logging          			Significant □     Moderate □	Insignificant □ √

Use of hydraulic excavator, mechanical compaction machine, concrete mixer machine, vibrator machine, and mobilization of the equipment, vehicles movement for the transportation of the materials for the implementation of the subproject will have negative impact on the physicochemical parameter (noise and air). However, construction work will be performed section wise. Hence, it is anticipated that the impacts will be less, temporary and site specific. The generated dust during construction activities may disturb the local people if measures are not taken. The generated construction wastes, un-suitable materials and solid wastes may degrade the adjacent water bodies if not properly collected, disposed and thrown. Improper collection and disposal of the generated wastes materials also may degrade the quality of the surrounding environment and degrade the aesthetic value. The anticipated water logging problem during construction period is insignificant because most of the activities will be performed in the dry season. However, erratic rainfall may cause drainage congestion and water logging problem in the construction site. Therefore, pumping facilities will be provided to drain-out the storm water if required. The subproject will have no adverse impact on the topography, climate, soils & regional hydrology.

- Socio-economic impacts:

- Traffic congestion 			Very likely □  	Likely       □   	Unlikely     □√

- Health and safety 			Significant □  	Moderate □√	Insignificant  □

- Impact on archaeological and historical   Significant □  	Moderate □  	Insignificant □√

- Employment generation 		Significant □ 	Moderate □ √  	Insignificant  □

During construction, the subproject will have localized negative impact in traffic congestion. However, there is sufficient space between the proposed drain alignment and existing road. Hence, the construction activities does not interrupt the vehicles movement. Though, width of the link is narrow, but motorized vehicles movement is very few. Hence, less impact on the local traffic is anticipated. To divert water from right side to left side of the drain, a cross drain will bisect the Dinajpur Road. Thus, it may interrupts the traffic movement. However, it will be constructed within short time. The construction work will follow simple procedure with commonly used equipment. Therefore, impact on health and safety is anticipated as moderate. There is no archaeological and historical site within the influence area. The subproject will have positive impacts by generating employment opportunities for the local people and creating business opportunity of supplying construction materials, equipment, food and other necessary staffs to the work camp.

(3)  Potential Environmental Impact during Operational Phase:

(d) Ecological impacts:

- Potential impact on species of aquatic 	Significant □  	      Moderate □	      Minor  □√

(i.e., water) environment

The drain is designed for discharging the storm water. Hence, anticipated impact on the aquatic environment due to the operation of the drain is likely minor. However, if the drain carries pollutants from any sources, it may degrade the quality of the aquatic environment.

(e) Physicochemical impacts:

- Potential air quality                      Improvement □√     No-improvement     □     Deterioration □

- Noise level                                   Improvement □       No-improvement     □     No impact □ √

- Drainage congestion    	         	 Improvement □√     Minor Improvement □    No Impact     □

- Risk of Water pollution                 Significant     □       Moderate   □√   	      Minor            □

- Pollution from solid waste            Improvement □√     No-improvement     □     Minor            □

The new drain will minimize drainage congestion and water logging problem. Thus, it will provide better environment to the community people. The covered drain will minimize spreading of the bad odor from the drain. The storm water to be discharged through the drain may degrade the water quality of the outfall if carries pollutants from any sources. The covered drain will reduce throwing of the waste materials into the drain.

- Socio-economic impacts:

- Traffic                                  Improvement  □√   	 No-improvement    Adverse  □

- Safety 			     Improvement  □√   	 No-improvement    Adverse □

- Employment generation      Significant      □   	 Moderate  □    	      Minor     □ √

After construction, the subproject has significant positive impact. The drain will complete the drainage network. In addition, new drain prevents the accumulation of the stagnant water on the road surface. Consequently, it will enhance safety traffic operation and pedestrians’ movement.

(4)  Summary of the Possible Environmental Impacts of the Subproject:

The ecological impact due to the subproject implementation is minor. The subproject activities have negative impacts on the physicochemical environment especially on noise and solid wastes. The subproject activities may degrade the air quality to a limited extent. The wastes generated due to the construction activities may degrade the quality of the surrounding environment. In fact, the inputs of the subproject that may affect the environment will be mainly at construction phase and limited within the subproject boundary. Moreover, mitigation measures will be taken according to the EMP for minimizing the air, dust, and noise pollution. The socio-economic environment mainly includes health and safety and employment generation. More importantly, safety concern is an issue for the construction phase. This subproject will have positive impacts in terms of the generation of the employment and business activities by supplying construction materials and equipment

The anticipated environmental impact for the installation of the street light is insignificant. However, it has significant positive social impacts after installation. Short circuit and improper electrical connection of the street light may cause potential health and safety risk.

# Specific impact, mitigation, AND enhancement measures

From the environmental study, the probable impacts of the subproject are mainly caused by the key activities required for the implementation of the subproject, raw materials and equipment to be used for the execution of the activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

  - Dismantle Work, Site Clearing, Excavation and Earth Work

The drain construction work consists of earth excavation, earth filling, back filling, cutting and removal of the unsuitable materials. These works lead dust blowing, improper disposal of the wastes, noise and vibration which may disturb the local people.

Mitigation Measures

- Careful operation of the hydraulic excavator if used to avoid any undue disturbances to the roadside residents;

- Cover the exposed earth with fabric;

- Quick disposal of the wastes at the Pourashava designated dumping site at Dolagash and or Vagra;

## Tree Felling, Clearing of Vegetation and Ecological Impact

The proposed drain will be constructed on the existing right of way and mostly developed land. In addition, brick drain exists in most of the section. Hence, only 10 nos. of planted trees (Mango, Shajina, Jackfruits and Eucalyptus) will be cut down. Therefore, the overall ecological impact is insignificant due to the subproject implementation.

Enhancement Measures

- Tree plantation to compensate the felled down trees and enhance the ecological condition in the subproject area- preferably local fruits, flowers, medicinal and ornamental trees-Mango, Jackfruit, Jam, Kathbadam, Krisnachura,  Bokul, Jarul, Kadom, Neem, Mahogany, Palm Tree, Rain Tree, including protection, fencing and conservation during project defect liability period): Preferably at one side of the Dinajpur Road where space is available and or anywhere Pourashava owned suitable places in the subproject influence area - 50 nos. of the trees (fencing as per LGED rate schedule 5.26.14). Contractors will also be instructed by the consultant and PIU prior to the tree plantation work.

## Pollution from the Construction Materials

Dumping of the construction spoils, including accidental leakage of the oil, grease, and fuel in equipment yards is a significant hazard. Surface water and soil quality might be polluted from these contaminants. Even the people to be engaged for the construction activities might endanger the physical and human habitats of the area.

Mitigation Measures

- Safe transport, storage, and disposal of the construction materials, and the equipment have to be carried out in order to avoid the accidental spillage and loss;

- Fuels, lubricants, and other hazardous materials should store over raised platforms and  not directly on the ground;

- The playgrounds of the educational institutions should not use as a stockyard or work camp.

## Air Quality and Dust

During construction phase, air pollutants will be emitted from the equipment and construction vehicles are expected to remain low. Local residents in the vicinity of the work sites will be temporarily disturbed by the limited dust pollution. The overall impacts, however, are expected to remain low.

Mitigation Measures

- Water should be sprayed at the work site for dust control.

- Ensure sprinkle and cover stockpiles of loose materials (e.g., fine aggregates);

- Maintain adequate moisture content of soil and sand for transportation, compaction and  handling;

- Avoid use of dust generating equipment which produce significant amount of particulate matter far from the local residents;

- Ensure all subproject vehicles are in good operating condition.

## Noise and Vibration

Noise and vibration caused by the equipment and movement of the construction vehicles may temporarily disturb nearby residents. In this subproject, sensitive areas like roadside houses, educational institutes, offices and shops are likely to be affected from the noise, though the impacts are anticipated to be limited.

Mitigation Measures

- Transportation of the construction materials have to be carried during the scheduled times, and mainly during the day;

- If needed and applicable, all powered mechanical equipment and machineries will be fitted with noise abating gear such as mufflers for effective sound reduction.

## Water Quality

The water quality may deteriorate if the construction materials, construction wastes, effluent from work camps, and food waste are dumped in the roadside water body and low land.

Mitigation Measures

- Proper construction management including waste management as well as training of the operators and other workers should provide to avoid pollution of the water bodies;

- Construction waste will dispose properly (not in the ditch, ponds or River) for which contractor will be responsible.

## Occupational Health and Safety

The most important risks associated with the construction activities are listed below:

- Exposure to the sunlight- workers are being exposed to the sun for long hours;

- Exposure to the high temperature, and humidity for a long time resulting in dehydration;

- Contact with the hazardous substances and wastes pose risks of  the infections and diseases;

- Risk from the  traffic collision or accidents during operation of the equipment such as      hydraulic excavator and vehicles movement for the transportation activities of the subproject;

- Risks from head loads for carrying soil, construction materials and construction equipment;

- Risk associated to the sudden bad weather working conditions such as storm, thunder storm and earth quake etc.

General Requirements for the Workers’ Health and Safety

The key salient features of the general requirements for the workers’ health and safety stated are presented in Table 5.7.1.

Table 5.7.1:  General Requirements for the Workers Health and Safety

| Issues | Requirements |
| --- | --- |
| Health and Hygiene | - Cleanliness at the site premises and workers living places and at the Labor Shed; - Arrangement of the proper ventilation and temperature at the Labor Shed; - Protection against dust and furnace by using of the nose masks and covering of the head and body; - Proper disposal of the wastes and effluents; - Provision of the adequate latrines and separate toilets for the women; - Sufficient dustbins for the solid waste management system. |
| Safety and First Aid Box | - Using of the personal protective equipment (helmet, gloves, goggles, nose mask, safety boots); - Precautions during work on or near machineries in motion; - Head loads are prohibited; - First aid facilities should be provided and maintained; - Ensure first aid box; - The first aid kit should include adhesive bandages, regular strength pain medication, gauze, and low grade disinfectant. |
| Compensation for Accidents at Work | - Contractors will bear medical treatment costs. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation-(the amount of the compensation should be fixed considering the type of accidents). |
| Dust and Fumes | - For any dust, fumes, or other impurities likely to be injurious to the workers, effective measures shall be taken to prevent their accumulation and its inhalation by the workers. |
| Overcrowding | - No labor room should be overcrowded. |
| Latrines and Urinals | - Sufficient latrines shall be provided; - Latrines shall be maintained in clean and sanitary condition; - Latrines shall be adequately lighted and ventilated. |
| Disposal of Wastes and Effluents | - Proper disposal system for the solid waste and effluent is required. |

## Impacts on Social Environment and Common Property Resources

Through comprehensive study, it is revealed that impacts are expected not to be severe and to be largely manageable. The following Table 5.8.1 presents impacts on socio-economic environment and common property resources.

Table 5.8.1: Impacts on Social Environment and Common Property Resources

| Social Components | Impacts on IECs | Impact Significance |
| --- | --- | --- |
| Community Perception | The local community people welcome this subproject and there is no visible objection from them. | Significant (+ve) |
| Employment and Business Opportunity | Community feels happy because generally the local contractor will be engaged for the construction works which will create work opportunity for the skilled and non-skilled labor. The subproject will create business opportunity for the equipment and materials suppliers’. | Significant (+ve) |
| Community Order and Security | This subproject activity does not create any severe security problems to the local community and community people. | Minor (-ve) |
| Possible damage to infrastructure and facilities | Possible damage of the existing road infrastructure by the construction equipment and vehicles used in this subproject. | Minor (-ve) |
| New infrastructure and facilities | Improvement of the existing drain will enhance infrastructure facilities and installation of street light will ensure night time safety for the pedestrians. | Moderate (+ve) |
| Agriculture and Land Use | No direct impact on agriculture however, it could affect the adjacent land to a very limited extent. | Minor (-ve) |
| Landscape and Aesthetics | This subproject activity temporarily will degrade landscape and aesthetics values of the subproject area to a limited extent. | Minor (-ve) |
| Labor Habitat | Most of the labors will stay at the Labor Shed which will have impacts on the environment relates to the generation of the solid wastes, effluent, and water consumption. | Moderate (-ve) |
| Health Care | Workers may suffer from the dehydration problems, respiratory problem, and other health hazards. | Minor (-ve) |
| Accident | In case of road accidents by the vehicles to be used for the transportation may have serious negative impact. | Significant (-ve) |

Mitigation Measures

- Conduct dissemination with the local community about the subproject details;

- Continue liaison with the community leaders in order to maintain the community support;

- Engage local contractor and local people as much as possible for positive perception of the local community;

- Follow traffic rules to avoid any accidents;

- Transportation and mobilization of the equipments and construction materials avoiding peak hours and scheduled time;

- Ensure first aid facilities and effective use of personal protective equipments where applicable

## Labor Influx and Anticipated Impacts

The labor force and associated goods and services required for the construction of infrastructure civil works under this subproject cannot be fully supplied locally. The migration to and temporary settlement of laborers in the subproject, referred to as labor influx, carries an array of potentially positive and negative impacts in terms of demands on public infrastructure, utilities, housing and sustainable resource management and the strain on social dynamics.

Labor influx effects on host communities include positive impacts such as:

- The subproject activities will generate work opportunities for the local people and supplying of the construction materials, equipments, food and other necessary stuffs to the campsite;

- Improved infrastructure and public service access and availability whereby subproject investment catalyzes larger allocation of resources to a region, stimulating the development or expansion of infrastructure and public services.

Critical negative social risks include:

- Increase in criminal activity and alcohol and drug abuse, domestic violence, political attachment and violence, smuggling and robbery etc.;

- Increase in gender-based violence, including eve teasing, sexual harassment etc;

- Increases in communicable diseases, including respiratory problems, diarrheal diseases, vector-borne diseases (e.g., malaria), and sexually transmitted infections (e.g., HIV/AIDS, syphilis, gonorrhea, hepatitis B);

- Conflicts arising from increased demand on existing infrastructure, services, and utilities, including transportation, health, education, water and sanitation, waste management, public utilities and community, religious, and recreational facilities and loss of land for access routes.

The general environmental impacts of labor influx include pressure on the natural resources such as using of the water, electricity, other fuel for cooking, loss of land for the labor establishment, depletion of the water supply, sewage and waste water generation, degradation of the air quality, waste generation,  increased demands on the local energy and resources and noise pollution effects. The number of local and migrated people involved in the subproject activities can be only identified in the construction phase. Hence, these specific impacts will be quantified during construction work and environmental assessment report will be modified accordingly. However, the following safeguard measures are recommended to avoid any risk of labor influx:

- Inform local people about the subproject activities;

- Liaison with the community leaders in order to get community support;

- Engage local people as much as possible to minimize workers from outsiders;

- Monitor workers attitude and behavioral matter;

- Monitor the workers movement for avoiding any unexpected social activities (robbery, crime, political attachment and conflicts, drugs abuse);

- Inform and use local administration to get support if needed;

- Inform local utilities service providers (such as for new electricity connection REB or any other department);

- Ensure effective use of natural resources such as water, electricity, fuel, wood etc.

## Impacts on Traffic Movement

During drain construction work, it does not need closing of the entire road.  In addition, there is sufficient space between the proposed drain alignment and existing road. Hence, the construction activities does not interrupt the vehicles movement. Thus, less impact on the local traffic is anticipated. However, during construction phase, the subproject activities may interrupt normal traffic movement and free movement of the pedestrians especially near Dinajpur Road Moar.

During construction phase, interruption of the traffic movement and impact on the local traffic system due to the subproject activities will be monitored closely. Then separate traffic management plan will be provided if required. However, the following safeguard measures are recommended to minimize the impacts associated to the traffic movement:

- Inform local people about the subproject activities;

- Inspire local people to use connecting and diversion roads;

- Ensure schedule deliveries of material/ equipment during off-peak hours;

- Place traffic sign/cautionary sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption;

- The place of construction works should be fenced off with fences if required and should be isolated from general public access and marked with signs to ensure safe movement.

# Environmental Management Plan (EMP)

The purpose of the Environmental Management Plan (EMP) is to ensure that the activities are undertaken in a responsible and non-detrimental manner. The EMP will guide the environmentally sound construction of the subproject and ensure efficient lines of communication between PMU (LGED), PIU (Saidpur Pourashava), DSM, and the contractors.

## Access to Information

The environmental assessment report should be translated into Bengali and disseminated locally. The copies of the report (both in English and Bengali) will be sent to the concerned personnel responsible for subproject implementation. It will also be made available to the public. The final assessment report will also be uploaded in the LGED website and the World Bank website after approval.

## Grievance Redress Mechanism

The project-specific Grievance Redress Mechanism (GRM) has been established at Saidpur Pourashava to receive, evaluate, and facilitate the solution of APs concerns, complaints, and grievances concerning the social and environmental performance of the subproject. The GRM aimed to provide a time-bound and transparent mechanism to voice and resolve social and environmental concerns linked to the subproject.

The grievance mechanism is related to resolve the risks and adverse impacts of the subproject. It addresses APs’ concerns and complaints promptly, using an understandable and transparent process that is also gender responsive, and culturally appropriate. It is readily accessible to all the segments of the affected people at no costs and without retribution. The mechanism should not impede access to the country’s judicial or administrative remedies. The affected people will be appropriately informed about the mechanism.

LGED has its own Grievance Redress Procedure (GRP), which it operates to address any dissatisfaction and complaints by the local people regarding its activities. This procedure is being applied to address any complaints or grievances through negotiations with the community leaders and representatives of APs during implementation of the MGSP.

### Grievance Redress Committee (GRC)

The discussions and negotiations will be conducted by the Project Implementation Unit (PIU) of Saidpur Pourashava, and will be involved the APs and Grievance Redress Committee (GRC) headed by the Pourashava Mayor. With the facilitation of DSM, the Pourashava Mayor nominated the GRC members and included representative from Government Agencies, local NGO, and Civil Society. The GRC has been formed and established at Saidpur Pourashava. A complaints box has already been provided in the Pourashava Office. The grievance response focal point is available at Saidpur Pourashava for instant response to an aggrieved person. The Focal Point will collect the written complaints or suggestions from the box, and produce them to the GRC for hearing and resolution. The GRC members are as follows:

Table 6.2.1.1: List of GRC committee members

| SL. No. | GRC Members Name | GRC Designation | Position |
| --- | --- | --- | --- |
| 1. | Md. Amzad Hossain Sarker | Chairman | Mayor, Saidpur Pourashava |
| 2. | Md. Ayub Ali | Member secretary | Executive Engineer, Saidpur Pourashava |
| 3. | Ahamed Mahbub-Ul- Islam | Member | A.C. Land, Saidpur Upazila |
| 4. | Md. Anowarul Islam | Member | Headmaster, Saidpur Pilot high school |
| 5. | Md. Mahab-ul Alam | Member | Head Executive, Sharp NGO |
| 6. | Md. Ruhul Alam | Member | Representative of Civil society |
| 7. | Minara Begum | Member | Female word Councilor of Ward 10,11 and 12 |
| 8. | Md. Kazi Monower Hossain Hyder | Member | Councilor, Ward No.11 |

## 6.2.2 Grievance Resolution Process

All complaints and suggestions will be received formally in the Pourashava Office by the GRC Member Secretary. A sample Grievance Redress Form has been prepared and sent to Saidpur Pourashava prior to the implementation of the subproject.

An intake register will be maintained at the office of the Member Secretary. Member Secretary will record the details of the grievances in the intake register for documentation and ensure impartiality, fairness, and transparency. The intake register will have data and information columns including (i) Case no., (ii) Date of receipt, (iii) name, type of complaint, grievance, (iv) father’s name, husband’s name, (v) sex, (vi) complete address of the person raising the complaint, grievance, (vii) main objection (loss of land, property, or entitlement), (viii) detailed complaint story, (ix) expectation with documentary evidence and previous records of similar grievances, etc.

No GRC members are allowed to contact the aggrieved persons in advance. Rather, the concerned persons are informed to attend the formal hearings at an appointed date. The GRC committee will sit for hearing the complaints of the aggrieved persons. The GRC will record salient points presented by the aggrieved person and will examine documentary evidence submitted during informal hearings. A resolution register will be maintained by the Member Secretary at the Saidpur Pourashava Office. The resolution register will contain (i) serial no., (ii) case no., (iii) name of complaint, (iv) complaint story and expectation, (v) date of hearing, (vi) date of field investigation (if any), (vii) results of hearing and field investigation, (viii) decision of GRC, (ix) progress (pending, solved) and (x) agreement or commitments. Closing register will keep records such as, (i) serial no., (ii) case no., (iii) name of complaint, (iv) decision and response to complaints, (v) mode and medium of communication, (vi) date of closing, (vi) confirmation of complainant's satisfaction, and (vii) management actions to avoid recurrence.

The GRC will decide within 30 days of receiving a complaint. There will also be an appeals procedure where, if a person is dissatisfied with the ruling of the GRC, he or she or a representative may attend their next meeting to present the case again. The committee will then reconsider the case in private, after which their decision is final. If the appellant is still not satisfied, he or she has the right to take the case to the public courts. Saidpur Pourashava should also publish the outcome of cases on public notice boards. All costs involved in resolving the complaints (meetings, consultations, communication, and information dissemination) will be borne by Saidpur Pourashava.

Based on consensus, this procedure will help to resolve issues or conflicts amicably and quickly, saving the aggrieved persons from having to resort to expensive, time consuming legal action. The procedure will however, not pre-empt a person’s right to go to the courts of law.

## Institutional Arrangement for Safeguard Compliance

In the institutional arrangement procedure, Project Director, and Team Leader/ Deputy Team Leader will directly involve. The PD, TL/DTL would be supported by DSM Environmental Safeguard Specialist and Social Management Specialist. The Pourashava Officials, especially engineers, would be responsible for supporting the construction supervision with the facilitation of DSM consultants. The civil works contractors will implement the environmental mitigation measures.

The PMU (LGED), with the facilitation of Environmental Safeguard Specialist and Social Management Specialist, will submit the monthly and quarterly progress reports on Environmental and Social Compliances to World Bank.

PD-MGSP

TL/DTL-DSM

Environmental Safeguard Specialist-DSM

Social Management Specialist-DSM

Senior Municipal Engineer-DSM

Junior Environmental Specialist-DSM

Assistant Municipal Engineer-DSM

Social Management Officer-DSM

Pourashava Representative/ Focal Point/ Asst. Engineer

Contractor

Figure 6.3.1: Environmental and Social Management Team (Tentative)

## Capacity Building

A demonstration training program has already been conducted by PMU (LGED) and DSM Consultants at Comilla City Corporation on 16 May 2016. Similar capacity building workshop will also be conducted by PMU and DSM Consultants in the Rangpur region for the capacity building. This workshop will be conducted by the PMU (LGED) and DSM Consultants. The participants from the Saidpur Pourashava will attend this training program. Under this training program, PMU and DSM will organize an introductory course for the training and preparing the participants on: (i) Environmental Screening, (ii) EMP Implementation, including environmental monitoring requirements related to mitigation measures; and (iii) taking immediate action to remedy unexpected adverse impacts or ineffective mitigation measures found during the course of the implementation. The contractor should be also included in the training program to enhance the Environmental awareness and orientation among the workers before deploying to the work sites in order to achieve the expected standards. A detailed training manual has already been developed by the Environmental Safeguard Specialist and Social Management Specialist prior to the demonstration training program in Comilla Region. Hence, the existing manual will be reviewed and modification will be done as per requirement.

## Emergency Response and Disaster Management

Disaster management can be defined as the organization and management of
resources and responsibilities for dealing with all humanitarian aspects of emergencies, in particular the preparedness, response, and recovery to lessen the impact of disasters. Emergency Preparedness Planning (EPP) and Contingency Planning (CP) are the processes of disaster management plan for developing strategies, arrangements, and procedures to address the humanitarian needs of those adversely affected by the crisis. There are four main types of disasters, namely: Natural disasters, Environmental Emergencies, Complex Emergencies, and Pandemic Emergencies.

For MGSP activities, Saidpur Pourashava would identify the immediate needs, prioritize the tasks, and identify resource requirements to address the humanitarian needs of those adversely affected by the crisis.

## Environmental Management Action Plan

The activity wise anticipated environmental impacts and corresponding mitigation measures have been outlined in Table 6.6.1 and 6.6.2.

Table 6.6.1: Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures (EMP Table)

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for the workers (Workforce and labor shed management ) | - Generation of sewage and solid waste may cause environmental pollution in the surrounding area | - Construction of sanitary latrine (considering 15 person for one toilet) at the labor shed and separate toilet for women; - Introduce separate waste bins for organic and inorganic waste materials at camp site; - Ensure wastes (solid wastes and other forms of the waste) disposal in the Vagra dump site. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. - Comply with requirements of Government of Bangladesh labor law of 2006 and all applicable laws and standards on worker’s Health and Safety. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well-being of the local people. |  |
| General construction work | - Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water, if needed; - Ensure provision for pumping of congested water, if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season. - Ensure select place for the storage of soils and other construction materials to avoid disturbance of natural drainage. - Dispose of construction materials and equipment appropriately so that they do not impede the local drainage. |  |
|  | - Air pollution | - Check regularly and ensure that all the subproject vehicles and equipment are in good operating condition; - Ensure contractor spray water on dry surfaces to reduce dust generation; - Ensure contractor sprinkle water and cover stockpiles of loose materials (e.g., fine aggregates); - Inform local people about the subproject activities and potential impact; |  |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Place traffic sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption; - Inform the local people about the subproject activities; - Avoid roadside storage of construction material; - Inspire local people for using of the connecting roads during construction period; - Pourashava should be ensured a flag man to regulate the traffic movement where drain will bisect the Dinajpur Road. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment such as concrete mixer machine, vibrator machine that produce excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles; - Inform local people about the subproject activities and potential impact. |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 50 nos. of the trees will be planted at the both sides of the Dinajpur Road where space is available to compensate the felled down trees and to enhance the ecological condition. |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accident; - Provides PPEs and ensure using of the personal protective equipment by the workers - Maintain the register to record accidental events if occur; |  |
|  | - Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil; - Collection and proper disposal of spills; - Refueling and storage of fuel tank should not be placed on the ground directly. - Ensure use of polythene or wooden platform below the refueling tank and storage of fuel tank. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures- tree plantation and cautionary sign will be provided; - Ensure complain box to collect the complaints from the affected person due to subproject activities; - Front size of cautionary sign to be large and shall be visible from 200m distance; - Placing of the cautionary sign in the strategic point such as road intersection; - For notification of construction activities, at least two signs shall be put up for each road, one close to the point of carriageway begins, and another will put on the end of the carriageway. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the RCC Drain |  |  |  |
| Dismantling Work ( Existing brick drain) for site clearing and Excavation Work | - Generation of loose and clay soil due to the earth excavation work. | - Cover exposed loose dry soil and wastes materials before disposal; - Disposal of soil and construction wastes at existing Vagra dumping site |  |
|  | - Accidents from careless use of hydraulic excavator and hammer | - Carefully handle of the hydraulic excavator and hammer if needed. |  |
|  | - Possible damage of road side infrastructure due to earth excavation for drain construction. | - Ensure drum sheet palisading work for shallow depth to stabilize the structure; - Ensure plunk palisading work for shallow depth to stabilize the structure; - Bolly drive for deep depth construction works. |  |
|  | - Air pollution due to black smoke emission from excavator. | - Regular maintenance of the equipment. |  |
| Sand filling /Back filling work | - Air and dust pollution affecting nearby settlement and worker | - Maintain adequate moisture content of soil and sand during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |
| Cutting and welding of the reinforcement | - Noise pollution due to rod cutter and welding machine if any | - Avoid using of rod cutter and wielding machine at night; - Avoid prolonged exposure to noise (produced by equipment) by workers. |  |
|  | - Potential health and safety risks from rod cutter and welding machine if any | - Ensure use of the personal protective equipment’s (helmet, goggles, gloves, safety boot); - Availability and access to first-aid equipment and medical supplies in case of any accidents. |  |
| RCC (reinforcement cement concrete) work | - Air pollution due to black smoke emission from concrete mixer machine and vibrator machine | - Regular maintenance of the concrete mixer and vibrator machine to avoid any black smoke emission. |  |
|  | - Health hazard may occur due to spread out of dust and cement material during mixing of aggregate | - Ensure use of masks for construction workers if required. - Ensure saturated fine material i.e. sand to dispose into the mixture machine if possible |  |
|  | - Noise nuisance from concrete mixer machine and vibrator machine | - Avoid operation of the concrete mixer and vibrator machine at night; - RCC work should be avoided at schooling time; - Inform local people about casting work and potential impacts. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the Street light |  |  | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM |
| Setting up the pole and electrical connection | - Potential health and safety risk | - Inform the local authority to switch off power during connection; - Ensure use of the PPEs. |  |
| Source of electricity and equipment | - Reduce of resource i.e., use of electricity | - Provision of renewable energy (solar panel electrification)and use of environmental friendly bulb ( LED bulb rather than CFL bulb |  |

Table 2: Anticipated environmental impacts during operation phase and corresponding mitigation and enhancement measures

| Activity/Issues | Potential Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Operation and maintenance for drain | - Pollution of downstream water body due to disposal of polluted water from the drain | - Ensure installation of septic tank by the household people in all establishment; - Avoid connecting sanitation facilities to storm drain. | Monitoring- Primarily by Pourashava |
|  | - Possible backflow of the drain water due to blockage in the drain | - Proper maintenance and cleaning of the drain and outfall on regular basis. |  |
|  | - Throwing of the wastes in to the drainage system | - Awareness program for the community people should be taken to increase knowledge about improper waste disposal and its impact. |  |
| Operation and maintenance for street light | - Accidents due to collapse of the arms, electric bulbs and pole | - Monthly checking and maintenance of the arms, switch box, electric bulbs if needed; - Provision of automatic shut-down the switch, lamps during thunder storm and other natural disasters. |  |
|  | - Traffic congestion, traffic problems for maintenance works | - Schedule deliveries of materials/ equipment’s/work during off-peak hours. |  |

## Environmental Monitoring Plan

Environmental Monitoring Plan for this subproject will help to evaluate the extent and severity of environmental impacts against the predicted impact and the performance of environmental protection measures. The following Table 6.7.1 has been prepared for the key environmental indicators.

Table 6.7.1: Matrix Table of Monitoring Plan (Visual observation during construction phase)

| Monitored Parameter/ Issues | Monitoring Method/ Key Aspects | Location of Monitoring | Period & Monitoring Frequency |
| --- | --- | --- | --- |
| Safety orientation & training of workers | Frequency of training & orientation of workers for safety | Subproject site | - Once in a month - Reporting: Once in a month |
| Personal Protective Equipment | Ensure every single person involved in the activities wear and use safety equipment | Subproject site | - Daily - Reporting: Once in a month |
| Worker’s health | Monitoring process of worker’s health | Subproject site | - Daily - Reporting: Once in a month |
| Sanitation & drinking water facility to the workers | Availability of safe drinking water and sanitation to the workers | Subproject site | - Daily - Reporting: Once in a month |
| Incident record & reporting | Documented record of all incident, accident, its remedial process | Subproject site | - Daily - Reporting: Once in a month |
| Site security | Isolation of site from general access | Subproject site | - Daily - Reporting: Once in a month |
| Bulletin/ announcement boards/ prohibition signs | Visible in good condition or not | Subproject site | - Daily - Reporting: Once in a month |
| Equipment /vehicles | -Switched-off diesel engines when not in use; -Search any possible leakage; -Fuelling. | Subproject site | - Daily - Reporting: Once in a month |
| Dust | Dust is visible or not | Subproject site | - Daily - Reporting: Once in a month |
| Oil waste generation and disposal | Quantity of oily waste, storage and disposal | Subproject site | - Daily - Reporting: Once in a week |
| Solid waste generation | Quantity of solid wastes and disposal | Subproject site | - Daily - Reporting: Once in a month |
| Drainage facilities | Provision of open /closed surface drainage | Subproject site | - Monthly - Reporting: Once in a month |
| Gender equity | No discrimination regarding payment | Subproject site | - Daily - Reporting: Once in a month |
| Child labor | No child will be engaged in the activities | Subproject site | - Daily - Reporting: Once in a month |
| Handling of hazardous materials | Fuelling, storage, operation | Subproject site | - Daily - Reporting: Once in a month |

In addition, a comprehensive Environmental Compliance Monitoring checklist has been preparing that is enclosed in Appendix-I.

## Environmental Management Budget

Considering the environmental impacts and their mitigation measures for this subproject, several items are included in the BOQ for environmental management. The estimated cost for environmental management is elaborated in Table 6.8.1.

Table 6.8.1: Environmental Management Budget

| Item No. | Description of the Works | Unit Rate (BDT) | Qty (Number) | Amount (BDT) |
| --- | --- | --- | --- | --- |
| eme-1 | Dust suppression measures by water spraying throughout the construction period in and around the subproject sites, uncovered aggregates and loose materials such as stockpiles of the sand, excavated earth etc. | Lumpsum |  | 10,000.00 |
| eme-5 | Prevention of the spillage and leakage of the polluting materials (Detailed procedure will be given in the EMP) | Lumpsum |  | 5,000.00 |
| eme-6 | Campsite wastes disposal facility during the construction period (collection, transportation and dumping of the wastes at Vagra or Dolagash dumping site): 2 nos. (1 no. for the organic wastes and 1 no. for the inorganic wastes disposal facility) | 20,000.00 | 2.00 | 40,000.00 |
| eme-7 | Campsite water supply facilities: Preferably 1 no. of tube well at the labor campsite (Depending on the site condition, DSM consultant will assist the contractor for selecting the option) | 20,000.00 | 1.00 | 20,000.00 |
| eme-8 | Campsite sanitation facilities: 2 nos. of the toilets preferably sanitary toilets at the labor campsite (1 no. for women and 1 no. for men) | 20,000.00 | 2.00 | 40,000.00 |
| eme-9 | a) Providing safety gear packages like hand gloves, spectacles for eye protection, ear plug, helmets, masks, visible jacket, safety shoes for at least 30 persons (20 workers and 10 visitors) | 3,000.00 | 30.00 | 90,000.00 |
|  | b) One first aid box with necessary accessories (contractor is responsible for providing necessary medicines, saline as per requirement during construction period) | 2,500.00 | 1.00 | 2,500.00 |
| eme-11 | Tree plantation to compensate the felled down trees and enhance the ecological condition in the subproject area- preferably local fruits, flowers, medicinal and ornamental trees-Mango, Jackfruit, Jam, Bel, Kathbadam, Krisnachura, Bokul, Jarul, Kadom, Neem, Mahogany, Palm Tree, Rain Tree, including protection, fencing and conservation during project defect liability period): Preferably at one side of the Dinajpur Road where space is available and or anywhere Pourashava owned suitable places in the subproject influence area - 50 nos. of the trees (fencing as per LGED rate schedule 5.26.14) (Tree plantation and fencing detailed will be given in the EMP. Contractors will also be instructed by the consultant and PIU prior to the tree plantation work) | 750.00 | 50.00 | 37,500.00 |
| eme-14 | Cautionary signs-6 nos. (Detailed specifications will be given in the EMP) | 2,500.00 | 6.00 | 15,000.00 |
|  | Total |  |  | 260,000.00 |

After approval to revise the cost estimate has lengthy complex procedure. Hence, as per project EMF, PMU suggestion and experience from other LGED projects, adequate budget has been allocated for the environmental management for the mitigation and enhancement measures. The subproject activities such as earth work, sand filling, black smoke emission from the subproject vehicles & equipment, etc. may degrade the air quality in the subproject area. In addition, mechanical compactor, hydraulic excavator, concrete mixer machine, vibrator and subproject vehicles may generate noise nuisance to the surrounding area. For this subproject, there is no provision of the analytical monitoring because air, noise and waste water quality data from the previous subproject (MGSP/SAD/2015-2016/W3) can be used as a baseline data to evaluate the impacts.

To enhance the ecological condition, adequate budget for tree plantation is included. The budget for labor shed and site office construction is included in the civil works items. Therefore, it is not included in the environmental budget. It should be noted that the contractor will be paid as per actual work done.

# Public Consultation and participation

## Methodology

In the context of preparing the Environmental Assessment (EA), participatory public consultation was conducted. The Pourashava Mayor, Officials, Engineers and Local Individuals as well as LGED and Consultant Participants participated. Informal Focus Group Discussions (FGD) and a formal CIP were conducted involving the participants. In addition, walk-through informal group consultations were also held. The local communities were informed about subproject interventions including their benefits. Suggestions made by the participants were listed and incorporated in the EMP accordingly.

Photographs 7.1.1: Consultation Meeting at Subproject Site with Stockholder, Local

People, Consultant and Pourashava Representatives.

## Issues Raised by the Participants

The participants raised the issues related to the infrastructure development of Saidpur Pourashava. They emphasized on the subproject selection for the future development and also discussed about the procedure for the quality construction work. In the FGD, the participants discussed the requirements of the infrastructure development of Saidpur Pourashava through a list of the subprojects.

## Feedback, Suggestions, and Recommendations of the Participants

The participants were presented with feedback, suggestions, and recommendations listed below:

- The FGD results confirmed that an improved communication network, drainage facility, and solid waste management is needed for future development of Saidpur Pourashava;

- Most of the participants stated that the number of subproject that have been selected for each financial year is insufficient;

- The Mayor and other Pourashava Officials emphasis for the construction of the Poura Office Building;

- The Mayor of the Pourashava stated that recreation center like children park is necessary for this Pourashava;

- The participants stated that the public water supply facilities, sanitation facilities, and drainage system is not adequate;

- The participants also addressed the solid waste management issue to reduce environmental and public health hazards.

# Conclusions and Recommendations

The anticipated impacts due to the subproject activities are relatively minor in comparison to the significant benefits that will derive due to the implementation of the subproject. In addition, the impacts will be minimized through the mitigation measures and monitoring by Pourashava, PMU (LGED) and DSM consultants. Furthermore, tree plantation will enhance the ecological condition in the subproject area.

A few key recommendations are outlined below:

- Regular monitoring by Pourashava, PMU (LGED) and DSM consultants is needed;

- The contractor will ensure using of the PPEs by the workers where applicable;

- Regular monitoring by Pourashava is needed to avoid any illegal households and toilets connection to the drain;

- The contractor is responsible for the proper disposal of the generated wastes materials from the subproject sites and Campsite.

It should be noted that environmental assessment report is a live document. Hence, due to changing circumstances during the construction phase if any, there might be minor adaptation needed for environmental, health and safety issues.

### APPENDIX

### Appendix- I: Environmental Monitoring Checklist

Local Government Engineering Department

Municipal Governance Services Project (MGSP)

Environmental Compliance Monitoring Form

Part A: General Subproject Information

| Subproject Name | (a) Construction of RCC Drain from Dinajpur Road Moar to Dohola Bridge (Ch.0-2000m) including link length 800m (Total Effective Length 4150m, Left & Right side). (b) Supplying, Fitting & Fixing of Street Light from Dinajpur Road to Khorkhoria River Bridge (Total Length 2800m). |
| --- | --- |
| Package No. | MGSP/SAD/2018-19/W-10 |
| ULB Name | Saidpur Pourashava |
| Approved Estimated Cost in BDT |  |
| Contract Amount in BDT |  |
| Contractor Name |  |
| Date of Commencement |  |
| Target Date of Completion |  |
| Physical Progress (%) |  |
| Financial Progress (%) |  |
| Person Responsible (PIU) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Person Responsible (DSM) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Form Completed by (Name, Designation &Phone) |  |
| Overall Environmental Description of the Subproject |  |

Part B: Design, Preparation, and Legal Requirements

| Environmental Concerns (PMU, PIU & DSM) (Name, Designation & Phone) | PMU- |  |
| --- | --- | --- |
|  | PIU- |  |
|  | DSM- |  |
| Subproject Category | DoE-BD- | WB- |
| Environmental Clearance Received? | Yes | No |
| EA Required? | Yes | No |
| EA Prepared and Delivered? | Yes | No |
| EMP Prepared & Delivered Separately? | Yes | No |
| Items and Cost of EMP Implementation Included in the Contract? | Yes | No |
| EMP Included in the Procurement Documents? | Yes | No |
| Inspection Schedule/Last Inspection/Monitoring by PMU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by PIU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by DSM Environmental Safeguard Specialist/Jr. Environmental Specialist | Date- |  |
|  | Key findings- |  |

Part C: Key Environmental Impacts

| The Subproject Results in any of the following Impacts? | Yes | No | NA | If yes, is the impact (give observation) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Significant | Moderate | Minor |
| Felling of the trees |  |  |  |  |  |  |
| Clearing of the vegetation that increase the risk of increased soil degradation or erosion |  |  |  |  |  |  |
| Disturbance of the terrestrial and or aquatic specifies |  |  |  |  |  |  |
| Noise pollution |  |  |  |  |  |  |
| Air pollution |  |  |  |  |  |  |
| Adverse effects on the quantity or quality of the surface water or groundwater |  |  |  |  |  |  |
| Production or increase the production of the solid waste |  |  |  |  |  |  |
| Drainage congestion |  |  |  |  |  |  |
| Water logging that increases the risk of the water related diseases |  |  |  |  |  |  |
| Traffic congestion |  |  |  |  |  |  |
| Public safety |  |  |  |  |  |  |

Part D: Work Place Environment and Gender Equity

| The Subproject Results in any of the following Impacts? | Yes | No | NA | Observations |
| --- | --- | --- | --- | --- |
| Does the contractor pay to the workers regularly? |  |  |  |  |
| Is there any discrepancy between the male and female workers regarding the wages or salary for the same works? |  |  |  |  |
| Is the contractor complying with the GOB labor law concerning the hiring of the workers? |  |  |  |  |
| Does the contractor engage women labors and does the project have suitable works for them? |  |  |  |  |
| Does the contractor engage child labor (less than 18 years) and aged people (more than 65 years old)? |  |  |  |  |
| Does the contractor force to the workers for the completion of the works? |  |  |  |  |
| Do the workers involve with the political activities, crime, drugs addiction and other forms of unwanted activities? |  |  |  |  |
| Are construction camps adequately equipped with water supply, sanitary toilets, washing facilities and facilities for waste collection and storage? |  |  |  |  |
| Has separate sanitation facilities been provided for women at work camps and the construction site? Do the laborers load heavy items on their heads or shoulders? |  |  |  |  |
| Has the contractor undertaken an awareness program for the sexually transmitted diseases especially for HIV-AIDS and other infectious diseases like TB? |  |  |  |  |
| Are first aid kits readily available for the workers at the job site along with the instructions for use? |  |  |  |  |
| Are supervisors or other site personnel trained in the basic first aid emergency response measures? |  |  |  |  |
| Has the contractor provided necessary safety equipment to the workers and training for use? |  |  |  |  |

Part E: Potential Impacts, Mitigation Measures, and Monitoring Indicator Mentioned in the EMP

| Activity/ Issues | Potential Impacts | Proposed Mitigation and Enhancement Measures | Monitoring Method | Monitoring Frequency | Implementation Status (Yes/No /NA) | Impact significance if not implemented |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Major | Moderate | Minor | No |  |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | - Generation of sewage and solid waste may cause environmental pollution in the surrounding area | - Construction of sanitary latrine (considering 15 person for one toilet) at the labor shed and separate toilet for women; - Introduce separate waste bins for organic and inorganic waste materials at camp site; - Ensure wastes (solid wastes and other forms of the waste) disposal in the Vagra dump site. |  |  |  |  |  |  |  |  |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. - Comply with requirements of Government of Bangladesh labor law of 2006 and all applicable laws and standards on worker’s Health and Safety. |  |  |  |  |  |  |  |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |  |  |  |  |  |  |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well-being of the local people. |  |  |  |  |  |  |  |  |
| General construction works | - Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water, if needed; - Ensure provision for pumping of congested water, if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season; - Ensure select place for the storage of soils and other construction materials to avoid disturbance of natural drainage; - Dispose of construction materials and equipment appropriately so that they do not impede the local drainage. |  |  | P |  |  |  |  |  |
|  | - Air pollution | - Check regularly and ensure that all the subproject vehicles and equipment are in good operating condition; - Ensure contractor spray water on dry surfaces to reduce dust generation; - Ensure contractor sprinkle water and cover stockpiles of loose materials (e.g., fine aggregates); - Inform local people about the subproject activities and potential impact; |  |  |  |  |  |  |  |  |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Place traffic sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption; - Inform the local people about the subproject activities; - Avoid roadside storage of construction material; - Inspire local people for using of the connecting roads during construction period; - Pourashava should be ensured a flag man to regulate the traffic movement where drain will bisect the Dinajpur Road. |  |  |  |  |  |  |  |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment such as concrete mixer machine, vibrator machine that produce excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles; - Inform local people about the subproject activities and potential impact. |  |  |  |  |  |  |  |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 50 nos. of the trees will be planted at the both sides of the Dinajpur Road where space is available to compensate the felled down trees and to enhance the ecological condition. |  |  |  |  |  |  |  |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accident; - Provides PPEs and ensure using of the personal protective equipment by the workers - Maintain the register to record accidental events if occur; |  |  |  |  |  |  |  |  |
|  | - Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil; - Collection and proper disposal of spills; - Refueling and storage of fuel tank should not be placed on the ground directly; - Ensure use of polythene or wooden platform below the refueling tank and storage of fuel tank. |  |  |  |  |  |  |  |  |
|  | - Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water, if needed; - Ensure provision for pumping of congested water, if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season; - Ensure select place for the storage of soils and other construction materials to avoid disturbance of natural drainage; - Dispose of construction materials and equipment appropriately so that they do not impede the local drainage. |  |  |  |  |  |  |  |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |  |  |  |  |  |  |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures- tree plantation and cautionary sign will be provided; - Ensure complain box to collect the complaints from the affected person due to subproject activities; - Front size of cautionary sign to be large and shall be visible from 200m distance; - Placing of the cautionary sign in the strategic point such as road intersection; - For notification of construction activities, at least two signs shall be put up for each road, one close to the point of carriageway begins, and another will put on the end of the carriageway. |  |  |  |  |  |  |  |  |
| Measures for the RCC drain |  |  |  |  |  |  |  |  |  |  |
| Dismantling Work ( Existing brick drain) for site clearing and Excavation Work | - Generation of loose and clay soil due to the earth excavation work. | - Cover exposed loose dry soil and wastes materials before disposal; - Disposal of soil and construction wastes at existing Vagra dumping site |  |  |  |  |  |  |  |  |
|  | - Accidents from careless use of hydraulic excavator and hammer | - Carefully handle of the hydraulic excavator and hammer if needed. |  |  |  |  |  |  |  |  |
|  | - Possible damage of road side infrastructure due to earth excavation for drain construction. | - Ensure drum sheet palisading work for shallow depth to stabilize the structure; - Ensure plunk palisading work for shallow depth to stabilize the structure; - Bolly drive for deep depth construction works. |  |  |  |  |  |  |  |  |
|  | - Air pollution due to black smoke emission from excavator. | - Regular maintenance of the equipment. |  |  |  |  |  |  |  |  |
| Sand filling /Back filling work | - Air and dust pollution affecting nearby settlement and worker | - Maintain adequate moisture content of soil and sand during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. |  |  |  |  |  |  |  |  |
| Cutting and welding of the reinforcement | - Noise pollution due to rod cutter and welding machine if any | - Avoid using of rod cutter and wielding machine at night; - Avoid prolonged exposure to noise (produced by equipment) by workers. |  |  |  |  |  |  |  |  |
|  | - Potential health and safety risks from rod cutter and welding machine if any | - Ensure use of the personal protective equipment’s (helmet, goggles, gloves, safety boot); - Availability and access to first-aid equipment and medical supplies in case of any accidents. |  |  |  |  |  |  |  |  |
| RCC (reinforcement cement concrete) work | - Air pollution due to black smoke emission from concrete mixer machine and vibrator machine | - Regular maintenance of the concrete mixer and vibrator machine to avoid any black smoke emission. |  |  |  |  |  |  |  |  |
|  | - Health hazard may occur due to spread out of dust and cement material during mixing of aggregate | - Ensure use of masks for construction workers if required. - Ensure saturated fine material i.e. sand to dispose into the mixture machine if possible; |  |  |  |  |  |  |  |  |
|  | - Noise nuisance from concrete mixer machine and vibrator machine | - Avoid operation of the concrete mixer and vibrator machine at night; - RCC work should be avoided at schooling time; - Inform local people about casting work and potential impacts. |  |  |  |  |  |  |  |  |
| Measure for the Street Light |  |  |  |  |  |  |  |  |  |  |
| Setting up the pole and electrical connection Source of electricity and equipment | - Potential health and safety risk | - Inform the local authority to switch off power during connection; - Ensure use of the PPEs. |  |  |  |  |  |  |  |  |
|  | - Reduce of resource i.e., use of electricity | - Provision of renewable energy (solar panel electrification)and use of environmental friendly bulb ( LED bulb rather than CFL bulb |  |  |  |  |  |  |  |  |

Prepared by-                                                                                                 Signature-                                                                   		       Date-

Copies to 1.  PIU

2.  SME

3.  AME

4. Jr. ES