| GOVERNMENT OF THE PEOPLE’S REPUBLIC OF BANGLADESH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Local Government Engineering Department (LGED) Local Government Division Ministry of Local Government, Rural Development and Cooperatives |  |  |  |  |
| Environmental Assessment (EA) REPORT Name of the Subproject: Construction of BC Road and RCC Drain at Akur Takur Para and Beautification Work at Soyani Pond Package No: MGSP/TAN/2017-2018/W-13 Tangail Pourashava , Tangail Municipal Governance and Services Project (MGSP) Design, Supervision, and Management (DSM) Consultancy Services |  |  |  |  |
| Joint Venture of Hifab International AB, Sweden AQUA Consultant and Associates Ltd., Bangladesh |  |  |  |  |

TABLE OF CONTENTS

|  |  |  |  | Page |
| --- | --- | --- | --- | --- |
| 1.0 |  | INTRODUCTION |  | 1-3 |
|  | 1.1 | Subproject Background |  | 1 |
|  | 1.2 | Objectives of the Study |  | 2 |
|  | 1.3 | Scope and Methodology of the Study |  | 2 |
| 2.0 |  | SUBPROJECT DESCRIPTION |  | 4-11 |
|  | 2.1 | The Study Area |  | 4 |
|  | 2.2 | Current Situation, Proposed Intervention and Need for the Subproject |  | 8 |
|  | 2.3 | Justification of Selection of this Subproject |  | 9 |
|  | 2.4 | Envisaged Subprojects Activities and Implementation Process |  | 9 |
|  | 2.5 | ## Category of the Subproject |  | 10 |
|  | 2.6 | Subproject Schedule |  | 11 |
| 3.0 |  | BASELINE ANALYSIS OF ENVIRONMENTAL CONDITION |  | 12-18 |
|  | 3.1 | Physical Environment |  | 12 |
|  | 3.2 | Biotic Environment |  | 16 |
|  | 3.3 | Socio-economic and Socio-cultural Environment |  | 17 |
| 4.0 |  | ENVIRONMENTAL SCREENING |  | 19-21 |
| 5.0 |  | SPECIFIC IMPACT, MITIGATION AND ENHANCEMENT MEASURES |  | 22-28 |
|  | 5.1 | Dismantle Work, Site Clearing, Excavation Work and Earth Work |  | 22 |
|  | 5.2 | Tree Felling, Clearing of the Vegetation and Ecological Impact |  | 22 |
|  | 5.3 | Pollution from Construction Materials |  | 22 |
|  | 5.4 | Air Quality and Dust |  | 23 |
|  | 5.5 | Noise and Vibration |  | 23 |
|  | 5.6 | Water Quality |  | 24 |
|  | 5.7 | Occupational Health and Safety |  | 24 |
|  | 5.8 | Impacts on Social Environment and Common Property Resources |  | 25 |
|  | 5.9 | Labor Influx and Anticipated Impacts |  | 26 |
|  | 5.10 | Impacts on Traffic Movement |  | 27 |
| 6.0 |  | ENVIRONMENTAL MANAGEMENT PLAN |  | 29-43 |
|  | 6.1 | Access to Information |  | 29 |
|  | 6.2 | Grievance Redress Mechanism |  | 29 |
|  |  | 6.2.1 | Grievance Redress Committee (GRC) | 29 |
|  |  | 6.2.2 | Grievance Resolution Process | 30 |
|  | 6.3 | Institutional Arrangement for Safeguard Compliances |  | 31 |
|  | 6.4 | Capacity Building |  | 32 |
|  | 6.5 | Emergency Response and Disaster Management |  | 32 |
|  | 6.6 | Environmental Management Action Plan |  | 33 |
|  | 6.7 | Environmental Monitoring Plan |  | 40 |
|  | 6.8 | Environmental Management Budget |  | 41 |
|  | 6.9 | ## Environmental Codes of Practice |  | 42 |
| 7.0 |  | PUBLIC CONSULTATION AND PARTICIPATION |  | 44-45 |
|  | 7.1 | Methodology |  | 44 |
|  | 7.2 | Issues Raised by the Participants |  | 44 |
|  | 7.3 | Feedback, Suggestions and Recommendations of the Participants |  | 45 |
| 8.0 |  | CONCLUSIONS AND RECOMMENDATIONS |  | 46 |
| 9.0 |  | # References |  | 47 |
| APPENDIX |  |  |  | 48 |
| Appendix I: Environmental Monitoring Checklist |  |  |  |  |

LIST OF TABLES

| - Table 3.1.1 | Major Environmental and Infrastructural Features from Akur Takur Para Road and Soyani Pond area |  |
| --- | --- | --- |
| - Table 3.1.2 | Average Annual Temperature, Rainfall Data |  |
| - Table 5.7.1 | General Requirements for the Workers Health and Safety |  |
| - Table 5.8.1 | Impacts on Social Environment and Common Property Resources |  |
| - Table 6.2.1.1 | List of GRC Committee Members |  |
| - Table 6.6.1 | Anticipated Environmental Impacts During Construction Phase and Corresponding Mitigation and Enhancement Measures |  |
| - Table 6.6.2 | Anticipated Environmental Impacts During Operational Phase and Corresponding Mitigation and Enhancement Measures |  |
| - Table 6.7.1 | Matrix Table of Monitoring Plan (Visual Observation During Construction Phase) |  |
| - Table 6.8.1 | Environmental Management Budget |  |

LIST OF FIGURES

| Figure 2.1.1 | - Base Map of the Subproject Sites |  |
| --- | --- | --- |
| Figure 2.1.2 | - Layout plan of the Akur Takur Para Choto Kalibari Road and Soyani Pond Area |  |
| Figure 2.1.3 | - Topographical View of the Akur Takur Para Choto Kalibari Road |  |
| Figure 2.1.4 | - Topographical View of the Soyani Pond |  |
| - Figure 3.1.1 | - General Soil classification of Bangladesh |  |
| - Figure 3.1.2 | - Earthquake Zones Map and Soil Region Map of Bangladesh |  |
| Figure 6.3.1 | Environmental and Social Management Team (Tentative) |  |

LIST OF PHOTGRAPHS

| Photograph 2.2.1 | Current Situation of the proposed Road and Drain |  |
| --- | --- | --- |
| Photograph 2.2.2 | Current Situation of the Soyani Pond |  |
| Photograph 7.1.1 | Consultation Meeting at Subproject Site with Pourashava Officials and Consultants Participants |  |

# ABBREVIATIONS

| AP persons (AP’s) | Affected Person |
| --- | --- |
| BDT | Bangladeshi Taka |
| BOQ | Bill of Quantity |
| CC | Cement Concrete |
| CIP | Capital Investment Plan |
| CP | Contingency Planning |
| DSM | Design, Supervision, and Management |
| EA | Environmental Assessment |
| ECR | Environmental Conservation Rules |
| EMP | Environmental Management Plan |
| EPP | Emergency Preparedness Planning |
| ES | Environmental Screening |
| FGD | Focal Group Discussion |
| GoB | Government of Bangladesh |
| GRC | Grievance Redress Committee |
| GRM | Grievance Redress Mechanism |
| GRP | Grievance Redress Procedure |
| LGED | Local Government Engineering Department |
| MGSP | Municipal Governance and Services Project |
| PD | Project Director |
| PMU | Project Management Unit |
| RCC | Reinforcement Cement Concrete |
| RP | Relevant Reports |
| TL | Team Leader |
| WB | World Bank |
| XEN | Executive Engineer |

# INTRODUCTION

## Subproject Background

Tangail Pourashava is situated at Tangail Upazila under Tangail District. Tangail declared as a Pourashava in the year of 1887. The Pourashava covers an area of 35.22 sq. km. At present, it is A type Pourashava having total population of 1, 67,412 (Population and Housing Census, 2011). The Pourashava consists of total 254.35 km road network in and around the Pourashava boundary (among them 127.75 km BC, 18.30 km RCC, 2.13 km HBB, 2.63 km brick soling and 103.72 km katcha). In addition, the Pourashava has 25 km brick drain, 9.18 km RCC drain, 22.23 km primary drain and 23.68 km katcha drain. (Source: Pourashava Website). From the current infrastructure situation, it is revealed that further urban facilities for township development are necessary. Therefore, this subproject significantly will contribute infrastructure development in the Pourashava area.

This subproject includes the following three components: BC Road and RCC Drain and Beautification work. The significant features of the subproject are stated below:

| Name of the Subproject : | a) Improvement of Akur Takur Para Choto Kalibari Road from Choto Kalibari Moar to Shimultoli Road (Ch.00-225m, Total length 225m); (b) Construction of RCC Drain at Akur Takur Para Shimultoli Road from Mymensingh Road to Louhajang River via Taltola (Ch.340-755m) with Akur Takur Para Chotokalibari Road; Link 225m, (Total Effective Length 640m); (c) Construction of Retaining Wall and Beautification Work of Soyani Pond (Wall Length-250m) at Thanapara. |
| --- | --- |
| Package No. : | MGSP/TAN/2017-2018/W-13 |
| District Name : | Tangail |
| ULB Name : | Tangail Pourashava |
| Jurisdiction Area : | Ward numbers 13 and 16 |
| Structural Design Option : | BC Road, RCC Drain and Beautification Work |
| Beneficiary Population : | About 10,000 as per information by the Pourashava Officials |
| Tribal People : | No tribal people settlement found in the subproject area |
| Land Acquisition : | Tangail Pourashava owned. So, private land acquisition is not required. |
| Estimated Cost : | 49.59 Million (BDT) |
| Subproject Duration : | 12 Months |
| Tentative Start Date : | 12 September 2018 |
| Tentative Completion Date : | 11 September 2019 |

## Objectives of the Study

The general objective of this study is to determine the major environmental impacts succeeding from execution of the subproject and to recommend mitigation measures to avoid or reduce adverse environmental impacts and to enhance positive impacts. The specific objectives include:

- To assess the existing environmental conditions of the subproject site and its influence area;

- To identify and assess impacts resulting from the subproject during its construction phase and operation phase;

- To develop a environmental management plan with recommendations for mitigating impacts and enhance positive impacts;

- To summarize environmental monitoring requirements.

## Scope and Methodology of the Study

The study methodology comprised the following activities:

- Desktop Study;

- Field Investigations and Data Collection;

- Data Analysis and Report Writing.

Desktop Study

The desktop study involved:

- Initial meetings with client, stakeholders to discuss the proposed subproject, including subproject activities;

- Collection and review of the baseline data, maps, reports and other relevant information;

- Review of the relevant existing legislation, regulation and policies;

- Understand the anticipated technical processes that may affect the environment.

Field Investigation and Data Collection

The following key tasks were performed during the field visit:

- Taking photographs of the significant aspects to assist in describing the baseline environmental conditions of the subproject area;

- Interviews with representatives of the relevant Pourashava officials, beneficiary and affected people within the subproject influence zone;

- Obtaining relevant documents from the Pourashava and local people within the subproject influence zone;

- Verifying information and data collected during the desktop study and to collect new information.

Data Analysis and Report Writing

The data and information collected from all the sources (literature review, secondary and primary data, public consultation) were analyzed to describe the existing environmental setting of the subproject area, to identify the potential positive and negative impacts of the proposed subproject, as well as to provide preliminary suggestions for mitigation measures. Finally, this environmental assessment report has been prepared.

# SUBPROJECT DESCRIPTION

## The Study Area

The subproject is located in the Tangail Pourashava within the Jurisdiction area of the Ward number 13 & 16. There is no protected area in or near the subproject location. The Base Map, Layout Plan and Topographical Features of this subproject are shown in Figure 2.1.1, Figure 2.1.2, Figure 2.1.3 and Figure 2.1.4.

Figure 2.1.1: Base Map of the Subproject Sites

Figure 2.1.2: Layout Plan of the Akur Takur Para Choto Kalibari Road and Soyani Pond Area

Figure 2.1.3: Topographical View of the Akur Takur Para Choto Kalibari Road

Figure 2.1.4: Topographical View of the Soyani Pond Area (1 km radius)

## Current Situation, Proposed Intervention and Need for the Subproject

The subproject area is mainly residential area. Beside this, there are educational institutes, varieties shops and restaurant in the subproject area.

The proposed Akur Takur Para Choto Kalibari Road (Length-225m) is partially damaged. The average width of the existing Choto Kalibari Road varies from 4m to 6m. Moreover, most of the wearing surface of the road is worn-out. There are pot holes at few segment of the road. Hence, vehicles movement on the worn-out surfaces generates dust. The potholes retain storm water after rainfall. As a result, smooth traffic flow is hampered and increases the frequency of the traffic accidents. Hence, the road will be improved by replacing the existing damaged BC (bituminous carpeting) road to new BC pavement. Therefore, BC road of 6 m width and effective length of 225 m has been proposed.

In the subproject area, the drainage facility is inadequate and inappropriate. There are brick drains at last few portion of the Akur Takur Para Road. While most section of the link road, there is brick drain. The existing drains are discontinuous. Additionally, these drains are narrow, damaged and blocked by various waste materials (polythene, plastic bottles, households waste etc.). Due to non-functionality, the existing drain cannot discharge the storm water properly. Subsequently, water logging problem creates environmental hazard in the subproject area. Hence, RCC drain with cover slab from Ch.340-755m of Akur Takur Para Shimultoli Road and 225m RCC drain with cover slab at Choto Kalibari Road has been proposed depending on the site condition and demands. The drain will be connected with the Louhajang River. The current situation of the proposed road and drain is further shown in the following Photograph 1.

## Photograph 2.2.1: Currents Situation of the Proposed Road and Drain

At Soyani Pond, there are brick guide wall all sides of the pond. However, few portion of the south side guide wall is totally damaged. Remaining part of the guide wall is partially damaged. From the site visit, it is also revealed that throwing of the waste materials degrades the pond water quality. Under this subproject, the Pourashava propose protection work by providing stainless steel railing, sitting arrangement with ceramic bricks and benching, providing mini-park facilities by the installation of the rocking cradle (Dolna) and tree plantation. The idea is to provide recreational facility and enhancing landscape condition in the Pourashava area. The current situation of the Soyani Pond is further shown in the following Photograph 2.

Photograph 2.2.2: Current situation of the Soyani Pond

## Justification of Selection of this Subproject

With the facilitation of PMU, WB and DSM Consultants, the Pourashava prepared the CIP list. According to the CIP list, Tangail Pourashava prepared the priority list of the subprojects considering the demand and requirement. As a part of the reconnaissance survey, PMU Officials and DSM consultants’ visited and evaluated the existing site condition of the subproject. To improve the substandard road network and minimize drainage congestion and water logging problem, this subproject has been selected on priority basis.

From the environmental screening it is revealed that the ecological impacts due to this subproject is not significant, though 34 numbers of trees to be felled down. The subproject has adverse impacts on the physicochemical environment. However, it is anticipated that the adverse impact is not significant and limited in the subproject boundary.

It should be noted that 65 numbers of tress will be planted under this subproject to compensate the ecological impact caused by the subproject and to enhance the ecological condition. In addition, adequate and appropriate mitigation measures will be formulated in the EMP to minimize the adverse impacts. In fact, this subproject will be designed and implemented considering the standard measures to safeguard environment. Consequently, subproject has significant benefits to the community people. Considering the benefits that will derive, the subproject has been selected for implementation.

## Envisaged Subproject Activities and Implementation Process

The general activities of the subproject includes: construction of the semi-pucca site office, construction of the labor shed and relocation of the electrical poles.

Specific activities for RCC Drain

- Earth work in excavation of the foundation;

- Pumping and bailing out of water as per requirement;

- Sand filling for the preparing foundation bed;

- Laying polythene sheet;

- Plain cement concrete work in foundation;

- Manufacturing CC blocks;

- Fabrication of the ribbed or deformed bar;

- Reinforced cement concrete work.

Specific activities for BC Road:

- Site clearing work;

- Preparation of the hard bed by scarifying and loosening of the existing top;

- Earth work in box cutting;

- Sand filling on the road bed;

- Brick on end edging;

- Compacted aggregates and sub-base course;

- Providing prime coat;

- Lying pre-mixed dense bituminous surfacing

Key activities of beautification of pond:

- Earth work in excavation;

- Backfilling work by Sand;

- Construction of RCC guide wall;

- Sitting arrangement by ceramic bricks and mortar;

- Railing work by stainless steel;

- Installation of the stainless steel rocking cradle;

- Tree plantation as ecological enhancement work;

- Providing adequate lighting facilities.

## Category of the Subproject

For BC Road and RCC Drain and Beautification work

- According to ECR 1997: Green □    Orange□  A  Orange B □√ Red □ Not Listed □

- According to WB classification: Category B □√		 Category C □

In the Project EMF, RCC drain is categorized as Orange A or Orange B depends on environmental impact. In the Project EMF, road is categorized as Orange B. considering the anticipated environmental impacts, primarily BC Road and RCC drain, can consider as Orange-B category as per ECR-97. According to the WB classification, it can classify as Category B.

## Subproject Schedule

The tentative schedule of construction of the subproject is:

(a)  Subproject Duration (months)  	:12 months

(b)  Tentative Start Date    		: 12 September 2018

(c) Tentative Completion Date  		: 11 September 2019

The daily construction hours will normally include regular working time. However, daily working hours may vary based on the on-site condition.  The detailed work program will be prepared by the contractor with the assistance of the PIU. Then it will be shared with the PMU, LGED and DSM consultants. In addition, the detailed work program will also be shared with the Bank as per requirement.

# Baseline ANALYSIS OF THE ENVIRONMENTAL CONDITION

## Physical Environment

### Important Environmental and Infrastructural Features

The detailed environmental and infrastructural features have been collected during the site visit. The conditional survey has also been performed for designing of the subproject.  Hence, data from conditional survey is also incorporated in the screening report. Effort has been given for listing the major environmental and infrastructural features within 100 m of the both sides from the center line of the road at 100m longitudinal intervals. The key environmental and infrastructural features are given in Table 3.1.1 and Table 3.1.2.

Table 3.1.1: Major Environmental and Infrastructural Features from Tangail Mymensingh Highway to Louhojong River (Ch.340-755m).

| Chainage (m) | Left | Right | Major Environmental and Infrastructural Features |
| --- | --- | --- | --- |
| Akur Takur Para Shimultoli Road from Tangail Mymensingh Highway to Louhojong River (Ch.340-755m) |  |  |  |
| 340-440 | √ |  | Photo Copy Shops, Electric Pole, Pucca, Semi pucca and Tin shed Residents, |
|  |  | √ | Rokomari Store, Brick Drain, Electric Pole, Pucca, Semi pucca and Tin shed Residents, |
| 440-540 | √ |  | Lekmi Beauty Parlor, Boundary Wall, Pucca, Semi pucca and Tin shed Residents, |
|  |  | √ | Vozon Roshik Resturanent, Manobadhikar Shongshta BD, Dia Beauty Parlor, Anupom Shomaj Kallan Shongsta. |
| 540-640 | √ |  | Ruma Traders, Taltola Dental Care, Electric Pole, Semi pucca and Tin shed Residents, |
|  |  | √ | Pucca, Semi pucca and Tin shed Residents, Boundary Wall, |
| 640-755 | √ |  | Pucca, Semi pucca and Tin shed Residents, Boundary Wall, Electric Pole |
|  |  | √ | Pucca, Semi pucca and Tin shed Residents, Boundary Wall, |
| Chotokalibari Moar to House of Edris Ali (Ch.0-225m) |  |  |  |
| 0-100 | √ |  | Faruk Departmental, Doha Telecom, Boundary Wall, Trees, Pucca, Semi-Pucca and Tin Shed Residents |
|  |  | √ | Boundary Wall, Brick Drain, Pucca, Semi-Pucca and Tin Shed Residents, Electric Pole |
| 100-200 | √ |  | Pucca, Semi-Pucca and Tin Shed Residents, Trees, Boundary Wall |
|  |  | √ | Brick Drain, Pucca, Semi-Pucca and Tin Shed Residents, Trees, Boundary Wall |
| 200-225 | √ |  | Pucca, Semi-Pucca and Tin Shed Residents, Trees, Boundary Wall |
|  |  | √ | Brick Drain, Pucca, Semi-Pucca and Tin Shed Residents, Trees, Boundary Wall |

Major environmental and infrastructural features in the subproject area of the Soyani Pond are shown in following Table 3.1.2.

Table 3.1.2.: Major Environmental and Infrastructural Features around the Soyani Pond

| Side / Direction | Major Environmental and Infrastructural Features (around the Pond sides) |
| --- | --- |
| Soyani Pond At Thanapara |  |
| North | BC Road, Tangail Girls High School, Boundary Wall, Trees, Pucca, Semi Pucca and Tin Shed Residents |
| South | Matador Furniture’s, Boundary Wall, Pucca, Semi Pucca and Tin Shed Residents, |
| East | BC Road, Rafiqu Razu Cadet Coaching School, Pucca Building, Trees |
| West | BC Road, Boundary Wall, Pucca, Semi Pucca and Tin Shed Residents, Trees |

### Geology, Topography and Soils

Soils are mixture of sandy and silty alluvium which occupies most char land. Overall silty deposits are more extensive than sandy deposits. Raw sand and silt deposits usually are bare or carry sparse grasses, herbs and woody plants. Organic matter content is low (< to 1.7%), neutral to moderately alkaline in reaction. Soils are clay (87%) loamy (13%) and sandy (<1%). Organic matter content is low (<1-5%). pH ranging from 4.5-7.8. The general soil classification of Bangladesh is shown in Figure 3.1.1.

Figure 3.1.1: General Soil classification of Bangladesh (Bangladesh Soil Resource Development Institute)

The area lies in the earthquake Zone 2 (intermediate level of seismic intensity) (shown in Figure 3.1.2) according to the seismic map of Bangladesh (GSB, 1978).

Figure 3.1.2: Earthquake Zoning Map of Bangladesh (GSB, 1978)

### Climate and Meteorology

Tangail has a tropical climate. In winter, there is much less rainfall than in summer. The average temperature is 25.5 °C. Mean annual rainfall is about 1500mm. June has a mean rainfall exceeding 300mm. According to the statistics of wind data from the Bangladesh Meteorological Department Climate Division, wind direction changes by month. Nevertheless, the northwest, south, and northeast winds are predominant. Table 3.1.2 shows the average annual key meteorological data.

Table 3.1.2:  Average Annual Temperature and Rainfall Data

|  | Jan | Feb | Mar | April | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Avg. Temperature (°C) | 18.3 | 20.8 | 25.3 | 28.6 | 28.8 | 28.4 | 28.5 | 28.5 | 28.6 | 27.3 | 23.3 | 19.9 |
| Min. Temperature (°C) | 11.7 | 13.6 | 18.2 | 22.3 | 24.2 | 25.3 | 25.9 | 26.1 | 25.8 | 23.6 | 17.9 | 13.6 |
| Max. Temperature (°C) | 25 | 28.1 | 32.5 | 35 | 33.5 | 31.6 | 31.1 | 31 | 31.5 | 31 | 28.7 | 26.2 |
| Avg. Temperature (°F) | 64.9 | 69.4 | 77.5 | 83.5 | 83.8 | 83.1 | 83.3 | 83.3 | 83.5 | 81.1 | 73.9 | 67.8 |
| Min. Temperature (°F) | 53.1 | 56.5 | 64.8 | 72.1 | 75.6 | 77.5 | 78.6 | 79.0 | 78.4 | 74.5 | 64.2 | 56.5 |
| Max. Temperature (°F) | 77.0 | 82.6 | 90.5 | 95.0 | 92.3 | 88.9 | 88.0 | 87.8 | 88.7 | 87.8 | 83.7 | 79.2 |
| Precipitation / Rainfall (mm) | 11 | 20 | 41 | 91 | 235 | 349 | 351 | 316 | 262 | 167 | 25 | 4 |

The construction works can be influenced by the climatic condition and meteorological components like humidity, temperature, sudden rainfall, and wind speed. During high winds, there might be possibility of quick spreading of dust generated from the construction activities. It is very risky to work during rain and in high winds because the possibility of getting injury increases. Furthermore, work under high temperature and excess humidity is extremely difficult, and may create health hazards.

### Water Resources

### In the subproject area, major water bodies include, Louhajong River, ponds, and ditches. Ground water is usually available by tube well. Groundwater is the main source of potable water in the subproject area. The groundwater (mainly shallow aquifer) is contaminated by arsenic (As) in some areas. Local people typically use deep tube-well water for drinking and other domestic purposes ( Master Plan, 2020-2031).

### Flooding, Water Logging and Drainage Pattern

This area is affected in historical severe flood events such as 1988, 1998 and 2004. This area is situated on the Active Brahmaputra–Jamuna Floodplain zone and flooding from river water overflow is a common phenomenon. Due to continuous heavy rain and delay discharge of storm water, this area is subjected to water logging problem. Inundation within the Pourashava area is experienced in the months of June to September when major rainfall occurs. According to Bangladesh Meteorological Department (BMD) flood zoning map, this area lies in the normal flood area.

Air Quality and Dust

Ambient air quality data have not been found. Air seems to be clean. However, due to poor maintenance of the road especially during dry season dust generates from the moving vehicles and windblown dust cause air pollution. In addition, black smoke emission from the non-combustion engine vehicles degrades the quality of the air. There are no remarkable sources of the air pollution such as heavy and small industries. There is no brick field observed during site visit.

Noise Level

Noise is not a major impediment for the quality of the environment in the subproject area. From the field visit it is found that, noise generated due to the vehicle movement and Bazar area. Vehicles such as trolleys, pick-up, motor cycles, CNG, rickshaws are major sources of noise. In the subproject activities, using of steel cutter, welding machine, concrete mixer machine and vibrator machine etc. may generate noise. However, it is anticipated that generated noise from the subproject activities will be within the tolerable limits.

Sanitation

The sanitation system in the Pourashava is not satisfactory. According to the DPHE, most of the toilets and septic tank has illegal connection to the nearby drains, ditches or ponds. The Pourashava has taken initiative to improve sanitation system through underground sewerage and strict regulation on katcha latrine. Awareness of the community people is also necessary for improving sanitation system. Improved low-cost sanitary system can be encouraged in low-income areas.

Solid Waste Management

Solid waste management is a crucial problem for the Pourashava. The Tangail Pourashava does not have sufficient capability to introduce effective waste management system. Narrowness of roads in few areas and lack of local collection sites, create problem for the effective solid waste management. Particularly, in informal areas due to narrow roads the garbage trucks cannot enter for removal and transhipment of the garbage. In most places, there is no road side open space for locating garbage bins. The local people also dump solid wastes in the vacant lowland, open space near to the houses and road side area (Master Plan 2020-2031).

## Biotic Environment

Flora and Fauna

Since the subproject area is situated in the developed area; hence, road side terrestrial floral species is less. But in the homestead area of the subproject influence area, there are lots of terrestrials’ floral species. The fruits and wooden based species of floral are dominated in the homestead area. In the homestead area, planted trees includes- Coconut (Cocos nucifera), Banana (Musa paradisiacal), Supari (Areca catechu), Eucalyptus (Eucalyptus teritocornis) Akashi (Acacia auriculiformis), Mango (Mangifera indica), Jackfruits (Artocarpus heterophyllus), Bokul (Mimusops eleng), Jarul (Lagerstroemia speciosa), Mahagony (Swietenia macrophylla), Lebu (Swietenia macrophylla), Shisho (Dalbergia sissoo), Goava (Psidium guajava), Tal (Borassus flabelliformis), Khejur (Phoenix paludosa), Ata (Anona Retcualta), Jolpai (Olea europea), and krisnochura (Caesalpinae Delonix) etc.

There are common local birds such as Shalik (Acridotheres fuscus), Choroi (Passer domesticus), Bok (Mesophoyx intermedia), Dowel (Copsychus saularis), Crow (Corvus splendens), Bulbuli (Pycnonotus barbatus) and Tuntuni (Orthotomus sutorius) etc. were found during the site visit. There is no historical important site in the subproject area. However, the educational institute and religious institutes of the subproject area bring cultural values to the local community.

## Socio-economic and Socio-cultural Environment

### Land Use,  Status of Housing and Built-up Infrastructure

The subproject area consists of mixed land use pattern-medium to high densely populated residential area, commercial area with large number of educational institutes The built-up infrastructure includes mainly pucca, semi-pucca and tin-shed residential houses and shops, educational institutions, religious centers, hospital and health care center, business centers (hotel and restaurant).

### Beneficiary Population

The subproject covers the jurisdiction of the ward number 13 and 16. As per information by the municipality, about 10,000 people will benefit directly and many others indirectly.

### Education

People of Tangail Pourashava mostly attained primary and secondary level education. 75.7 % of male and 67.8% female population are educated. Literacy rate is 71.8% at Tangail Pourashava (Population and Housing Census, 2011).

### Tribal Communities

There is no indigenous or tribal people settlement in the subproject area. Therefore, there are no measures needed for indigenous peoples’ safeguard.

### Land Acquisition and Resettlement

There is no private land acquisition needed. Soyani Pond is the govt. property. Hence, Tangail Pourashava needs NOC from district commissioner office. In addition, construction of drain at Akur Takur Para Road will cross RHD Road. So, Tangail Pourashava needs NOC from RHD. Furthermore, few road side boundary walls at different segments of Choto Kalibari Road need to be removed for site clearing work. The Pourashava authority will clear the site amicably prior to the commencement of the subproject. However, in case of any objection from the affected persons, DSM will investigate the actual situation. Then, measures will be taken as per rules.

Principal Livelihoods and Economic Activities

The subproject area is inhabited by the mixed occupational people. In the Pourashava area, major income comes from business, enterprises, small trades, private sector jobs, and government jobs in the town.

Cultural Heritage and Protected Areas

There is no protected and important cultural or historical site in the subproject influence area. However, there are religious centers and educational institutes which convey the cultural value for the community people.

Social Conflicts and Development Activities

The local people welcome the subproject and there is no conflict between the local communities regarding the subproject implementation.

# ENVIRONMENTAL SCREENING

Environmental Screening Checklist, as adopted in Appendix C of the Environmental Management Framework (EMF) of the MGSP, was administered for identifying the impacts and their extents.

- The screening data for the RCC Drain, BC Road and Allied works (Retaining Wall, Beautification work) have been formulated and are shown in below:

(2)  Potential environmental impact during construction phase:

- Ecological impacts:

34

- Felling of trees  			Significant □    Moderate □     Minor □ √   Number of trees

- Clearing of vegetation  		Significant □    Moderate □     Minor □√

- Potential impact on species of 	Significant □    Moderate □     Minor □ √

aquatic (i.e., water) environment

There are 34 nos. of trees (Mango, Jackfruits, Coconut, Betel Nut, and Mahogany etc) to be felled down due to implementation of this subproject especially at Choto Kalibari Road for widening of the road. Most of the trees to be felled down at Choto Kalibari Road are betel nut. Even the trees to be felled down are small to medium size considering the diameter and length. Furthermore, there is no tree to be felled down has religious and cultural importance. The vegetation to be cleared due to the subproject implementation is also minimal. Anticipated impact on the species of aquatic environment is also minor because except throwing of the waste materials and accidental leakage and spillage of the oil and grease from subproject vehicles and equipment in to the existing water bodies, the subproject activities do not have any direct impact on the aquatic environment. Considering number of trees to be felled down, ecological impact is considered as minor.

- Physicochemical impacts:

- Air Pollution        			Significant □     Moderate □√	Insignificant □

- Noise Pollution        			Significant □     Moderate □	Insignificant □√

- Drainage Congestion         		Very likely □     Likely □ √ 	Unlikely □

- Water Pollution         			Significant □     Moderate □√	Insignificant □

- Pollution from solid/ construction wastes Significant □     Moderate □√	Insignificant □

- Water logging          			Significant □     Moderate □	Insignificant □ √

The subproject will have impact on noise and air quality during construction phase due to mobilization of the equipment, vehicles movement for the transportation of the materials, hydraulic excavator, brick breaking machine, mechanical compaction machine, paver, asphalt plant, concrete mixer machine, vibrator machine etc. The length of the road and drain is less. Beautification of Soyani Pond also includes simple construction activities. Hence, construction work will require short time. In addition, the construction work will be performed section wise. Hence, anticipated impact is less, temporary and site specific. So, overall impact on air and noise will be insignificant. The generated dust during construction activities may disturb the local people if measures are not taken. The generated construction wastes, un-suitable materials and solid wastes may degrade the adjacent water bodies if not properly collected, disposed and thrown. Improper collection and disposal of the generated wastes materials also may degrade the quality of the surrounding environment and degrade the aesthetic value. Special care needs to be taken for the beautification work of Soyani Pond to avoid any degradation of the pond water due to spillage and unexpected throwing of the waste materials in to the Pond. The anticipated water logging problem during construction period is insignificant because most of the activities will be performed in the dry season.  However, erratic rainfall may cause drainage congestion and water logging problem in the construction site. Therefore, pumping facilities will be provided to drain-out the storm water if required. The subproject will have no adverse impact on the topography, climate, soils and regional hydrology.

- Socio-economic impacts:

- Traffic congestion 			Very likely □  	Likely       □ √   	Unlikely     □

- Health and safety 			Significant □  	Moderate □√	Insignificant  □

- Impact on archaeological and historical   Significant □  	Moderate □  	Insignificant □√

- Employment generation 		Significant □ 	Moderate □ √  	Insignificant  □

During construction, the subproject will have likely localized negative impact in traffic congestion. However, length of the road construction is less and the use of the road for motorized vehicles is very few. On the other hand, since drain construction work does not require closing of the entire road. Hence, pedestrian and non-motorized vehicles can use the road. The activities for the beautification work at Pond do not have any likely impact on the local traffic. The construction work will follow simple procedure with commonly used equipment. Therefore, impact on health and safety is anticipated as moderate. There is no archaeological and historical site within the influence area. The subproject will have positive impacts by generating employment opportunities for the local people and creating business opportunity of supplying construction materials, equipment, food and other necessary staffs to the work camp.

(3)  Potential Environmental Impact during Operational Phase:

- Ecological impacts:

- Potential impact on species of aquatic 	Significant □ Moderate □ Minor  □√

(i.e., water) environment

The drain is designed for discharging the storm water. Hence, anticipated impact on the aquatic environment due to the operation of the drain is likely minor. However, if the drain carries pollutants from any sources, it may degrade the quality of the aquatic environment. During operation phase, throwing of the solid wastes (cans, plastic bottle, polythene, chocolate cover, chips cover etc.) in to the Soyani Pond by the visitors may degrade the quality of the aquatic environment.

- Physicochemical impacts:

- Potential air quality      	Improvement □      	No-improvement □          Deterioration □√

- Potential noise level      	Improvement □      	No-improvement □          Deterioration □√

- Drainage congestion      Improvement □√    	Minor Improvement □ 	 No Impact □

- Risk of water pollution     Significant □  		Moderate □  		 Minor □√

- Pollution from solid waste Improvement □ 	No-improvement □          Minor □√

After completion, due to improve road traffic volume may increase. Consequently, air quality and noise level may be degraded due to black smoke emission and hydraulic horn from the vehicles. The drain component does not have any impact on the noise. The new drain will minimize drainage congestion and water logging problem. The new drain also minimizes throwing of the waste materials and spreading of the bad odor from the drain. The storm water to be discharged through the drain may degrade the water quality of the outfall and adjacent water bodies if carry pollutants from any sources. During operation phase, throwing of the solid wastes (cans, plastic bottle, polythene, chocolate cover, chips cover etc.) in to the Pond by the visitors may pollute the water quality of the Soyani Pond.

- Socio-economic impacts:

- Traffic                                  Improvement  □√   	 No-improvement    Adverse □

- Safety 			     Improvement  □√   	 No-improvement    Adverse □

- Employment generation      Significant      □   	 Moderate  □    	      Minor     □√

After completion, the subproject road will enhance transport facilities in the subproject area. The new drain will reduce drainage congestion and water logging problem in the subproject area. The new drain will also reduce stagnation of the water on the road. Thus, it will enhance road safety. Pond side land sliding possibility due to soil erosion will be minimized after construction of the retaining wall and sitting arrangement. Additionally, beautification work of the pond will enhance landscape and aesthetic value and will provide recreational facilities to the local people.

(4)  Summary of the Possible Environmental Impacts of the Subproject:

The ecological impact due to the subproject implementation is mainly due to tree felling. However, the ecological imbalance to be caused due to felled down of the trees will be compensated by the plantation of the trees.

The overall impact due to implementation of the subproject is not severe. The physicochemical components will be disturbed during the construction phase. However, the anticipated impacts will be mainly within the subproject boundary. The wastes generated due to the construction activities should be properly collected and disposed in a designated site. There is also some adverse impact at operational stage. Due to improved road, motorized traffic volume may increase. Thus, air quality and noise level may be deteriorated due to vehicles emission and horns by the vehicles. The storm water to be discharged through the drain may degrade the water quality of the outfall and adjacent water bodies if carry pollutants from any sources. Retaining wall and sitting arrangement at the Soyani Pond will reduce soil erosion. Beautification work at Soyani Pond will enhance landscape and aesthetic value which will provide recreational facilities in the subproject area.

Safety concern is an important issue for both the construction and operation phases that should be considered properly. The socio-economic aspect mainly includes health and safety and employment generation. This subproject will have positive impacts in terms of the generation of the employment and business activities by supplying construction materials and equipment.

# Specific impact, mitigation AND enhancement measures

From the environmental study, the likely impacts of the subproject are mainly caused by the key activities required for the implementation of the subproject, raw materials and equipment to be used for the execution of the activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

## Dismantle Work, Site Clearing, Excavation Work and Earth Work

The road improvement work consists of dismantling of the existing BC road, earth excavation, removal of the un-used materials etc. The drain construction work consists of dismantling of existing damaged brick drain, earth excavation, earth filling, back filling and removal of the unsuitable materials. These works lead dust blowing, construction waste generation, noise and vibration which disturb the local people.

Mitigation Measures

- Proper care will be taken by the contractor during dismantle work, excavation, earth work and back filling so that this activity does not disturb the nearby establishment.

- Avoid loss of the topsoil for the earth filling work;

- Cover the exposed dry loose soil  with fabric to avoid any dust generation;

- Disposal of soil and construction wastes at designated dump site near Rabna bypass.

## Tree Felling, Clearing of the Vegetation and Ecological Impact

There are 34 numbers of planted roadside trees will be felled down due to implementation of this subproject.

Mitigation Measures

- 65 nos. of the trees will be planted at periphery of the Soyani Pond to compensate the felled down trees and to enhance the ecological condition.

- Planting trees will enhance the ecological balance of the area after their successful growth.

## Pollution from the Construction Materials

Dumping of the construction spoils, including accidental leakage of the oil, grease, and fuel in equipment yards is a significant hazard. Storm water may carry these substances during rain. Thus, both surface and groundwater might be polluted from these contaminants. Air pollution and dust may affect the nearby settlement which is generated from fine aggregate such as sand and loose dry soil.

Mitigation Measures

- Safe transport, storage, and disposal of the construction materials, and the equipment have to be carried out in order to avoid the accidental spillage and loss;

- Fuels, lubricants, and other hazardous materials should store over raised platforms and  not directly on the ground;

- Maintain adequate moisture content of soil and sand during transportation, compaction and handling;

- Carry the materials especially loose soil and sand with adequate cover.

## Air Quality and Dust

During the construction phase, air pollutants will be emitted from the equipment and construction vehicles are expected to remain low. Local residents in the vicinity of the work sites will be temporarily disturbed by the limited dust pollution. The overall impacts, however, are expected to remain low.

Mitigation Measures

- Water should be sprayed to control the dust, which is the main way to suppress dust at the worksite.

- Ensure sprinkle and cover stockpiles of loose materials (e.g., fine aggregates);

- Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and  handling;

- Avoid use of dust generating equipment which produce significant amount of particulate matter far from the local residents;

- Ensure that all subproject vehicles are in good operating condition.

## Noise and Vibration

Noise and vibration caused by the equipment and movement of the construction vehicles may temporarily disturb nearby residents. The sensitive areas like roadside residents, office, shops, educational institute could be affected from the noise, though the impacts are anticipated to be limited.

Mitigation Measures

- Transportation of the construction materials have to be carried during the scheduled times, and mainly during the day;

- If needed, all powered mechanical equipment will be fitted with noise abating gear such as mufflers for effective sound reduction;

- Avoid using of the steel cutter at night;

- Avoid using of the concrete mixer and vibrator machine at night and schooling time.

## Water Quality

There are road side ditches, ponds and Dewla Khal. The water quality may deteriorate if the construction materials such as loose soil, sand, construction waste, effluent from work camps, and food waste are dumped in the roadside water bodies (ditches, ponds and Dewla Khal).

Mitigation Measures

- Proper construction management including waste management as well as training of the operators and other workers should provide to avoid pollution of the water bodies;

- Construction waste will dispose properly (not in water bodies or lowland), for which contractor will be responsible.

## Occupational Health and Safety

The most important risks associated with the construction activities are listed below:

- Exposure to the sunlight- workers are being exposed to the sun for long hours;

- Exposure to the high temperature, and humidity for a long time resulting in dehydration;

- Contact with the hazardous substances and wastes pose risks of the infections and diseases;

- Risk of the poor air quality due to the dust;

- Risk of the collision (traffic) during mobilization of the equipment and transportation of the equipment;

- Risks from head loads for carrying soil, construction materials and construction equipment;

- Risks of the using of the machineries in motion;

- Risk associated to the sudden bad weather working conditions.

General Requirements for the Workers’ Health and Safety

The key salient features of the general requirements for the workers’ health and safety stated are presented in Table 5.7.1.

Table 5.7.1:  General Requirements for the Workers Health and Safety

| Issues | Requirements |
| --- | --- |
| Health and Hygiene | - Cleanliness at the site premises and workers living places and at the Labor Shed; - Arrangement of the proper ventilation and temperature at the Labor Shed; - Protection against dust and furnace by using of the nose masks and covering of the head and body; - Proper disposal of the wastes and effluents; - Introduce waste bins for the solid waste management system. |
| Safety and First Aid Box | - Using of the personal protective equipment (helmet, gloves, goggles, nose mask, safety boots); - Precautions during work on or near machineries in motion; - Head loads are prohibited; - First aid facilities should be provided and maintained; - The first aid kit should include adhesive bandages, regular strength pain medication, gauze, and low grade disinfectant. |
| Compensation for Accidents at Work | - Contractors will bear medical treatment costs. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation-(the amount of the compensation should be fixed considering the type of accidents). |
| Dust and Fumes | - For any dust, fumes, or other impurities likely to be injurious to the workers, effective measures shall be taken to prevent their accumulation and its inhalation by the workers. |
| Overcrowding | - No labor room should be overcrowded. |
| Latrines and Urinals | - Sufficient latrines shall be provided; - Latrines shall be maintained in clean and sanitary condition; - Latrines shall be adequately lighted and ventilated. |
| Disposal of Wastes and Effluents | - Proper disposal system for the solid waste and effluent is required; - Waste bins must be provided by the contractor at labor shed. |

## Impacts on Social Environment and Common Property Resources

Through comprehensive study, it is revealed that impacts are expected not to be severe and to be largely manageable. The following Table 5.8.1 presents impacts on socio-economic environment and common property resources.

Table 5.8.1: Impacts on Social Environment and Common Property Resources

| Social Components | Impacts on IECs | Impact Significance |
| --- | --- | --- |
| Community Perception | The local community people welcome this subproject and there is no visible objection from them. | Significant (+ve) |
| Employment and Business Opportunity | Community feels happy because generally the local contractor will be engaged for the construction works which will create work opportunity for the skilled and non-skilled labor. The subproject will create business opportunity for the equipment and materials suppliers’. | Significant (+ve) |
| Community Order and Security | This subproject activity does not create any severe security problems to the local community and community people. | Minor (-ve) |
| Possible damage to existing infrastructure and facilities | Degradation of the existing road infrastructure by the construction equipment/vehicles used in this subproject. | Minor (-ve) |
| New infrastructure and facilities | Improvement of the existing road and construction of new drain will increase municipality infrastructure facilities. Beautification of pond will provide recreational facilities to the community people. | Significant (+ve) |
| Labor Habitat | The labors will stay at the Labor shed which will have impacts on the environment relates to the generation of the solid wastes, effluent, and water consumption. | Moderate (-ve) |
| Health Care | Workers may suffer from the dehydration problems, respiratory problem, and other health hazards. | Minor (-ve) |
| Accident | In case of road accidents by the vehicles to be used for the transportation may have serious negative impact. | Significant (-ve) |

Mitigation Measures

- Conduct dissemination with the local community about the subproject details;

- Continue liaison with the community leaders in order to maintain the community support;

- Engage local contractor and local people as much as possible for positive perception of the local community;

- Follow traffic rules to avoid any accidents;

- Transportation and mobilization of the equipments and construction materials avoiding peak hours and scheduled time;

- Ensure first aid facilities and effective use of personal protective equipments where applicable.

## Labor Influx and Anticipated Impacts

The labor force and associated goods and services required for the construction of infrastructure civil works under this subproject cannot be fully supplied locally. The migration to and temporary settlement of laborers in the subproject, referred to as labor influx, carries an array of potentially positive and negative impacts in terms of demands on public infrastructure, utilities, housing and sustainable resource management and the strain on social dynamics.

Labor influx effects on host communities include positive impacts such as:

- The subproject activities will generate work opportunities for the local people and supplying of the construction materials, equipment, food and other necessary stuffs to the campsite;

- Improved infrastructure and public service access and availability whereby subproject investment catalyzes larger allocation of resources to a region, stimulating the development or expansion of infrastructure and public services.

Critical negative social risks include:

- Increase in criminal activity and alcohol and drug abuse, domestic violence, political attachment and violence, smuggling and robbery etc;

- Increase in gender-based violence, including eve teasing, sexual harassment etc;

- Increases in communicable diseases, including respiratory problems, diarrheal diseases, vector-borne diseases (e.g., malaria), and sexually transmitted infections (e.g., HIV/AIDS, syphilis, gonorrhea, hepatitis B);

- Conflicts arising from increased demand on existing infrastructure, services, and utilities, including transportation, health, education, water and sanitation, waste management, public utilities and community, religious, and recreational facilities and loss of land for access routes.

The general environmental impacts of labor influx include pressure on the natural resources such as using of the water, electricity, other fuel for cooking, loss of land for the labor establishment, depletion of the water supply, sewage and waste water generation, degradation of the air quality, waste generation,  increased demands on the local energy and resources and noise pollution effects. The following safeguard measures are recommended to avoid any risk of labor influx:

- Inform local people about the subproject activities;

- Liaison with the community leaders in order to get community support;

- Engage local people as much as possible to minimize workers from outsiders;

- Monitor workers attitude and behavioral matter;

- Monitor the workers movement for avoiding any unexpected social activities (robbery, crime, political attachment and conflicts, drugs abuse);

- Inform and use local administration to get support if needed;

- Inform local utilities service providers (such as for new electricity connection REB or any other department);

- Ensure effective use of natural resources such as water, electricity, fuel, wood etc.

## Impacts on Traffic Movement

The motorized traffic volume is very few in the subproject road due to bad condition. The traffic congestion due to drain construction work is comparatively less because it does not require closing of the entire road during construction period. During construction period the pedestrians, cycle rider, biker can use the road. In addition, the Pourashava authority will inform local people for using of the motorized vehicles such as private car, pick-up etc. in the connecting and other adjacent road during construction period. Though, it is limited; however, the subproject activities will have likely impact on the local traffic system by restriction of the free movement of the pedestrians, cycle rider, biker etc.

During construction phase, interruption of the traffic movement and impact on the local traffic system due to the subproject activities will be monitored closely. Then separate traffic management plan will be provided if required. However, the following safeguard measures are recommended to minimize the impacts associated to the traffic movement:

- Inform local people about the subproject activities;

- Inspire local people to use connecting and diversion roads;

- Ensure schedule deliveries of material/ equipment during off-peak hours;

- Place traffic sign/cautionary sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption;

- The place of construction works should be fenced off with fences and should be isolated from general public access and marked with signs to ensure safe movement.

# Environmental Management Plan (EMP)

The purpose of the Environmental Management Plan (EMP) is to ensure that the activities are undertaken in a responsible and non-detrimental manner. The EMP will guide the environmentally sound construction of the subproject and ensure efficient lines of communication between the Project Management Unit (PMU, LGED), Project Implementation Unit (PIU) of Tangail Pourashava, DSM, and the contractors.

## Access to Information

The environmental assessment report should be translated into Bengali and disseminated locally. The copies of the report (both in English and Bengali) will be sent to all the concerned personnel responsible for subproject implementation. It will also be made available to the public. The final assessment report will also be uploaded in the LGED website and the World Bank website after approval.

## Grievance Redress Mechanism

The project-specific Grievance Redress Mechanism (GRM) has been established by the PIU of Tangail Pourashava to receive, evaluate, and facilitate the solution of APs concerns, complaints and grievances concerning the social and environmental performance of the subproject. The GRM is aimed to provide a time-bound and transparent mechanism to voice and resolve social and environmental concerns linked to the subproject.

The grievance mechanism is related to resolve the risks and adverse impacts of the subproject. It addresses APs’ concerns and complaints promptly, using an understandable and transparent process that is also gender responsive, and culturally appropriate. It is readily accessible to all segments of the affected people at no costs and without retribution. The mechanism should not impede access to the country’s judicial or administrative remedies. The affected people will be appropriately informed about the mechanism.

LGED has its own Grievance Redress Procedure (GRP), which it operates to address any dissatisfaction and complaints by the local people regarding its activities. This procedure is being applied to address any complaints or grievances through negotiations with the community leaders and representatives of the APs during implementation of the MGSP.

### Grievance Redress Committee (GRC)

The discussions and negotiations will be conducted by the PIU of Tangail Pourashava and will be involved the APs and Grievance Redress Committee (GRC) headed by The Mayor. With the facilitation of DSM, the Mayor nominated the GRC members and included representative from the Government Agencies, local NGO, and Civil Society. The GRC has been formed and established at Tangail Pourashava. The grievance box has already been delivered to the Pourashava to receive complaints. The grievance response focal point will be available at Pourashava for instant response to an aggrieved person. It will receive written complaints or suggestions, and produce them to the GRC for hearing and resolution. The GRC includes the following key members.

Table 6.2.1.1: List of GRC committee members

| SL. No. | GRC Members Name | GRC Designation | Position |
| --- | --- | --- | --- |
| 1. | Mr. Md. Jamilur Rahman Miron | Chairman | Mayor, Tangail Pourashava, Tangail |
| 2. | Mr. Mirza Arif | Member Secretary | Executive Engineer, Tangail Pourashava, Tangail |
| 3. | Begum Nafisa Katter | Member | Assistant Commissioner, Tangail. |
| 4. | Mr. Ananda Mohan De | Member | Principal, Bibekananda School & College, Tangail |
| 5. | Mr. Chairman/Director | Member | Social Advancement thru Unity (SETU), Tangail |
| 6. | Mr. Md. Animus Rahman Anis | Member | Freedom Fighter, Tangail Pourashava, Tangail |

### Grievance Resolution Process

All complaints and suggestions will be received formally in the Pourashava Office by the GRC Member Secretary. A sample Grievance Redress Form has been prepared and sent to the GRC and Tangail Pourashava prior to the implementation of the subproject.

An intake register will be maintained at the office of the Member Secretary of Tangail Pourashava. Member Secretary will record the details of the grievances in the intake register for documentation and ensure impartiality, fairness, and transparency. The intake register will have data and information columns including (i) Case no., (ii) Date of receipt, (iii) name, type of complaint, grievance, (iv) father’s name, husband’s name, (v) sex, (vi) complete address of the person raising the complaint, grievance, (vii) main objection (loss of land, property, or entitlement), (viii) detailed complaint story and (ix) expectation with documentary evidence and previous records of similar grievances, etc.

No GRC members are allowed to contact the aggrieved persons in advance. Rather, the concerned persons are informed to attend the formal hearings at an appointed date. The GRC committee will sit for hearing the complaints of the aggrieved persons. The GRC will record salient points presented by the aggrieved person and will examine documentary evidence submitted during informal hearings. A resolution register will be maintained by the Member Secretary at the Tangail Pourashava Office. The resolution register will contain (i) serial no., (ii) case no., (iii) name of complaint, (iv) complaint story and expectation, (v) date of hearing, (vi) date of field investigation (if any), (vii) results of hearing and field investigation, (viii) decision of GRC, (ix) progress (pending, solved) and (x) agreement or commitments. Closing register will keep records such as, (i) serial no., (ii) case no., (iii) name of complaint, (iv) decision and response to complaints, (v) mode and medium of communication, (vi) date of closing, (vi) confirmation of complainant’s satisfaction and (vii) management actions to avoid recurrence.

The GRC will decide within 30 days of receiving a complaint. There will also be an appeals procedure where, if a person is dissatisfied with the ruling of the GRC, he or she or a representative may attend their next meeting to present the case again. The committee will then reconsider the case in private, after which their decision is final. If the appellant is still not satisfied, then GRC will refer the complaint with the minutes of the hearings to the PD-LGED for further review. If the case at this level is again found unacceptable by the aggrieved person/s, PD, LGED will advise the Pourashava to drop the subproject. Tangail Pourashava should publish the outcome of cases on public notice boards. All costs involved in resolving the complaints (meetings, consultations, communication, and information dissemination) will be borne by the PIU.

Based on consensus, this procedure will help to resolve issues or conflicts amicably and quickly, saving the aggrieved persons from having to resort to expensive, time consuming legal action. The procedure will however, not pre-empt a person’s right to go to the courts of law.

## Institutional Arrangement for Environmental Safeguard Compliance

In the institutional arrangement procedure, Project Director, Team Leader/Deputy Team Leader will directly involve. The PD and TL/DTL would be supported by DSM Environmental Safeguard Specialist and Social Management Specialist. The Pourashava Officials, especially engineers, would be responsible for supporting the construction supervision with the facilitation of DSM consultants. The civil works contractors will implement the environmental mitigation measures.

The PMU, with the help of Environmental Safeguard Specialist and Social Safeguard Specialist will submit the monthly monitoring reports on Environmental and Social Compliances to the World Bank.

PD-MGSP

TL/DTL-DSM

Environmental Safeguard Specialist-DSM

Social Management Specialist-DSM

Senior Municipal Engineer-DSM

Junior Environmental Specialist-DSM

Assistant Municipal Engineer-DSM

Social Management Officer-DSM

Pourashava Representative/ Focal Point/ Engineer

Contractor

Figure 6.3.1: Environmental and Social Management Team (Tentative)

## Capacity Building

A series of training program has already been conducted by the PMU and DSM to build the capability of the PMU, PIUs and DSM field staffs. Under this training program, PMU and DSM will organize an introductory course for training the participants on: (i) Environmental Screening, (ii) EMP Implementation, including environmental monitoring requirements related to mitigation measures; and (iii) taking immediate action to remedy unexpected adverse impacts or ineffective mitigation measures found during the course of the implementation. A separate training program for the contractor has also been conducted to enhance the Environmental awareness. A detailed training manual has already been developed by the Environmental Safeguard Specialist. Hence, the existing manual will be reviewed and modification will be done as per requirement for the next training program.

## Emergency Response and Disaster Management

Disaster management can be defined as the organization and management of
resources and responsibilities for dealing with all humanitarian aspects of emergencies, in particular the preparedness, response and recovery to lessen the impact of disasters. Emergency Preparedness Planning (EPP) and Contingency Planning (CP) are the processes of disaster management plan for developing strategies, arrangements, and procedures to address the humanitarian needs of those adversely affected by the crisis. There are four main types of disasters, namely: Natural Disasters, Environmental Emergencies, Complex Emergencies, and Pandemic Emergencies.

For MGSP activities, Tangail Pourashava would identify the immediate needs, prioritize the tasks, and identify resource requirements to address the humanitarian needs of those adversely affected by the crisis.

## Environmental Management Action Plan

The activity wise anticipated environmental impacts and corresponding mitigation measures have been outlined in Table 6.6.1 and Table 6.6.2.

Table 6.6.1: Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for the workers (Workforce and labor shed management ) | - Generation of sewage and solid waste may cause environmental pollution in the surrounding area | - Construction of sanitary latrine at the labor shed (consider 15 person for one toilet, separate toilet for male and female); - Ensure provision of waste bins (separate waste bins for organic and inorganic wastes ); - Ensure wastes (solid wastes and other forms of the waste) disposal near Rabna bypass dumping site. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU,LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well-being of the local people. |  |
| General construction work | - Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water if needed; - Ensure provision for pumping of congested water if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season. |  |
|  | - Air pollution | - Check regularly and ensure that all the subproject vehicles and equipment are in good operating condition; - Ensure contractor spray water on dry surfaces/ unpaved to reduce dust generation; - Ensure contractor sprinkle water and cover stockpiles of loose materials (e.g., fine aggregates); - Inform local people about the subproject activities and potential impact; |  |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Place traffic sign/cautionary sign to avoid undue traffic congestion if required; - Inform the local people about the subproject activities; - Inspire local people for using of the connecting roads during construction period; - Avoid the road side storage of materials. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment such as concrete mixer machine, vibrator machine that produce excessive noise at night; - Avoid prolonged exposure to noise (produced by equipment) by the workers; - Regulate use of horns and avoid un-due use of hydraulic horns by subproject vehicles; - Inform local people about the subproject activities and potential impact. |  |
|  | - Felling of trees, clearing of vegetation and ecological disturbances | - 65 nos. of the trees will be planted at the periphery of the Soyani Pond to enhance the ecological condition. |  |
|  | - Destruction of aquatic habitat | - Avoid throwing of the construction waste material in to the water bodies (Soyani Pond); - Any illegal discharge of waste water and spillage shall be prohibited. |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accident; - Provides PPEs and ensure using of the personal protective equipment by the workers. |  |
|  | - Spills and leaks of oil, toxic chemicals | - Refueling and fuel tank should not be placed on the ground directly; - Proper handling of lubricating oil and fuel so that it does not fall on the soil; - Place plane sheet/dry wood and polythene beneath the fuel tank, diesel engine used for vibrator machine, concrete mixer machine etc. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |
|  | - General degradation of the environment | - Ensure environmental enhancement measures, such as 65 nos. of the trees plantation and by placing cautionary sign; - In cautionary signs-text should be written in Bengali and English; - In cautionary signs-front size should be big so that it can be visible from at least 200m distance; - Cautionary signs should be placed at strategic points such as road intersection. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the RCC Drain with cover slab and retaining wall & beautification work at Soyani Pond |  |  |  |
| Dismantling work (Existing brick drain and boundary wall) for site clearing and excavation work | - Generation of loose and clay soil due to the earth excavation work. | - Cover exposed loose dry soil and wastes materials before disposal; - Disposal of soil and construction wastes at existing dumping site near Rabna Bypass. |  |
|  | - Accidents from careless use of hydraulic excavator and hammer | - Carefully handle of the hydraulic excavator and hammer if needed. |  |
|  | - Possible damage of road side infrastructure due to earth excavation for drain construction. | - Ensure drum sheet palisading work for shallow depth to stabilize the structure; - Ensure plunk palisading work for shallow depth to stabilize the structure; - Bolly drive for deep depth construction works. |  |
|  | - Air pollution due to black smoke emission from excavator. | - Regular maintenance of the equipment. |  |
| Sand filling /Back filling work | - Air and dust pollution affecting nearby settlement and worker | - Maintain adequate moisture content of soil and sand during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover; - Ensure use mask for the associate workers. |  |
| Cutting and welding of the reinforcement | - Noise pollution due to rod cutter and welding machine if any | - Avoid using of rod cutter and wielding machine at night; - Avoid prolonged exposure to noise (produced by equipment) by workers. |  |
|  | - Potential health and safety risks from rod cutter and welding machine if any | - Ensure use of the personal protective equipment’s (helmet, goggles, gloves, safety boot); - Availability and access to first-aid equipment and medical supplies in case of any accidents. |  |
| RCC (reinforcement cement concrete) work | - Air pollution due to black smoke emission from concrete mixer machine and vibrator machine | - Regular maintenance of the concrete mixer and vibrator machine to avoid any black smoke emission. |  |
|  | - Noise nuisance from concrete mixer machine and vibrator machine | - Avoid operation of the concrete mixer and vibrator machine at night; - RCC work should be avoided at schooling time; - Inform local people about casting work and potential impacts. |  |
|  | - Health hazard may occur due to spread out of dust and cement material during mixing of aggregate | - Ensure use of masks for construction workers if required; - Ensure saturated fine material i.e. sand before placing into the mixture machine if possible. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the construction of BC Road |  |  |  |
| Dismantle Work (existing damaged BC road) Excavation/ Earth work | - Generation of loose soil due to the earth excavation work. | - Cover exposed loose dry soil and wastes materials before disposal; - Disposal of soil and construction wastes at existing dumping site near Rabna Bypass. |  |
|  | - Accidents due to careless use of hydraulic excavator | - Operate the hydraulic excavator carefully. |  |
|  | - Air pollution | - Regular maintenance of the equipment to avoid black smoke emission. |  |
| ISG (Improved Sub grade)/ Sand filling for the road widening | - Air and dust pollution affecting nearby settlements and workers | - Maintain adequate moisture content of soil during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover; - Ensure use of mask to the associates workers. |  |
| Setting up and operation of asphalt plant | - Possible degradation of the air quality by the suspended particles and increase of the noise level from asphalt plant affecting nearby settlements; | - Locate plant away from residential settlements; - Consider use of emulsified bitumen. |  |
|  | - Possible water pollution (surface and groundwater) by bitumen and solvents; | - Avoid spills and proper collection and disposal of the generated spills. |  |
|  | - Possible preparation of the bitumen in open air and using of charcoal and wood as fuel | - Strictly prohibit bitumen preparation in the open air and use of charcoal and wood as fuel. |  |

Table 2: Anticipated environmental impacts during operation phase and corresponding mitigation and enhancement measures

| Activity/Issues | Potential Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Operation of the BC road | - Increase traffic speed and accidents; - Possible traffic congestion due to movement of increased traffic vehicles; - Possible road damage due to movement of heavy vehicles. | - Better traffic management; - Control heavy traffic movement that may exceed load carrying capacity of the road; | Monitoring by Pourashava |
|  | - Increased air and noise pollution affecting surrounding areas | - Better traffic management is needed. |  |
| Operation and maintenance for drain | - Pollution of downstream water body due to disposal of polluted water from the drain | - Ensure installation of septic tank by the household people in all establishment; - Avoid connecting sanitation facilities to storm drain. |  |
|  | - Possible backflow of the drain water due to blockage in the drain | - Proper maintenance and cleaning of the drain and outfall on regular basis. |  |
|  | - Throwing of the wastes in to the drainage system | - Awareness program for the community people should be taken to increase knowledge about improper waste disposal and its impact. |  |
| Operation and maintenance of the beautification work at Soyani Pond | - Improper use of the Dolna and mini-park facilities may create safety risks to the children; - Throwing of the wastes in to the Pond may degrade pond water quality. | - Care should be taken for using of the Dolna to avoid any accidental risks; - Waste bins should be properly used; - Awareness program for the community people should be taken to increase knowledge about improper waste disposal and its impact. |  |

## Environmental Monitoring Plan

Environmental Monitoring Plan for this subproject will help to evaluate the extent and severity of environmental impacts against the predicted impact and the performance of environmental protection measures. The following Table 6.7.1 has been prepared for the key environmental indicators.

Table 6.7.1: Matrix Table of Monitoring Plan (Visual observation during construction phase)

| Monitored Parameter/ Issues | Monitoring method/ Key aspects | Location of monitoring | Period & Monitoring Frequency |
| --- | --- | --- | --- |
| Safety orientation & training of workers | Frequency of training & orientation of workers for safety | Subproject site | - Once in a month - Reporting: Once in a month |
| Personal Protective Equipment and safety equipment | Ensure every single person involved in the activities wear and use safety equipment | Subproject site | - Daily - Reporting: Once in a month |
| Worker’s health | Monitoring process of worker’s health | Subproject site | - Daily - Reporting: Once in a month |
| Sanitation & drinking water facility to the workers | Availability of safe drinking water and sanitation to the workers | Sub-project site | - Daily - Reporting: Once in a month |
| Incident record & reporting | Documented record of all incident, accident and its remedial process | Subproject site | - Daily - Reporting: Once in a month |
| Site security | Isolation of site from general access | Subproject site | - Daily - Reporting: Once in a month |
| Bulletin/ announcement boards/ prohibition signs | Visible in good condition or not | Subproject site | - Daily - Reporting: Once in a month |
| Equipment /vehicles | Switched-off diesel engines when not in use; Search any possible leakage; Fuelling. | Subproject site | - Daily - Reporting: Once in a month |
| Dust | Dust is visible or not | Subproject site | - Daily - Reporting: Once in a month |
| Oil waste generation and disposal | Quantity of oily waste, storage and disposal | Subproject site | - Daily - Reporting: Once in a week |
| Solid waste generation | Quantity of solid wastes and disposal | Subproject site | - Daily - Reporting: Once in a month |
| Drainage facilities | Provision of open /closed surface drainage | Subproject site | - Monthly - Reporting: Once in a month |
| Gender equity | No discrimination regarding payment | Subproject site | - Once in a month - Reporting: Once in a month |
| Child labour | No child will be engaged in the activities | Subproject site | - Daily - Reporting: Once in a month |
| Handling of hazardous materials | Fuelling, storage, operation | Subproject site | - Daily - Reporting: Once in a month |

In addition, a comprehensive Environmental Compliance Monitoring checklist has been enclosed in Appendix-I.

## Environmental Management Budget

Considering the environmental impacts and their mitigation measures for this subproject, several items are included in the BOQ for environmental management. The estimated cost for environmental management budget is elaborated in Table 6.8.1.

Table 6.8.1: Environmental Management Budget

| Item No. | Description of the Works | Unit Rate (BDT) | Qty (Number) | Amount (BDT) |
| --- | --- | --- | --- | --- |
| eme-1 | a) Dust suppression measures by water spraying throughout the construction period in and around the subproject sites, uncovered aggregates and loose materials such as stockpiles of the sand, excavated earth etc. | Lump sum |  | 10,000.00 |
|  | b) Dust suppression measures for BC road by water sprinkling (it includes water sprinkling for overall BC road construction works that includes-ISG, WBM, ASG and sub-grade) | Lump sum |  | 10,000.00 |
| eme-5 | Prevention of the spillage and leakage of the polluting materials | Lump sum |  | 5,000.00 |
| eme-6 | Campsite wastes disposal facility during the construction period (collection, transportation and dumping of the wastes at authorized dumping site near Rabna Bypass Road: 2 nos. (1 no. for the organic wastes and 1 no. for the inorganic wastes disposal facility) | 20,000.00 | 2.00 | 40,000.00 |
| eme-7 | Campsite water supply facilities: Preferably 1 no. of tube well at the labor campsite (Depending on the site condition, DSM consultant will assist the contractor for selecting the option) | 20,000.00 | 1.00 | 20,000.00 |
| eme-8 | Campsite sanitation facilities: 2 nos. of the toilets preferably sanitary toilets at the labor campsite (1 no. for women and 1 no. for men) | 20,000.00 | 2.00 | 40,000.00 |
| eme-9 | a) Providing safety gear packages like hand gloves, spectacles for eye protection, ear plug, helmets, masks, visible jacket, safety shoes for at least 20 persons (15 workers and 5 visitors) | 3,000.00 | 20.00 | 60,000.00 |
|  | b) One first aid box with necessary accessories (contractor is responsible for providing necessary medicines, saline as per requirement during construction period) | 2,500.00 | 1.00 | 2,500.00 |
| eme-11 | Tree plantation to compensate the felled down trees and enhance the ecological condition in the subproject area- preferably local fruits, flowers, medicinal and ornamental trees-Mango, Jam, Jackfruit, Jarul, Krisnachura, Bokul, Sonalu, Polash, Kadom, Kathbadam, Neem, Amloki, Mahogany, Rain Tree, Palm, Jhau (including protection, fencing and conservation during project defect liability period): Preferably at periphery of the Soyani Pond-65 nos. of the trees | 750.00 | 65.00 | 48,750.00 |
| eme-14 | Cautionary signs-4 nos. | 2,500.00 | 4.00 | 10,000.00 |
| Total |  |  |  | 246,250.00 |

After approval to revise the cost estimate has lengthy complex procedure. Hence, as per project EMF, PMU suggestion and experience from other LGED projects, adequate budget has been allocated for the environmental management for the mitigation and enhancement measures. For this subproject, there is no provision of the analytical monitoring for air quality and noise level because air quality and noise data from the previous subprojects MGSP/TAN/2015-16/W-05 and MGSP/TAN/2016-2017/W-06 can be used as a baseline data to evaluate the impacts. To compensate the felled down trees and to enhance the ecological condition, adequate budget for tree plantation is also included. The budget for labor shed and site office construction is included in the civil works items. Therefore, it is not included in the environmental budget. It should be noted that the contractor will be paid as per actual work done.

## Environmental Codes of Practice

This section identifies and specifies environmental management guidelines and practices to be followed by the contractor for sustainable management of all environmental issues. The Contractor shall carry out the subproject related activities as specified in contract agreement. Tangail Pourashava shall ensure that contractor take due responsibility to mitigate those negative impacts. Environmental awareness creation, particularly about the direct construction impacts and for the health, pollution and safety issues will be Contractor’s responsibility. Clauses that may be incorporated in the tender documents are:

- ECoP-1 (Overall Environmental Protection): Contractor shall take all steps to protect environment and avoid causing all types of public nuisances during implementation;

- ECoP-2 (Labor shed Management): Contractor shall maintain the work camp and construction sites in clean and tidy conditions and shall ensure standard facilities;

- ECop-3(Workforce Environment): Contractor shall engage local people as much as possible where applicable and ensure prohibition of the child labor (less than 18 years) and aged labor (more than 65 years) in heavy works;

- ECoP-4 (Waste Management): Contractor shall be responsible for the safe transportation and disposal of the wastes generated due to the subproject activities;

- ECoP- 5 (Workers Health and Safety): Contractor shall be responsible for providing personal protective equipments and first aid facilities as per requirements;

- ECoP-6 (Compensation for Accidents):  Contractor shall bear medical treatment costs for any accidents. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation- (the amount of the compensation should be fixed considering the type of accidents);

- ECoP-7 (Implementation of the Mitigation Measures): Contractor shall responsible for the implementation of the mitigation measures mentioned in the EMP;

- ECoP-8 (Spill Prevention, Fuels and Hazardous Substances Management): Contractor shall take preventive measures for spill prevention and fuels and hazardous substances management;

- ECoP-9 (Restoration of the Facilities): The contractor on completion of the contract shall remove the equipment, surplus materials, and rubbish and temporary structures of all types and shall leave sites in clean condition to the satisfaction of Pourashava and local people.

# Public Consultation and participation

## Methodology

In the context of preparing the Environmental Assessment (EA), participatory public consultation was conducted. The Pourashava Mayor, Officials, Engineers and local individuals as well as LGED and Consultant participants participated. Informal Focus Group Discussions (FGD) and a formal CIP were conducted involving the participants (Photographs 7.1.1). In addition, walk-through informal group consultations were also held. The local communities were informed about the subproject interventions including their benefits. Suggestions made by the participants were listed and incorporated in the EMP Accordingly.

Photographs 7.1.1: Consultation Meeting at Subproject Site with Pourashava Officials and Consultants Participants

## Issues Raised by the Participants

The participants raised the issues related to the infrastructure development of Tangail Pourashava. They emphasized on the subproject selection for the future development and also discussed about the procedure for the quality construction work. In the FGD, the participants discussed the requirements for the Pourashava future development through a list of the subprojects.

## Feedback, Suggestions, and Recommendations of the Participants

The participants were presented with feedback, suggestions, and recommendations listed below:

- The local people emphasized that construction work should create minimum adverse impact on the environment;

- In the group discussion for the future development of Tangail Pourashava the participants demands (a) Planned and green municipality, (b) Planned hygienic and enhanced municipality, (c) Echo-friendly technology base industrial neighborhood;

- The participants stated that the public water supply facilities, sanitation facilities, drainage congestion and access road is not adequate;

- The FGD results confirmed that an improved communication network, drainage facility, solid waste management, water treatment plant and water supplying system, sewerage treatment plant and sanitation facilities, recreational facilities and health and sports facilities are needed for future development of Tangail Pourashava;

- Local people also believed that the importance of the area would be elevated and various economic activities would be started in the area after subproject implementation;

- Local people also showed strong expectation for the increased opportunities for employment for the local labor in the construction work.

# Conclusions and Recommendations

The subproject intervention has adverse impacts on ecological environment due to felling of the trees. However, 65 numbers of trees will be replanted to compensate felled down trees and to enhance ecological condition. In addition, the subproject intervention has impacts on physicochemical components and socio-economic environment. However, from the study it is revealed that the anticipated impacts on physicochemical components and socio-economic environment are not significant. Most of the individual elements of the subproject involve simple construction and operation. Hence, impacts will be mainly localized and limited within the subproject boundary. Most of the predicted impacts are associated with the construction process which is invasive. Hence, the impacts are largely manageable through mitigation measures. Mitigation will be assured by a program of environmental monitoring to ensure that all measures are implemented, and will determine whether the environment is protected. It will include observations on and off-site, document checks and interviews with workers and beneficiaries. Any requirements for corrective action will be reported to the WB.

This study formulates well-structured EMP. This EMP will assist Pourashava Officials, PMU-LGED, contractor and DSM personnel in mitigating the environmental impacts, and guide them in the environmentally sound execution of the proposed subproject. A copy of the EMP shall be kept on-site during the construction period at all times. The EMP shall be made binding on contractor operating the site, and will be included in the contractual clauses.

It should be noted that environmental assessment report is a live document. Hence, due to changing circumstances during the construction phase if any, there might be minor adaptation needed for environmental, health and safety issues.

# Reference

### Tangail Pourashava Master Plan: 2011-2031 Urban Area Plan;

### Web Site: Tangail Pourashava http://www.tangailpourashava.gov.bd/;

### Environmental Assessment Volume 1: Overall Environmental Assessment [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) and Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh;

### Environmental Assessment Volume 2: Environmental Management Framework (EMF) [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) and Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh;

### Geological Survey of Bangladesh, 1978. Earthquake Zoning Map;

### Bangladesh Meteorological Department, Flood Zoning Map;

### Population and Housing Census, 2011, National Volume-II: Union Statistics.

### APPENDIX

### Appendix- I: Environmental Monitoring Checklist

Local Government Engineering Department

Municipal Governance and Services Project (MGSP)

Environmental Compliance Monitoring Form

Part A: General Subproject Information

| Subproject Name | a) Improvement of Akur Takur Para Choto Kalibari Road from Choto Kalibari Moar to Shimultoli Road (Ch.00-225m, Total length 225m); (b) Construction of RCC Drain at Akur Takur Para Shimultoli Road from Mymensingh Road to Louhajang River via Taltola (Ch.340-755m) with Akur Takur Para Chotokalibari Road; Link 225m, (Total Effective Length 640m); (c) Construction of Retaining Wall and Beautification Work of Soyani Pond (Wall Length-250m) at Thanapara. |
| --- | --- |
| Package No. | MGSP/TAN/2017-2018/W-13 |
| ULB Name | Tangail Pourashava |
| Approved Estimated Cost in BDT |  |
| Contract Amount in BDT |  |
| Contractor Name |  |
| Date of Commencement |  |
| Target Date of Completion |  |
| Physical Progress (%) |  |
| Financial Progress (%) |  |
| Person Responsible (PIU) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Person Responsible (DSM) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Form Completed by (Name, Designation &Phone) |  |
| Overall Environmental Description of the Subproject |  |

Part B: Design, Preparation, and Legal Requirements

| Environmental Concerns (PMU, PIU & DSM) (Name, Designation & Phone) | PMU- |  |
| --- | --- | --- |
|  | PIU- |  |
|  | DSM- |  |
| Subproject Category | DoE-BD- | WB- |
| Environmental Clearance Received? | Yes | No |
| EA Required? | Yes | No |
| EA Prepared and Delivered? | Yes | No |
| EMP Prepared & Delivered Separately? | Yes | No |
| Items and Cost of EMP Implementation Included in the Contract? | Yes | No |
| EMP Included in the Procurement Documents? | Yes | No |
| Inspection Schedule/Last Inspection/Monitoring by PMU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by PIU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by DSM Environmental Safeguard Specialist/Jr. Environmental Specialist | Date- |  |
|  | Key findings- |  |

Part C: Key Environmental Impacts

| The Subproject Results in any of the following Impacts? | Yes | No | NA | If yes, is the impact (give observation) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Significant | Moderate | Minor |
| Felling of the trees |  |  |  |  |  |  |
| Clearing of the vegetation that increase the risk of increased soil degradation or erosion |  |  |  |  |  |  |
| Disturbance of the terrestrial and or aquatic specifies |  |  |  |  |  |  |
| Noise pollution |  |  |  |  |  |  |
| Air pollution |  |  |  |  |  |  |
| Adverse effects on the quantity or quality of the surface water or groundwater |  |  |  |  |  |  |
| Production or increase the production of the solid waste |  |  |  |  |  |  |
| Drainage congestion |  |  |  |  |  |  |
| Water logging that increases the risk of the water related diseases |  |  |  |  |  |  |
| Traffic congestion |  |  |  |  |  |  |
| Public safety |  |  |  |  |  |  |

Part D: Work Place Environment and Gender Equity

| The Subproject Results in any of the following Impacts? | Yes | No | NA | Observations |
| --- | --- | --- | --- | --- |
| Does the contractor pay to the workers regularly? |  |  |  |  |
| Is there any discrepancy between the male and female workers regarding the wages or salary for the same works? |  |  |  |  |
| Is the contractor complying with the GOB labor law concerning the hiring of the workers? |  |  |  |  |
| Does the contractor engage women labors and does the project have suitable works for them? |  |  |  |  |
| Does the contractor engage child labor (less than 18 years) and aged people (more than 65 years old)? |  |  |  |  |
| Does the contractor force to the workers for the completion of the works? |  |  |  |  |
| Do the workers involve with the political activities, crime, drugs addiction and other forms of unwanted activities? |  |  |  |  |
| Are construction camps adequately equipped with water supply, sanitary toilets, washing facilities and facilities for waste collection and storage? |  |  |  |  |
| Has separate sanitation facilities been provided for women at work camps and the construction site? Do the laborers load heavy items on their heads or shoulders? |  |  |  |  |
| Has the contractor undertaken an awareness program for the sexually transmitted diseases especially for HIV-AIDS and other infectious diseases like TB? |  |  |  |  |
| Are first aid kits readily available for the workers at the job site along with the instructions for use? |  |  |  |  |
| Are supervisors or other site personnel trained in the basic first aid emergency response measures? |  |  |  |  |
| Has the contractor provided necessary safety equipment to the workers and training for use? |  |  |  |  |

Part E: Potential Impacts, Mitigation Measures, and Monitoring Indicator Mentioned in the EMP

| Activity/ Issues | Potential Impacts | Proposed Mitigation and Enhancement Measures | Monitoring Method |  | Monitoring Frequency |  | Implementation Status (Yes/No /NA) |  | Impact significance if not implemented |  |  |  |  |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  | Major |  | Moderate |  | Minor |  | No |  |  |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | Generation of sewage and solid waste may pollute surrounding environment | - Construction of sanitary latrine (considering 15 person for one toilet) at the labor shed and separate toilet for women; - Introduce separate waste bins for organic and inorganic waste materials at camp site; - Ensure wastes (solid wastes and other forms of the waste) disposal near Rabna bypass dumping site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Health of workers | - Comply with requirements of Government of Bangladesh labor law of 2006 and all applicable laws and standards on worker’s Health and Safety. - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment’s (one first aid box) and necessary medical supplies for the workers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well-being of the local people. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General construction Works | Drainage congestion and flooding | - Ensure provision for adequate drainage of storm water if needed; - Ensure provision for pumping of congested water if needed; - Ensure adequate monitoring of drainage effects, especially if construction works are carried out during the wet season. - Ensure select place for the storage of soils and other construction materials to avoid disturbance of natural drainage. - Dispose of construction materials and equipment appropriately so that they do not impede the local drainage. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces/ unpaved roads regularly to reduce dust generation; - Maintain adequate moisture content of the sand used for bed preparation and backfilling; - Ensure contractor sprinkle water and cover stockpiles of loose materials (e.g., fine aggregates); - Inform local people about the subproject activities and potential impact. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of material/ equipment during off-peak hours; - Place traffic sign and cautionary sign to avoid undue traffic congestion; - Inform local people about the subproject activities; - Inspire local people for using the connecting and diversion road during construction; - Follow section wise construction work and follow road intersection for road safety; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Noise pollution | - In front of the road side educational institute, residents construction work should be performed at holiday and or during break time; - Check and maintenances the equipment’s properly; - Avoid using of construction equipment’s producing excessive noise at night; - Regulate use of horns and avoid use of hydraulic horns in the subproject vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes in the road side area and at stack yard. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Accidents | - Conduct formal and informal discussion for creating awareness about the accident; - Provides PPEs and ensure using of the personal protective equipment by the workers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Felling of trees, clearing of vegetation and ecological disturbances | - 65 nos. of the trees will be planted at t periphery of the soyani pond to enhance the ecological condition.. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Destruction of aquatic habitat | - Avoid throwing of the construction waste material in to the water bodies (Soyani Pond); - Any illegal discharge of waste water and spillage shall be prohibited. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil; - Collection and proper disposal of spills; - Refueling and fuel tank should not be placed on the ground directly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| All construction works | Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within project area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | General degradation of the environment | - Ensure environmental enhancement measures, such as 65 nos. of the trees plantation and by placing cautionary sign. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Possible complaints and suggestion from the local people and stakeholder about the subproject activities | - Use existing grievance resister and complaints box that has been already delivered in the Pourashava |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specific Activities for BC Road |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dismantle work for the existing damaged BC road/Excavation/Earth work | Generation of solid and construction waste due to the dismantle works; Generation of loose soil due to the earth excavation work and earth work. | - Cover expose construction wastes and loose dry soil with fabric; - Disposal of soil and construction wastes at dumping site near Rabna bypass. - Ensure reuse of generated solid waste if possible. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Accidents | - Operate hydraulic excavator for excavation and hammer for dismantle work carefully. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Air and dust pollution affecting nearby settlements | - Regular maintenance of the excavator equipment to avoid black smoke emission. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ISG (Improved Sub grade)/ Sand filling | Air and dust pollution affecting nearby settlement and on site worker | - Maintain adequate moisture content of the soil during construction transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. - Ensure use of masks to construction workers if dust content is high. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Setting up and operation of asphalt plant | Possible degradation of the air quality by the suspended particles and increase of the noise level from asphalt plant affecting nearby settlements; | - Locate plant away from residential settlements; - Consider use of emulsified bitumen. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Possible water pollution (surface and groundwater) by bitumen and solvents; | - Avoid spills and proper collection and disposal of the generated spills. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Possible preparation of the bitumen in open air and using of charcoal and wood as fuel. | - Strictly prohibit bitumen preparation in the open air and use of charcoal and wood as fuel. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Specific Activities for RCC Drain with cover slab and RCC retaining wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Dismantle work for the existing brick drain/ Excavation/Earth work if any | Generation of solid & construction wastes due to the dismantle works; Generation of loose soil due to the earth excavation works | - Cover exposed earth works with fabric; - Disposal of soil and construction wastes to Rabna bypass dumping site. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Accident | - Operate the hydraulic excavator carefully; - Operate the hummer carefully for the dismantle work. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Air pollution due to black smoke emission from excavator | - Regular maintenance of the equipment to avoid black smoke emission. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Possible damage of road side infrastructure due to earth excavation for drain construction | - Ensure drum sheet palisading work for shallow depth to stabilize the structure; - Ensure plunk palisading work for shallow depth to stabilize the structure; - Bolly drive for deep depth construction works. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sand filling/Back filling work | Air and dust pollution affecting nearby settlement and worker | - Maintain adequate moisture content of soil and sand during transportation, compaction and handling; - Carry the materials especially loose soil and sand with adequate cover. - Ensure use of face mask for the associates worker |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cutting & welding of the reinforcement | Noise pollution due to rod cutter and welding machine if any | - Avoid using of rod cutter and wielding machine at night; - Avoid prolonged exposure to noise (produced by equipment) by workers. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Potential health and safety risks from rod cutter and welding machine if any | - Ensure use of the personal protective equipment’s (helmet, goggles, gloves, safety boot); - Availability and access to first-aid equipment and medical supplies in case of any accidents. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RCC (reinforcement concrete) work | Air pollution due to black smoke emission from concrete mixer machine and vibrator machine | - Regular maintenance of the concrete mixer and vibrator machine to avoid any black smoke emission. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Noise nuisance from concrete mixer machine and vibrator machine | - Avoid operation of the concrete mixer and vibrator machine at night; - RCC work should be avoided at schooling time; - Inform local people about casting work and potential impacts. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Health hazard may occur due to spread out of dust and cement material during mixing of aggregate | - Ensure use of masks for construction workers if required. - Ensure saturated fine material i.e. sand before dispose into the mixture machine if possible. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Prepared by-                                                                                                 Signature-                                                                   Date-

Copies to 1.  PIU

2. SME

3. AME

4. Jr. ES