| GOVERNMENT OF THE PEOPLE’S REPUBLIC OF BANGLADESH |  |  |  |  |
| --- | --- | --- | --- | --- |
| Local Government Engineering Department (LGED) Local Government Division Ministry of Local Government, Rural Development, and Cooperatives |  |  |  |  |
| Environmental Assessment (EA) REPORT Name of the Subproject: Construction of RCC Road from Ellenga College Moar to Ellenga Bazar and RCC Drain at Ellenga Bazar. Package No: MGSP/ELN/2017-18/W-07 Ellenga Pourashava, Kalihati, Tangail Municipal Governance and Services Project (MGSP) Design, Supervision, and Management (DSM) Consultancy Services |  |  |  |  |
| Joint Venture of Hifab International AB, Sweden AQUA Consultant and Associates Ltd., Bangladesh |  |  |  |  |

TABLE OF CONTENTS

|  |  |  | Page No. |
| --- | --- | --- | --- |
| 1.0 | INTRODUCTION |  | 1-3 |
|  | 1.1 | Subproject Background | 1 |
|  | 1.2 | Objectives of the Study | 2 |
|  | 1.3 | Scope and Methodology of the Study | 2 |
| 2.0 | SUBPROJECT DESCRIPTION |  | 4-10 |
|  | 2.1 | The Study Area | 4 |
|  | 2.2 | Current Situation, Proposed Intervention and Need For the Subproject | 8 |
|  | 2.3 | Justification of Selection of this Subproject | 9 |
|  | 2.4 | Envisaged Subproject Activities and Implementation Process | 9 |
|  | 2.5 | Category of the Subproject | 10 |
|  | 2.7 | Subproject Schedule | 10 |
| 3.0 | BASELINE ANALYSIS OF ENVIRONMENTAL CONDITION |  | 11-16 |
|  | 3.1 | Physical Environment | 11 |
|  | 3.2 | Biotic Environment | 15 |
|  | 3.3 | Socio-economic and Socio-cultural Environment | 15 |
| 4.0 | ENVIRONMENTAL SCREENING |  | 17-19 |
| 5.0 | SPECIFIC IMPACT, MITIGATION AND ENHANCEMENT MEASURES |  | 20-26 |
|  | 5.1 | Dismantle Work, Site Clearing, Excavation and Earth Work | 20 |
|  | 5.2 | Pollution from the Construction Materials | 20 |
|  | 5.3 | Air Quality and Dust | 21 |
|  | 5.4 | Noise and Vibration | 21 |
|  | 5.5 | Water Quality | 21 |
|  | 5.6 | Occupational Health and Safety | 22 |
|  | 5.7 | Impacts on Social Environment and Common Property Resources | 23 |
|  | 5.8 | Labor Influx and Anticipated Impacts | 24 |
|  | 5.9 | Impacts on Traffic Movement | 25 |
| 6.0 | ENVIRONMENTAL MANAGEMENT PLAN |  | 27-38 |
|  | 6.1 | Access to Information | 27 |
|  | 6.2 | Grievance Redress Mechanism | 27 |
|  |  | 6.2.1 Grievance Redress Committee (GRC) | 27 |
|  |  | 6.2.2 Grievance Resolution Process | 28 |
|  | 6.3 | Institutional Arrangement for Safeguard Compliances | 29 |
|  | 6.4 | Capacity Building | 30 |
|  | 6.5 | Emergency Response and Disaster Management | 30 |
|  | 6.6 | Environmental Management Action Plan | 31 |
|  | 6.7 | Environmental Monitoring Plan | 36 |
|  | 6.8 | Cost of Environmental Enhancement Works in BOQ | 37 |
| 7.0 | PUBLIC CONSULTATION AND PARTICIPATION |  | 39-40 |
|  | 7.1 | Methodology | 39 |
|  | 7.2 | Issues Raised by the Participants | 40 |
|  | 7.3 | Feedback, Suggestions and Recommendations of the Participants | 40 |
| 8.0 | CONCLUSIONS AND RECOMMENDATIONS |  | 41 |
| 9.0 | REFERENCES |  | 42 |
| APPENDIX |  |  |  |
| Appendix I: Environmental Monitoring Checklist |  |  | 43 |

# LIST OF TABLES

| - Table 3.1.1 | Major Environmental and Infrastructural Features for the Ellenga College Moar to Ellenga Bazar Road |  |
| --- | --- | --- |
| - Table 3.1.2 | Temperature, Rainfall and Humidity during 2001-2012 |  |
| - Table 5.6.1 | - General Requirements for the Workers Health and Safety |  |
| - Table 5.7.1 | Impacts on Social Environment and Common Property Resources |  |
| - Table 6.2.1.1 | - List of GRC committee members |  |
| - Table 6.6.1 | - Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures (EMP Table) |  |
| - Table 6.6.2 | - Anticipated Environmental Impacts during Operational Phase and Corresponding Mitigation and Enhancement Measures (EMP Table) |  |
| - Table 6.7.1 | - Matrix Table of Monitoring Plan (Visual observation during construction phase) |  |
| - Table 6.8.1 | - Environmental Measurement Budget |  |

# LIST OF FIGURES

| - Figure 2.1.1 | - Location Map of the Subproject Site (Ellenga College Moar to Ellenga Bazar) |  |
| --- | --- | --- |
| - Figure 2.1.2 | - Location Map of the Subproject Site (Link Road at Ellenga Bazar) |  |
| - Figure 2.1.3 | - Layout Plan of the Subproject Site |  |
| - Figure 2.1.4 | - Topographic Features of the Subproject Site |  |
| - Figure 6.3.1 | - Environmental and Social Management Team (Tentative) |  |

# LIST OF PHOTOGRAPHS

| - Photographs 2.2.1 | - Current Situation of the Subproject Site |  |
| --- | --- | --- |
| - Photographs 7.1.1 | - Consultants Team Visited the Subproject Site with Pourashava Officials, Local Councilor and Mayor of Ellenga Pourashava. |  |
| - Photographs 7.1.2 | - Consultation Meeting at Subproject Site with Local Individuals. |  |

# ABBREVIATIONS

| AP (AP’s) | Affected Person |
| --- | --- |
| BDT | Bangladeshi Taka |
| BOQ | Bill of Quantity |
| CC | Cement Concrete |
| CIP | Capital Investment Plan |
| CP | Contingency Planning |
| DSM | Design, Supervision, and Management |
| EA | Environmental Assessment |
| ECR | Environmental Conservation Rules |
| EMP | Environmental Management Plan |
| EPP | Emergency Preparedness Planning |
| ES | Environmental Screening |
| FGD | Focal Group Discussion |
| GoB | Government of Bangladesh |
| GRC | Grievance Redress Committee |
| GRM | Grievance Redress Mechanism |
| GRP | Grievance Redress Procedure |
| LGED | Local Government Engineering Department |
| MGSP | Municipal Governance and Services Project |
| PD | Project Director |
| PMU | Project Management Unit |
| RCC | Reinforcement Cement Concrete |
| RP | Relevant Reports |
| TL | Team Leader |
| ULB | Urban Local Body |
| WB | World Bank |
| XEN | Executive Engineer |

# INTRODUCTION

## Subproject Background

Ellenga Pourashava is situated in kalihati Upazilla under Tangail district. Ellenga Pourashava was established in the year of 2011. The Pourashava cover an area of 23.24 sq. km. The present population of the Pourashava is 55000. Among them 26,950 are male and 28050 are female (Wikipedia, 2017).

As a newly developed Pourashava, Ellenga needs continuous development of the infrastructures. In fact, road network condition of the Ellenga Pourashava is substandard. Importantly, most of the core area’s road is very much inconvenient for the traffic movement. Hence, to boosting the economic growth and minimize the trouble of the traffic movement and expansion of the Pourashava construction of road is necessary.

In recent days, there has been a trend of growing importance of urban areas. This subproject emphasizes on the planning to address the problems and to guide future planning of Ellenga Pourashava. Without adequate infrastructure and services provision, it would be difficult to turn urban centers as environmentally congenial livable places. Hence, as a part of the peripheral infrastructural development work, this subproject will significantly contribute infrastructure development of Ellenga Pourashava.

This subproject includes construction of RCC Road and Drain at Ellenga Bazar Road. The major features of the subproject are mentioned below:

| Name of the Subproject | (a) Construction of RCC Road from Ellenga College Moar to Ellenga Bazar (Ch.0+00- Ch. 700m) including Link Road at Ellenga Bazar Area (Ch.0+00- Ch. 160m); (b) Construction of RCC Drain in Link Road from Ch.0+00- Ch. 180m. |
| --- | --- |
| Package No. | MGSP/ELN/2017-18/W-07 |
| District Name | Tangail |
| ULB Name | Ellenga Pourashava. |
| Jurisdiction Area | Ward Number 6 |
| Structural Design Option | RCC Road and Drain |
| Beneficiary Population | About 10,000 as per information by the Pourashava Officials |
| Tribal People | No tribal people settlement found in the subproject area |
| Land Acquisition | No land acquisition is needed |
| - Estimated Cost | 24.094 million in BDT |
| Subproject Duration | 12 months |
| Tentative Start Date | 10 January 2019 |
| Tentative Completion Date | 09 January 2020 |

## Objectives of the Study

The general objective of this study is to determine the major environmental impacts succeeding from execution of the subproject and to recommend mitigation measures to avoid or reduce adverse environmental impacts and to enhance positive impacts. The specific objectives include:

- To assess the existing environmental conditions of the subproject site and its influence area;

- To identify and assess impacts resulting from the subproject during its construction phase and operation phase;

- To develop a environmental management plan with recommendations for mitigating impacts and enhance positive impacts;

- To summarize environmental monitoring requirements.

## Scope and Methodology of the Study

The subproject appraisal is included environmental screening for all the subproject components. According to that screening, environmental assessment is required to fulfill the regulatory requirement for RCC Road and Drain. The study methodology comprised the following activities:

- Desktop Study;

- Field Investigations and Data Collection;

- Data Analysis and Report Writing.

Desktop Study

The desktop study involved:

- Initial meetings with client, stakeholders to discuss the proposed subproject;

- Collection and review of the baseline data, maps, reports and other relevant information;

- Review of the relevant existing legislation, regulation and policies;

- Understand the anticipated technical processes that may affect the environment.

Field Investigation and Data Collection

A team of the consultants made a field investigation to the proposed site. Field investigations involved mainly site walks at the subproject areas for identification of the environmental components to be affected due to the subproject intervention.

The following key tasks were performed during the field visit:

- Taking photographs of the significant aspects to assist in describing the baseline environmental conditions of the subproject area;

- Interviews with representatives of the relevant Pourashava officials and beneficiary and affected people within the subproject influence zone;

- Obtaining relevant documents from Pourashava and local people within the subproject influence zone;

- Verifying information and data collected during the desktop study and to collect new information.

Data Analysis and Report Writing

The data and information collected from all the sources (literature review, secondary and primary data, public consultation) were analyzed to describe the existing environmental setting of the subproject area, to identify the potential positive and negative impacts of the proposed subproject, as well as to provide preliminary suggestions for mitigation measures. Finally, this environmental assessment report has been prepared.

# SUBPROJECT DESCRIPTION

## The Study Area

The subproject site is located in the jurisdiction of ward number 6 of Ellenga Pourashava. There are no protected areas in or near the subproject location. The geographic coordinate of the proposed site at staring point is Latitude 24°20'6.27"N, Longitude 89°55'14.68"E and at end pint is Latitude 24°20'31.23"N, Longitude 89°55'15.73"E. The location map and layout plan of the proposed bridges with approach roads are shown in Figure 2.1.1 and Figure 2.1.2.

Figure 2.1.1: Location Map of the Subproject Site (Ellenga College Moar to Ellenga Bazar)

Figure 2.1.2: Location Map of the Subproject Site (Link Road at Ellenga Bazar)

Figure 2.1.3: Layout Plan of the Subproject Site

- Figure 2.1.4: Topographic Features of the Subproject Site

## Current Situation, Proposed Intervention and Need For the Subproject

The subproject site is situated within the jurisdiction of the Ward number no. 6 of the Ellenga Pourashava. The subproject area is the urban area of the Pourashava. The important features of the subproject area includes: road side permanent and semi-permanent tin-shed and semi pucca and pucca structures of residential houses and shops, Pourashava bhabon, mosques, Ellenga Bazar, Madrasha.

Proposed road starts from Ellenga College Moar and ends at Ellenga Bazar. In addition, 160m long RCC link road will be constructed in the Ellenga Bazar area. It is the busy area of the Pourashava due to commercial activities of Ellenga Bazar and as well administrative work in the Pourashava Office.

From Ch. 0-280m (approximately) and link road is HBB road. Rest of the portion is BC road. Except last 100 m section, the entire road is badly damaged. Due to missing and displacement of the bricks at the HBB road, some portions forms earthen muddy surface. There are lots of pot holes at the road surface. In fact, the whole road is very much inconvenient for the normal traffic movement. Additionally, width of the existing road is not uniform. Hence, to improve the situation existing HBB and BC road will be replaced by RCC pavement with uniform road width from Ch. 0-700m and at link road.

On the contrary, there is no drain in the Ellenga Bazar area. In addition, Ellenga Bazar area has low elevation than adjacent to the road. Hence, after each and heavy rain, Ellenga Bazar area becomes water logged and creates unhygienic environment. To improve the situation, 180m RCC drain is being proposed in the Ellenga Bazar area. This drain will be connected with Louhojong River. The present situation of the existing bridge and earthen access and link road is further elaborated in the following Photographs 2.2.1.

Photograph 2.1.1: Current Situation of the Subproject Site

## Justification of Selection of this Subproject

The subproject road has significant importance because it goes through core area of the Pourashava. Most importantly, Ellenga bazar is located beside the proposed road. Hence, this subproject has been considered as priority subproject in the CIP. PMU representative, and DSM consultant visited the subproject site to assess the site condition.  From the site visit, it is revealed that this road is very much inconvenience for the movement of the motorized vehicles. The subproject is located mostly in the right of way. Since the Pourashava is owned the subproject. Hence, land acquisition is not an issue for implementation of this subproject. From the site inspection, it is also revealed that roadside built-up infrastructure will not be severely affected by the implementation of the subproject. After completion, the road subproject will provide uniform width. By eliminating pot holes and providing smooth road surface it will provide better, easy and safe movement facilities for the travelers.

On the contrary, after construction the drain will minimize the water logging problem in the Ellenga Bazar area. The subproject has significant benefit to the community people. Hence, considering the benefits that will derive, the subproject has been selected for implementation.

## Envisaged Subproject Activities and Implementation Process

RCC Road includes:

- Clearing and Grubbing,

- Earth Work in Box Cutting,

- Preparation of Bed by Cutting and Filling,

- Mechanical Compaction of the Earth Work,

- Sand Filling On the Road Bed,

- Manufacturing and placing of CC blocks;

- Fabrication of the Ribbed or Deformed Bar Reinforcement,

- Reinforced Cement Concrete Work.

RCC Drain includes:

- Earth work in excavation of the foundation;

- Pumping and bailing out of water as per requirement;

- Sand filling for the preparing foundation bed;

- Laying polythene sheet;

- Plain cement concrete work in foundation;

- Manufacturing CC blocks;

- Fabrication of the ribbed or deformed bar;

- Reinforced cement concrete work.

The materials and resources to be used for the key activities: soil in earth work, sand, stone chips, cement, reinforcement, diesel, MS sheet, steel shutter, electricity and water.

The major equipment to be used for the implementation of the subproject: excavator, concrete mixer, diesel engine, steel cutter, steel shutter, mechanical vibrator, steel drum rollers, dump truck, water tanker and pump.

## Category of the Subproject

For RCC Road and RCC Drain

- According to ECR 1997: Green □    Orange□  A  Orange B □√  Red □  Not Listed □

- According to WB classification: Category B □√		 Category C □

In the Project EMF, RCC drain is categorized as Orange A or Orange B depends on environmental impact. In the Project EMF, road is categorized as Orange B. considering the anticipated environmental impacts, primarily RCC Road and RCC drain, can consider as Orange-B category as per ECR-97. According to the WB classification, it can classify as Category B.

## Subproject Schedule

The tentative schedule of construction of the subproject is:

(a)  Subproject Duration (months)  	: 12 months

(b)  Tentative Start Date    			: 10 January 2019

(c) Tentative Completion Date  		: 09 January 2020

The daily construction hours will normally include regular working time. However, daily working hours may vary based on the on-site condition.  The detailed work program will be prepared by the contractor with the assistance of the PIU. Then it will be shared with the PMU, LGED and DSM consultants. In addition, the detailed work program will also be shared with the Bank as per requirement.

# Baseline ANALYSIS OF THE ENVIRONMENTAL CONDITION

## Physical Environment

### Important Environmental and Infrastructural Features

During site visit effort has been given for getting the major environmental features in the subproject area. The major environmental features listed considering the surroundings of the subproject site. The major environmental features are given in the Table 3.1.1 for Ellenga College Moar to Ellenga Bazar (Ch.0+00 - Ch.0+700km) and Link Road at Ellenga Bazar Area (Length-180m).

Table 3.1.1: Major Environmental and Infrastructural Features for the Ellenga College Moar to Ellenga Bazar Road

| Chainage (m) | Left | Right | Major Environmental and Infrastructural Features |
| --- | --- | --- | --- |
| Construction of RCC Road from Ellenga College Moar to Ellenga Bazar (Ch.0+00- Ch. 0+700km) |  |  |  |
| 0-100 | √ |  | Two Storied Mondol Plaza, Bagan Bari Market, Varieties Shops, Hazrat Asia Khatun Mohila Madrasa, Electric Pole, Tin-shed Shops |
|  |  | √ | Under Constructed Building, RCC Drain, Trees, Varieties Shops, Janata Bank |
| 100-200 | √ |  | Connecting Road, Vai Vai Super Market, Varieties Shops, Electric Pole |
|  |  | √ | Varieties shops, Ellenga Pourashava, Electric Pole, RCC Drain |
| 200-300 | √ |  | Faruk House, Shuvon Traders, Pucca, Semi Pucca and Tin Shed Commercial Structure, |
|  |  | √ | Rabbi New Modern Steel, Samiya Steel Workshops, RCC Drain, Electric Pole |
| 300-400 | √ |  | Shre Shre Radha Gobindho Temple, Tonmoy Jewelry, Electric Pole, Varieties Shops |
|  |  | √ | Gu Khado Shops, RCC Drain, Tube Well, Jinnat Sound System, Bismillah Homeo Hall, |
| 400-500 | √ |  | Stationary Shops, Agro Industry Trust, Electric Pole, |
|  |  | √ | Joy Enterprise, Ma Bosraloy, Debi Jewelry, Bipul Jewelary, RCC Drain |
| 500-600 | √ |  | Electric pole, Varieties Shops, Ellenga Bazar, |
|  |  | √ | Varieties Shops, Open Space, RCC Drain, Louhojong River, |
| 600-700 | √ |  | Manik Enterprise, Ma Bosraloy, Varieties Shops, Ellenga Bazar |
|  |  | √ | Pond, Lucky Hardwar Shops, Temple, Varieties Shops, Open Space, RCC Drain, Louhojong River, |
| Link- Ellenga Bazar Area- (160m) |  |  |  |
| 00-100 | √ |  | Pucca, Semi-pucca and Tin Shed Residents and Commercial Structure, Trees |
|  |  | √ | Ellenga Bazar, Electric Pole |
| 100-180 | √ |  | Pucca, Semi-pucca and Tin Shed Residents and Commercial Structure, |
|  |  | √ | Ellenga Bazar, Electric Pole |

**Note: There are 8 no electrical pole and GI pole to be re-located for the implementation of the subproject.

### Geology, Topography, and Soils

The Ellenga Pourashava area is low to medium high land with respect to the topography of the surrounding area. Average ground level varies from 11.5 to 13.5 m PWD. The maximum elevation is 16.4m PWD and the minimum elevation is 3.3 m PWD. Geology of the Ellenga Pourashava belongs to alluvial Sand. Surface geology is consisted of the sedimentary rocks originated by revering processes. The aquifers are generally thick multilayered with high transmissivity and storage coefficient. The soil is made of the non-calcareous alluvium soils. The area is occupied by the sandy and silty alluvium, rich in weather able K minerals that are slightly alkaline in reaction. Six general soil types occupy the area. Organic matter status is low and fertility status is low to medium. The area lies in the earthquake Zone 2 according to the seismic map of Bangladesh (GSB, 1978).

Climate and Meteorology

The climate of the subproject area can be described as Tropical Monsoon. It is characterized by warm, humid summers and cool, dry winters. From November to March, it is dry and cool while from April to May it is extremely hot during the pre-monsoon season. From June to October, the monsoon season is warm, cloudy, and wet. The warmest month is April, the coolest is January, the wettest is July and the driest is January. It is observed that temperature fluctuation from 1 to 2°C and humidity also varied from 1 to 2 % each year through all over the country. Inadequacy of very recent data, to get an idea we used metrological data from 2001 to 2012 of Tangail district for this subproject.

The minimum temperature was 10.1°C in 2011 and maximum temperature was 35.4°C in 2009. The minimum humidity was 64% in 2010 and the maximum humidity was 79.0% in 2009. The mean annual rainfall is 1452 mm. About 92% of the total annual rainfall occurs in the period from May to October.  The highest annual rainfall during the last ten years (2001–2012) was 1830 mm in 2011, and the lowest was 1031 mm in 2009.

According to the statistics of the wind data from the Bangladesh Meteorological Department Climate Division, wind direction changes by month. Nevertheless, the northwest, south, and northeast winds are predominant. The monthly average wind speed as recorded that the wind speeds mostly remain in the range of 2 – 4 knots/ hr. However, in the winter months it remains in the range of 1- 2.5 knots/hour. The maximum monthly speed is generally in the period of the month Mar-Apr reaching a maximum of 6.30 Knots per hour. Table 3.1.2 shows the key meteorological data from 2001 to 2012 (BMD, 2013).

Table 3.1.2: Temperature, Rainfall and Humidity during 2001-2012

| Month | Jan | Feb | Mar | Apr | May | Jun | Jul | Aug | Sep | Oct | Nov | Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Rainfall (mm) | 6 | 10 | 54 | 83 | 238 | 322 | 331 | 253 | 265 | 191 | 27 | 7 |
| Mean Temp (°C) | 16.5 | 20.5 | 25.0 | 27.9 | 28.4 | 28.7 | 28.8 | 28.9 | 28.5 | 26.8 | 22.7 | 18.4 |
| Max Temp (°C) | 27.8 | 32.0 | 36.2 | 37.2 | 37.3 | 36.6 | 35.2 | 35.4 | 35.7 | 35.0 | 32.2 | 29.0 |
| Min Temp (°C) | 7.7 | 10.0 | 13.7 | 19.0 | 20.4 | 22.6 | 24.2 | 24.3 | 24.0 | 19.7 | 13.8 | 9.6 |
| Humidity (%) | 81 | 73 | 69 | 74 | 78 | 83 | 85 | 84 | 85 | 83 | 80 | 82 |
| Sunshine (Hours) | 5.8 | 7.8 | 7.8 | 7.9 | 6.8 | 4.1 | 4.3 | 4.8 | 4.5 | 6.0 | 7.3 | 5.5 |

The construction works can be influenced by the climatic condition and meteorological components like humidity, temperature, sudden rainfall, and wind speed. During high winds, there might be possibility of quick spreading of the dust generated from the construction activities. It is very risky to work during rain and in high winds because the possibility of getting injury increases. Furthermore, work under high temperature and excess humidity is extremely difficult and may create dehydration problem.

### Hydrology (Surface Water, Ground Water and Rainwater)

The Louhajang River, Pathailkandi Canal and the ponds, ditches are the main sources of the surface water for the Pourashava area. Surface water in the river, ponds and ditches is not saline. Nevertheless, it needs treatment for drinking. However, the river, canal, ponds and ditches water can be used both for the domestic purposes (especially for bathing, cleaning and washing) and construction works.

Groundwater is the main source of the potable water in the subproject area. Deep groundwater is not saline and normally arsenic and iron free. Local people typically use deep tube-well water for drinking and other domestic purposes. Salinity problems are not commonly visible. Rain water harvesting system is not common in and around the subproject area.

The construction period is normally in summer season. During the summer, generally the ground water and surface water level goes down. In extreme hot weather condition, the pond and ditch water dried out. Therefore, ensuring the water requirements for the construction works and domestic uses are the key issue in dry summer. On the contrary, if the construction period also includes wet summer, there might be less complexity for ensuring the water requirements due to rainfall and excess humidity. The intention of the discussion is that to reduce the use of the ground water resource and to maximize the use of surface water resource by the municipality.

### Flooding, Water Logging, and Drainage Pattern

Due to continuous heavy rain, temporarily this area is subjected to water logging problems. Structured and functional drainage system in the subprojects area is entirely absent. The existing earthen drain is also not functional because people throw and dispose wastes in the drains. Therefore, due to absent of the structured drainage system, sometimes delayed discharge of the storm water causes water congestion.

### Air Quality and Dust

Ambient air quality data have not been found. Air seems to be clean. However, due to poor condition of the earthen road dust is generated during the movement of the vehicles and windblown dust cause air pollution. There are no remarkable sources of air pollution such as heavy and small industries as well as brick fields observed in the subproject area.

### Noise Level

Subproject components are in the built-up part of Ellenga Pourashava, which mainly buildup with residential and commercial activities. The volume of traffic that passes through these sections is not significant but sometimes road side loading and unloading of goods creates short terms traffic jams. So, vehicular movement can be considered as major cause of noise pollution particularly near to the Ellenga bazar. In addition, sophisticated machineries will be used at construction period which will produce little significant noise. But it would be temporary and site specific. Furthermore, there are no other perceptible sources of noise generation such as heavy factories or industries were found near the subprojects area.

Solid Waste Management

In Ellenga municipality, each day one-ton solid waste generates in total. There is no structured solid waste management system (Public trash bins/dustbins or Transfer Station) in the municipality. Ellenga municipality has only one truck to transport solid waste. But no designated dumping zone in this municipality. Presently, municipality dumped solid waste on roadside selected place at Moshajan. Lack of public trash bins /dustbins and dumping zone makes the municipal’s solid waste management initiatives futile and pollutes the environment. There is no designated site for disposal of solid waste; these are disposed in low lands. Blockage of drains by solid waste is therefore a major concern.

## Biotic Environment

Flora and Fauna

The subproject area is the developed urban area. So, road side planed and natural grown trees is less. However, diversify terrestrial flora species is found in the subproject influence area. In the homestead area, planted trees includes- Akashi (Acacia auriculiformis), Mango (Mangifera indica), Goava (Psidium guajava), Tal (Borassus flabelliformis), Kejur (Phoenix paludosa), Koroi (A. lebbeck), Kath Badam (Terminalia catappa),Coconut (Coca’s nucifera), Banana (Musa paradisiacal), Supari (Areca catechu), Eucalyptus (Eucalyptus teritocornis), Ata (Anona retcualta), Jackfruits (Artocarpus heterophyllus), Bokul (Mimusops eleng), Mahogany (Swietenia macrophylla), Shisho (Dalbergia sissoo), and krisnochura (Caesalpinae Delonix) etc.

There are common local birds such as Crow (Corvus splendens), Dowel (Copsychus saularis), Bulbuli (Pycnonotus barbatus), and Tuntuni (Orthotomus sutorius), Shalik (Acridotheres fuscus), Choroi (Passer domesticus), Bok (Mesophoyx intermedia),) etc. were found during the site visit.

## Socio-economic and Socio-cultural Environment

Land Use, Status of Housing and Built-up Infrastructure

The subproject sites are located in the urban area of the Pourashava and it’s consist of mix land use pattern includes commercials, administrative and residential area. The built up infrastructure includes Pourashava office buildings, post office, personal homes, supermarkets, government offices, NGO Offices, Ellenga Bazar and health care services etc.

### Beneficiary Population

This subproject site is sited in the jurisdiction area of the Ward number 06. As per information by the municipality, considering the wards’ population about 10,000 people will benefit directly and many others indirectly.

### Education

In the subprojects area, literacy among the population is about 42.9%. This is lower than the national average (51.8%). The literacy rate among males (46.1) is still more than females (39.8) (Population and Housing Census 2011).

### Tribal Communities

There is no indigenous or tribal people settlement in the subproject area. Therefore, there is no measure needed for indigenous peoples’ safeguard.

Principal Livelihoods and Economic Activities

The subprojects area is now inhabited by the mixed occupational people where, the major income comes from non-farm activities such as business, enterprises, jobs, transport vehicle ownership, and operation. Presently, a significant number of people work in the small trades, private sector jobs and government jobs in the town, and transport operations. Increased awareness of the social issues and NGO activities has changed the poverty scenarios of the subprojects area.

Cultural Heritage and Protected Areas

No known remarkable archeological or historically important structure or sites are reported in the subproject influence area. So, no cultural impact is expected.

Social Conflicts and Development Activities

There are no visible conflicts between the local communities regarding the subproject implementation. The local people welcome the subproject. There are some ongoing subproject under Pourashava funded by government and different donor agencies. However, there is no visible conflict between these donors and agencies.

# ENVIRONMENTAL SCREENING

Environmental Screening Checklist, as adopted in Appendix C of the Environmental Management Framework (EMF) of the MGSP, was administered for identifying the impacts and their extents.

The screening data for the RCC Road and Drain from Ellenga College Moar to Ellenga Bazar have been formulated and are shown in below:

(2)  Potential environmental impact during construction phase:

- Ecological impacts:

Nill

- Felling of trees  			Significant □	Moderate □  	Minor □√    Number of trees

- Clearing of vegetation  		Significant □	Moderate □  	Minor □√

- Potential impact on species of 	Significant □  	Moderate □  	Minor □√

aquatic (i.e., water) environment

The subproject site is located in the developed urban area. Hence, there is no tree felling and vegetation clearing work is needed. Anticipated impact on the species of aquatic environment is also minor because except throwing of the waste materials and accidental leakage and spillage of the oil and grease from subproject vehicles and equipment into the existing water bodies, the subproject activities do not have any direct impact on the aquatic environment.  Based on the above analysis it’s seems that overall ecological impact is considered as minor.

- Physicochemical impacts:

- Noise and Air pollution        		Significant □     Moderate □√ 	Insignificant □

- Drainage congestion         		Very likely □     Likely □√ 	Unlikely □

- Water pollution         			Significant □     Moderate □√	Insignificant □

- Pollution from solid/ construction wastes Significant □    Moderate □√	Insignificant □

- Water logging          			Significant □     Moderate □	Insignificant □ √

Construction of RCC road and drain will have negative impact on noise and air quality due to mobilization of the equipment, vehicles movement for the transportation of the materials and use of hydraulic excavator, brick breaking machine, mechanical compaction machine, etc. The construction work will be performed section wise. Hence, it is anticipated that the impacts will be temporary, site specific and not significant. The generated dust during construction activities may disturb the local people if measures are not taken. The generated construction wastes, un-suitable materials and solid wastes may degrade the adjacent water bodies if not properly collected, disposed and thrown in to the nearby water bodies.  Improper collection and disposal of the generated wastes materials also may degrade the quality of the surrounding environment and degrade the landscape value. The anticipated water logging problem during construction period of RCC road is insignificant. The subproject will have no adverse impact on the topography, climate, soils & regional hydrology.

- Socio-economic impacts:

- Traffic congestion 			Very likely □  	Likely       □√    	Unlikely     □

- Health and safety 			Significant □  	Moderate □√	Insignificant  □

- Impact on archaeological and historical   Significant □  	Moderate □  	Insignificant □√

- Employment generation 		Significant □ 	Moderate □ √  	Insignificant  □

Currently due to bad condition of the road, motorized vehicles movement is very few. However, three wheeler auto bikes are commonly using this road. Therefore, during construction work free movement of the motorized vehicles and pedestrian will impede.  However, construction work will be performed section wise to continue the traffic movement. The construction work will follow simple and conventional method and use of equipment is not heavy. So, impact on heath and safety is considered as moderate. There is no archaeological and historical site within the influence area. The subproject will have positive impacts by generating employment opportunities for the local people and creating business opportunity of supplying construction materials, equipment, food and other necessary staffs to the work camp.

(3)  Potential Environmental Impact during Operational Phase:

- Ecological impacts:

- Potential impact on species of aquatic 	Significant □  	      Moderate □	      Minor  □√

(i.e., water) environment

The anticipated impact on the aquatic environment is insignificant due to operation of the road. The drain is designed for discharging the storm water. Hence, anticipated impact on the aquatic environment due to the operation of the drain is likely minor. However, if the drain carries pollutants from any sources, it may degrade the quality of the aquatic environment.

- Physicochemical impacts:

- Potential air quality      	Improvement □      	No-improvement □          Deterioration □√

- Potential noise level      	Improvement □      	No-improvement □          Deterioration □√

- Drainage congestion      Improvement □√    	Minor Improvement □ 	 No Impact □

- Risk of water pollution    Significant □  		Moderate □  		 Minor □√

- Pollution from solid waste Improvement □ 	No-improvement □          Minor □√

After completion, due to improve road traffic volume may increase. Consequently, air quality and noise level may be degraded due to black smoke emission and hydraulic horn from the vehicles. The drain component does not have any impact on the noise. The new drain will minimize drainage congestion and water logging problem. The new drain also minimizes throwing of the waste materials and spreading of the bad odor from the drain. The storm water to be discharged through the drain may degrade the water quality of the outfall and adjacent water bodies if carry pollutants from any sources.

(f)Socio-economic impacts:

- Traffic                                  Improvement  □√   	 No-improvement    Adverse  □

- Safety 			     Improvement  □√   	 No-improvement    Adverse □

- Employment generation      Significant      □   	 Moderate  □    	      Minor     □ √

After completion, this road will improve the transportation facilities. By providing uniform pavement, it will also enhance traffic safety. New road will prevent the accumulation of the stagnant water on the road surface. This will prevent formation of muddy and slippery surface on the road. The new drain will also reduce stagnation of the water on the road. Thus, it will enhance road safety.

(4)  Summary of the Possible Environmental Impacts of the Subproject:

Due to implementation of the subproject ecological impact is minor. There will be temporary negative impacts on air and noise quality during construction. Nevertheless, these impacts are significantly manageable if mitigation measures are taken according to EMP. Solid wastes generation from the construction activities should be disposed in a designated dump site. Health and safety issue should be considered properly especially during construction phase. The subproject will also create some employment and business opportunities for the local people.

# Specific impact, mitigation, AND enhancement measures

This subproject includes construction of RCC road and drain. The possible impacts are mainly caused by the key activities to be performed for the implementation of the subproject, raw materials and equipment to be needed for execution of the subproject activities. This section describes some specific impacts due to the subproject activities and their mitigation measures.

  - Dismantling ,Site Clearing, , Excavation and Earth Work

The subproject works includes demolition of existing damaged BC road and HBB road, earth excavation and earth filling work. These works lead dust blowing, improper disposal of the wastes, noise and vibration which may disturb the local people.

Mitigation Measures

- Proper care will be taken by the contractor during site clearing work, excavation work, earth work and disposal work to avoid any undue disturbances to the nearby people;

- Cover exposed earth works and other wastes materials with fabric;

- Disposal of soil to designated dump site at Moshajan dumping site.

- Avoid loss of the topsoil for the earth work.

  - Pollution from the Construction Materials

Dumping of the construction spoils, including accidental leakage of the oil, grease, and fuel in equipment yards is a significant hazard. Surface water and soil quality might be polluted from these contaminants.

Mitigation Measures

- Safe transport, storage, and disposal provisions for the construction materials, and the equipment have to be carried out in order to avoid the accidental spillage and loss;

- Maintain adequate moisture content of soil and sand during transportation, compaction and handling;

- Carry the materials especially loose soil and sand with adequate cover;

- Fuels, lubricants, and other hazardous materials should store over raised platforms and  not directly on the ground;

- Disposal of un-used soils and construction wastes designated dump site at Moshajan dumping site.

- The playgrounds of the educational institutions should not use as a stockyard or work campsite.

  - Air Quality and Dust

During the construction phase, air pollutants will be emitted from the equipment and construction vehicles are expected to remain low. Local residents in the vicinity of the work sites will be temporarily disturbed by the limited dust pollution. The overall impacts, however, are expected to remain low.

Mitigation Measures

- When dust is visible water should be sprayed to control the dust;

- Maintain adequate moisture content of sand for handling, transportation, and compaction;

- Avoid use of dust generating equipment which produce significant amount of particulate matter far from the local residents;

- Ensure that all subproject vehicles are in good operating condition.

  - Noise and Vibration

Noise and vibration may be generated due to demolition work, from the movement of the subproject vehicles, using of concrete mixer machine, and vibrator machine etc. It may temporarily disturb the nearby people. The impacts are anticipated to be limited and within the limit.

Mitigation Measures

- Transportation of the construction materials have to be carried during the scheduled times, and mainly during the day;

- Casting work should be performed during the scheduled times ;

- Avoid use of rod cutter, concrete mixer machine and vibrator machine at night.

  - Water Quality

There is no remarkable sources of the water pollution have been found at the subproject area. The water quality may deteriorate if the construction materials, sand, construction wastes, effluent from the work camps, oil leaking from construction vessel and food wastes are dumped in the low land and Louhojong River.

Mitigation Measures

- Proper construction management including waste management as well as training of the operators and other workers should provide to avoid pollution of the water bodies;

- Construction wastes will dispose properly (not in water bodies or lowland), for which contractor will be responsible;

- Construction wastes will dispose properly at the designated dumping site at Moshajan dumping site (not in the water bodies or lowland), for which contractor will be responsible.

- Refueling and fuel tank should not be placed on the ground directly.

- Fuel and oil should be stored at concrete floored tank surrounded with concrete fence

  - Occupational Health and Safety

The most important risks associated with the construction activities are listed below:

- Exposure to the sunlight- workers are being exposed to the sun for long hours;

- Exposure to the high temperature, and humidity for a long time resulting in dehydration;

- Contact with the hazardous substances and wastes pose risks of  the infections and diseases;

- Risk of the poor air quality due to dust;

- Risk of the collision (traffic);

- Risks from the head loads for carrying materials;

- Risks of using of the machinery in motion such as rod cutter and concrete mixture machine;

- Risk associated to the sudden bad weather working conditions;

- Risk associated due to haphazard work by the workers.

General Requirements for the Workers’ Health and Safety

The key salient features of the general requirements for the workers’ health and safety stated is presented in Table 5.6.1.

Table 5.6.1:  General Requirements for the Workers Health and Safety

| Issues | Requirements |
| --- | --- |
| Health and Hygiene | - Cleanliness at the site premises and workers living places and at the Labor Shed; - Arrangement of the proper ventilation and temperature at the Labor Shed; - Protection against dust and furnace by using of the nose masks and covering of the head and body; - Proper disposal of the wastes and effluents; - Provision of the adequate latrines and separate toilets for the women; - Sufficient dustbins for the solid waste management system. |
| Safety and First Aid Box | - Using of the personal protective equipment (helmet, gloves, goggles, nose mask, safety boots); - Precautions during work on or near machineries in motion; - Head loads are prohibited; - First aid facilities should be provided and maintained; - Ensure first aid box with the necessary medicine at the labor shed; - The first aid kit should include adhesive bandages, regular strength pain medication, gauze, and low grade disinfectant. |
| Compensation for Accidents at Work | - Contractors will bear medical treatment costs. If any severe accidents such as loss of hands, legs or loss of working ability or any case of death needs compensation-(the amount of the compensation should be fixed considering the type of accidents). |
| Dust and Fumes | - For any dust, fumes, or other impurities likely to be injurious to the workers, effective measures shall be taken to prevent their accumulation and its inhalation by the workers. |
| Overcrowding | - No labor room should be overcrowded. |
| Latrines and Urinals | - Sufficient latrines shall be provided; - Latrines shall be maintained in clean and sanitary condition; - Latrines shall be adequately lighted and ventilated. |
| Disposal of Wastes and Effluents | - Proper disposal system for the solid waste and effluent is required; - Waste bins must be provided by the contractor at labor shed. |

  - Impacts on Social Environment and Common Property Resources

Through comprehensive study, it is revealed that impacts are expected not to be severe and to be largely manageable. The following Table 5.7.1 presents impacts on socio-economic environment and common property resources

Table 5.7.1: Impacts on Social Environment and Common Property Resources

| Social Components | Impacts on IECs | Impact Significance |
| --- | --- | --- |
| Community Perception | The local community people welcome this subproject and there is no visible objection from them. | Significant (+ve) |
| Employment and Business Opportunity | Community feels happy because generally the local contractor will be engaged for the construction works which will create work opportunity for the skilled and non-skilled labor. The subproject will create business opportunity for the equipment and materials suppliers’. | Significant (+ve) |
| Community Order and Security | This subproject activity does not create any severe security problems to the local community and community people. | Minor (-ve) |
| Possible damage to existing infrastructure and facilities | Degradation of the existing road infrastructure by the construction equipment/vehicles used in this subproject. | Minor (-ve) |
| New infrastructure and facilities | Construction of new road and drain will enhance municipality infrastructure facilities. | Significant (+ve) |
| Labor Habitat | Most of the labors will stay at the Labor shed which will have impacts on the environment relates to the generation of the solid wastes, effluent, and water consumption. | Moderate (-ve) |
| Health Care | Workers may suffer from the dehydration problems, and other health hazards. | Minor (-ve) |
| Accident | In case of road accidents by the vehicles to be used for the transportation may have serious negative impact. | Significant (-ve) |

Specific Measures and Guidelines for the Key Social Issues

The primary objective of these guidelines is to ensure social compliances and requirements required for the subproject. The social guidelines required for the subproject as appeared in this report are given in below.

- Conduct dissemination with details about the subproject to the local community;

- Continue liaison with the community leaders in order to maintain the community support;

- Engage local contractor and local people for the positive perception of the local community;

- Avoid peak hours for transportation of the equipment and construction materials to avoid any undue disturbance to the roadside people;

- Ensure no child workers (less than 18 years) and aged worker (more than 65 years);

- Ensure no discrimination between the male and female in terms of the wages and getting work opportunity;

- Ensure regular payment to the workers;

- Consult with the workers for feeling of any health problems and take measures accordingly;

- Monitor the workers movement for avoiding any unexpected social activities (robbery, crime, political attachment and conflicts, taking drugs);

- Monitor contractor behavior and attitude to the workers.

  - Labor Influx and Anticipated Impacts

The labor force and associated goods and services required for the construction of infrastructure civil works under this subproject cannot be fully supplied locally. The migration to and temporary settlement of laborers in the subproject, referred to as labor influx, carries an array of potentially positive and negative impacts in terms of demands on public infrastructure, utilities, housing and sustainable resource management and the strain on social dynamics.

Labor influx effects on host communities include positive impacts such as

- The subproject activities will generate work opportunities for the local people and supplying of the construction materials, equipment, food and other necessary stuffs to the campsite;

- Improved infrastructure and public service access and availability whereby subproject investment catalyzes larger allocation of resources to a region, stimulating the development or expansion of infrastructure and public services.

Critical negative social risks include:

- Increase in criminal activity and alcohol and drug abuse, domestic violence, political attachment and violence, smuggling and robbery etc;

- Increase in gender-based violence, including eve teasing, sexual harassment etc.;

- Increases in communicable diseases, including respiratory problems, diarrheal diseases, vector-borne diseases (e.g., malaria), and sexually transmitted infections (e.g., HIV/AIDS, syphilis, gonorrhea, hepatitis B);

- Conflicts arising from increased demand on existing infrastructure, services, and utilities, including transportation, health, education, water and sanitation, waste management, public utilities and community, religious, and recreational facilities and loss of land for access routes.

The general environmental impacts of labor influx include pressure on the natural resources such as using of the water, electricity, other fuel for cooking, loss of land for the labor establishment, depletion of the water supply, sewage and waste water generation, degradation of the air quality, waste generation,  increased demands on the local energy and resources and noise pollution effects. The number of local and migrated people involved in the subproject activities can be only identified in the construction phase. Hence, these specific impacts will be quantified during construction work and environmental assessment report will be modified accordingly. However, the following safeguard measures are recommended to avoid any risk of labor influx:

- Inform local people about the subproject activities;

- Liaison with the community leaders in order to get community support;

- Engage local people as much as possible to minimize workers from outsiders;

- Monitor workers attitude and behavioral matter;

- Monitor the workers movement for avoiding any unexpected social activities (robbery, crime, political attachment and conflicts, drugs abuse);

- Inform and use local administration to get support if needed;

- Inform local utilities service providers (such as for new electricity connection REB or any other department);

- Ensure effective use of natural resources such as water, electricity, fuel, wood etc.

  - Impacts on Traffic Movement

The subproject activities may create traffic congestion during construction phase at Ellenga Bazar area. Hence, the construction work should be performed section wise with keeping provision for normal traffic operation. The pedestrians, cycle rider, biker and three wheelers can use the road at construction period. In fact, due to bad condition of the road currently vehicles movement is mostly battery operated three wheelers. Even, the medium to heavy motorized vehicles can use the alternative road during construction period. In addition, the Pourashava authority will inform local people to use connecting and other adjacent road during construction period to avoid any traffic congestion.

During construction phase, interruption of the local traffic system due to the subproject activities will be monitored closely. Then separate traffic management plan will be provided if required. However, the following safeguard measures are recommended to minimize the impacts associated to the traffic movement:

- Inform local people about the subproject activities;

- Inspire local people to use connecting and alternative roads;

- Ensure schedule deliveries of material/ equipment during off-peak hours;

- Place traffic sign/cautionary sign to avoid undue traffic congestion and associated traffic control measures to limit possible disruption;

- The place of construction works should be fenced off with fences if required and should be isolated from general public access and marked with signs to ensure safe movement.

- Environmental ManagementPlan (EMP)

The purpose of the Environmental Management Plan (EMP) is to ensure that the activities are undertaken in a responsible and non-detrimental manner. The EMP will guide the environmentally sound construction of the subproject and ensure efficient lines of communication between the PMU (LGED), Ellenga Pourashava, DSM and the contractors.

  - Access to Information

The environmental assessment report should be translated into Bengali and disseminated locally. The copies of the report (both in English and Bengali) will be sent to all the concerned personnel responsible for subproject implementation. It will also be made available to the public. The final assessment report will also be uploaded in the LGED website and the World Bank website after approval.

  - Grievance Redress Mechanism

The project-specific Grievance Redress Mechanism (GRM) has been established at Ellenga Pourashava to receive, evaluate, and facilitate the solution of APs concerns, complaints, and grievances concerning the social and environmental performance of the subproject. The GRM aimed to provide a time-bound and transparent mechanism to voice and resolve social and environmental concerns linked to the subproject.

The grievance mechanism is related to resolve the risks and adverse impacts of the subproject. It addresses APs’ concerns and complaints promptly, using an understandable and transparent process that is also gender responsive, and culturally appropriate. It is readily accessible to all the segments of the affected people at no costs and without retribution. The mechanism should not impede access to the country’s judicial or administrative remedies. The affected people will be appropriately informed about the mechanism.

LGED has its own Grievance Redress Procedure (GRP), which it operates to address any dissatisfaction and complaints by the local people regarding its activities. This procedure is being applied to address any complaints or grievances through negotiations with the community leaders and representatives of APs during implementation of the MGSP.

    - Grievance Redress Committee (GRC)

The discussions and negotiations will be conducted by the Project Implementation Unit (PIU) of Ellenga Pourashava, and will be involved the APs and Grievance Redress Committee (GRC) headed by the Mayor. With the facilitation of DSM, the Mayor nominated the GRC members and included representative from Government Agencies, local NGO, and Civil Society. The GRC has been formed and established at Ellenga Pourashava. A complaints box has already been provided in the Pourashava Office. The grievance response focal point is available at Ellenga Pourashava for instant response to an aggrieved person. The Focal Point will collect the written complaints or suggestions from the box, and produce them to the GRC for hearing and resolution. The GRC members are as follows:

Table 6.2.1.1: List of GRC committee members

| SL. No. | GRC Members Name | GRC Designation | Position |
| --- | --- | --- | --- |
| 1. | Nure Alam Siddique | Chairman | Mayor |
| 2. | Md. Jahurul Hoque | Member Secretary | Asst. Engineer, Ellenga Pourashava. |
| 3. | Md. Shahiduzzaman Mahmud | Member | Upazilla Social Services Officer |
| 4. | Suporna Bhowmick | Member | Head teacher , Ellenga govt. P/School |
| 5. | Md. Shamsul Alam | Member | Sr. Branch Manager , BURO Bangladesh |
| 6. | Md. Abdul Halim Sarker | Member | Secretary , Ellenga Bus-Stand Bonik Shamity |
| 7. | Project Related Word Councilor (Female) | Member | Councilor |
| 8. | Project Related Word Councilor(Male) | Member | Councilor |

    - Grievance Resolution Process

All complaints and suggestions will be received formally in the Pourashava Office by the GRC Member Secretary. A sample Grievance Redress Form has been prepared and sent to Ellenga Pourashava prior to the implementation of the subproject.

An intake register will be maintained at the Office of the Member Secretary. Member Secretary will record the details of the grievances in the intake register for documentation and ensure impartiality, fairness, and transparency. The intake register will have data and information columns including (i) Case no., (ii) date of receipt, (iii) name, type of complaint, grievance, (iv) father’s name, husband’s name, (v) sex, (vi) complete address of the person raising the complaint, grievance, (vii) main objection (loss of land, property, or entitlement), (viii) detailed complaint story, (ix) expectation with documentary evidence and previous records of similar grievances, etc.

No GRC members are allowed to contact the aggrieved persons in advance. Rather, the concerned persons are informed to attend the formal hearings at an appointed date. The GRC committee will sit for hearing the complaints of the aggrieved persons. The GRC will record salient points presented by the aggrieved person and will examine documentary evidence submitted during informal hearings. A resolution register will be maintained by the Member Secretary at the Ellenga Pourashava office. The resolution register will contain (i) serial no., (ii) case no., (iii) name of complaint, (iv) complaint story and expectation, (v) date of hearing, (vi) date of field investigation (if any), (vii) results of hearing and field investigation, (viii) decision of GRC, (ix) progress (pending, solved) and (x) agreement or commitments. Closing register will keep records such as, (i) serial no., (ii) case no., (iii) name of complaint, (iv) decision and response to complaints, (v) mode and medium of communication, (vi) date of closing, (vi) confirmation of complaintant’s satisfaction, and (vii) management actions to avoid recurrence.

The GRC will decide within 30 days of receiving a complaint. There will also be an appeals procedure where, if a person is dissatisfied with the ruling of the GRC, he or she or a representative may attend their next meeting to present the case again. The committee will then reconsider the case in private, after which their decision is final. If the appellant is still not satisfied, he or she has the right to take the case to the public courts. Ellenga Pourashava should also publish the outcome of the cases on the public notice boards. All costs involved in resolving the complaints (meetings, consultations, communication, and information dissemination) will be borne by the Ellenga Pourashava.

Based on consensus, this procedure will help to resolve issues or conflicts amicably and quickly, saving the aggrieved persons from having to resort to expensive, time consuming legal action. The procedure will however, not pre-empt a person’s right to go to the courts of the law.

  - Institutional Arrangement for the Safeguard Compliances

In the institutional arrangement procedure, Project Director, Team Leader/Deputy Team Leader will directly involve. The PD and TL/DTL would be supported by an environmental safeguard specialist and social management expert. The Pourashava Officials, especially members of infrastructure improvement sections, would be responsible for supporting the construction supervision as well as environment and social management with the help of the DSM consultants. The civil works contractors will implement these environmental mitigation measures.

The PMU (MGSP-LGED), with the help of environment and social management specialist, will submit the monthly and quarterly progress reports on environmental and social compliances to the World Bank.

PD-MGSP

TL/DTL-DSM

Environmental Safeguard Specialist-DSM

Social Management Specialist-DSM

Senior Municipal Engineer-DSM

Junior Environmental Specialist-DSM

Assistant Municipal Engineer-DSM

Social Management Officer-DSM

Pourashava

Engineer/Focal Point

Contractor

Figure 6.3.1: Environmental and Social Management Team (Tentative)

  - Capacity Building

A demonstration training program has already been conducted by PMU and DSM Consultants at Comilla City Corporation on 16 May 2016. Similar capacity building workshop will also be conducted in the Rangpur Zone to develop the capability of the PMU, Pourashava Officials and DSM field staffs. This workshop will be conducted by the PMU and DSM Consultants. The training program will include introductory course for the training the Ellenga Pourashava Officials and DSM field staffs, preparing them on: (i) Environmental screening, (ii) EMP implementation, including environmental monitoring requirements related to the mitigation measures; and (iii) Taking immediate action to remedy unexpected adverse impacts or ineffective mitigation measures found during the course of the implementation. The contractor should be also included in the training program to enhance the Environmental awareness and orientation among the workers.

A detailed training manual has already been developed by the Environmental Safeguard Specialist and Social Management Expert prior to the demonstration training program in Comilla region. Hence, the existing manual will be reviewed and modification will be done as per requirement.

  - Emergency Response and Disaster Management

Disaster management can be defined as the organization and management of
resources and responsibilities for dealing with all humanitarian aspects of emergencies, in particular the preparedness, response, and recovery to lessen the impact of disasters. Emergency Preparedness Planning (EPP) and Contingency Planning (CP) are the processes of disaster management plan for developing strategies, arrangements, and procedures to address the humanitarian needs of those adversely affected by the crisis. There are four main types of disasters, namely: Natural disasters, Environmental Emergencies, Complex Emergencies, and Pandemic Emergencies.

For MGSP activities, Pourashava officials would identify the immediate needs, prioritize the tasks, and identify resource requirements to address the humanitarian needs of those adversely affected by the crisis.

  - Environmental Management Action Plan

The environmental management action plan has been outlined in Table 6.6.1 and 6.6.2. The mitigation measures as well as monitoring program of the EMP are also incorporated in the environmental management action plan.

Table 6.6.1 Anticipated Environmental Impacts during Construction Phase and Corresponding Mitigation and Enhancement Measures (EMP Table)

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Construction and operation of labor shed for workers | - Generation of sewage and solid waste may cause water/ environmental pollution | - Construction of sanitary latrine considering 15 persons for one toilet at the labor shed and separate toilet for male and female; - Ensure provision of waste bins (introduce separate waste bins for organic and inorganic wastes); - Ensure wastes (solid wastes and other forms of the wastes) disposal at Moshajan dumping site. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment and medical supplies for the workers. |  |
|  | - Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |
|  | - Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well-being of the local people. |  |
| General construction works | - Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces regularly to reduce dust generation; - Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and handling; - Ensure contractor sprinkle and cover stockpiles of loose materials (e.g. fine aggregates). |  |
|  | - Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Construction work should be performed section wise; - Inform the local people about the subproject activities; - Provision of alternative and connecting road for Light and non-motorized vehicles to avoid undue traffic congestion and accidents. |  |
|  | - Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment producing excessive noise at night; - Regulate use of horns and avoid undue use of hydraulic horns in subproject vehicles. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes into adjacent water bodies (ponds, ditches, low wetlands and River if any). |  |
|  | - Accidents | - Conduct formal and informal discussion for creating awareness about the accidents; - Provide PPEs and ensure using of the personal protective equipment by the workers. |  |
|  | - Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil; - Collection, proper treatment and disposal of the spills. |  |
| All construction works | - Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - General degradation of the environment | - Ensure environmental enhancement measures, such as plantation, traffic/ direction signs and cautionary signs. |  |
| Environmental impacts due to the key construction activities and corresponding mitigation measures for the RCC Road and Drain |  |  |  |
| Excavation/ Dismantle work for existing HBB and BC road /Site clearing | - Generation of solid and construction wastes; - Generation of loose and clay soil due to the earth excavation work. | - Exposed earth works with much fabric and cover; - Disposal of soil and construction wastes at Pourashava designated Moshajan dumping site. | Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM Contractor Monitoring- Primarily by Pourashava Secondarily by PMU, LGED and DSM |
|  | - Accidents | - Carefully handle of the hydraulic excavator and hammer. |  |
|  | - Possible damage of road side infrastructure due to earth excavation for drain construction | - Ensure drum sheet palisading work for shallow depth to stabilize the structure; - Ensure plunk palisading work for shallow depth to stabilize the structure; - Bolly drive for deep depth construction works; |  |
|  | - Air pollution | - Regular maintenance of the equipment. |  |
| Sand filling/ Back filling work | - Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil and sand during transportation, compaction and handling; - Use cover for carrying sand and soil. |  |
| Cutting & welding of the reinforcement for RCC road and drain | - Noise pollution due to rod cutter and welding machine if any | - Avoid using of rod cutter and wielding machine at night; - Avoid prolonged exposure to noise (produced by equipment) by workers. |  |
|  | - Potential health and safety risks from rod cutter and welding machine if any | - Ensure use of the personal protective equipment (helmet, goggles, gloves, safety boot); - Availability and access to first-aid equipment and medical supplies in case of any accidents. |  |
| RCC (reinforcement concrete) work for RCC drain and road | - Air pollution due to black smoke emission from concrete mixer machine and vibrator machine | - Regular maintenance of the concrete mixer and vibrator machine to avoid any black smoke emission. |  |
|  | - Noise nuisance from concrete mixer machine and vibrator machine | - Avoid operation of the concrete mixer and vibrator machine at night; - RCC work should be avoided at schooling time; - Inform local people about casting work and potential impacts. |  |

Table 2: Anticipated environmental impacts during operation phase and corresponding mitigation and enhancement measures

| Activity / Issues | Potentials Impacts | Proposed Mitigation and Enhancement Measures | Responsible Parties |
| --- | --- | --- | --- |
| Operation of the road | - Increased air and noise pollution affecting surrounding areas | - Improved road may increase the traffic volume. Hence, effective traffic management is needed; - Better traffic management and avoid movement of heavy loaded vehicles that may exceed the load carrying capacity of the road; - Better traffic management. | Monitoring Primarily by Pourashava |
|  | - Possible road damage by movement of heavy loaded vehicles |  |  |
|  | - Increase traffic speed and accidents |  |  |
|  | - Increased traffic congestion due to increased traffic volume |  |  |
| Operation of RCC drain | - Pollution of downstream water body due to disposal of polluted water from the drain | - Ensure installation of septic tank by the household people in all establishment; - Avoid connecting sanitation facilities to storm drain. |  |
|  | - Possible backflow of water through drainage canal causing water logging | - Proper maintenance and cleaning of the drain and outfall on regular basis. |  |

  - Environmental Monitoring Plan

Environmental Monitoring Plan will help to evaluate the extent and severity of environmental impacts against the predicted impact and the performance of environmental protection measures. The following Table 6.7.1 has been prepared for the key environmental indicators.

Table 6.7.1: Matrix Table of Monitoring Plan (Visual observation during construction phase)

| Monitored Parameter/ Issues | Monitoring method/ Key aspects | Location of monitoring | Period & Monitoring Frequency |
| --- | --- | --- | --- |
| Safety orientation and training of workers | Frequency of training and orientation of workers for safety | Subproject site | - Once in a month - Reporting: Once in a month |
| Personal protective equipment and safety equipment | Ensure workers to wear and use safety equipment | Subproject site | - Daily - Reporting: Once in a month |
| Worker’s health | Monitoring process of worker’s health | Subproject site | - Daily - Reporting: Once in a month |
| Sanitation & drinking water facility to the workers | Availability of safe drinking water and sanitation to the workers | Subproject site | - Daily - Reporting: Once in a month |
| Incident record & reporting | Documented record of all incident, and accident | Subproject site | - Daily - Reporting: Once in a month |
| Site security | Isolation of site from general access | Subproject site | - Daily - Reporting: Once in a month |
| Bulletin/ announcement boards/ prohibition signs | Visible in good condition or not | Subproject site | - Daily - Reporting: Once in a month |
| Equipment / vehicles | Switched-off diesel engines/ equipment when not in use; Search any possible leakage; Fuelling. | Subproject site | - Daily - Reporting: Once in a month |
| Dust | Dust is visible or not | Subproject site | - Daily - Reporting: Once in a month |
| Oil waste generation and disposal | Quantity of oily waste, storage and disposal | Subproject site | - Daily - Reporting: Once in a week |
| Solid waste generation | Quantity of solid wastes and disposal | Subproject site | - Daily - Reporting: Once in a month |
| Drainage facilities at labor shed | Provision of open /closed surface drainage at camp site and stack yard | Subproject site | - Monthly - Reporting: Once in a month |
| Gender equity | No discrimination regarding payment between male and female workers | Subproject site | - Daily - Reporting: Once in a month |
| Child labor | No child will be engaged in the activities | Subproject site | - Daily - Reporting: Once in a month |
| Handling of hazardous materials | Fuelling, storage, and operation | Subproject site | - Daily - Reporting: Once in a month |

In addition, a comprehensive Environmental Compliance Monitoring checklist has been enclosed in Appendix-I.

  - Cost of Environmental Enhancement Works in BOQ

Considering the environmental impacts and their mitigation measures for this subproject, several items are included in the BOQ to address these issues. The estimated cost for implementing the EMP is elaborated in Table 6.8.1.

Table 6.8.1: Environmental Measurement Budget

| Item No. | Description of the Works | Unit Rate (BDT) | Qty (Number) | Amount (BDT) |
| --- | --- | --- | --- | --- |
| eme-1 | Dust suppression measures by water spraying throughout the construction period in and around the subproject sites, uncovered aggregates and loose materials such as stockpiles of the sand, excavated earth etc. | Lumpsum |  | 15,000.00 |
| eme-5 | Prevention of the spillage and leakage of the polluting materials (Detailed procedure will be given in the EMP) | Lumpsum |  | 5,000.00 |
| eme-6 | Temporary campsite wastes disposal facility during the construction period (collection, transportation, and dumping of the wastes at authorized dumping site at Moshajan: 2 nos. (1 no. for the organic wastes and 1 no. for the inorganic wastes disposal facility) | 20,000.00 | 2.00 | 40,000.00 |
| eme-7 | Campsite water supply facilities: Preferably 1 no. of tube well at the labor campsite (Depending on the site condition, DSM consultant will assist the contractor for selecting the option) | 20,000.00 | 1.00 | 20,000.00 |
| eme-8 | Campsite sanitation facilities: 2 nos. of the toilets preferably sanitary toilets at the labor campsite (1 no. for women and 1 no. for men) | 20,000.00 | 2.00 | 40,000.00 |
| eme-9 | a) Providing safety gear packages like hand gloves, spectacles for eye protection, ear plug, helmets, masks, visible jacket, safety shoes for at least 20 persons (15 workers and 5 visitors) | 3,000.00 | 20.00 | 60,000.00 |
|  | b) One first aid box with necessary accessories (contractor is responsible for providing necessary medicines, saline as per requirement during construction period) | 2,500.00 | 1.00 | 2,500.00 |
| eme-14 | Cautionary signs-6 nos. (Detailed specifications will be given in the EMP) | 2,500.00 | 6.00 | 15,000.00 |
| Total |  |  |  | 197,500.00 |

After approval to revise the cost estimate has lengthy complex procedure. Hence, as per project EMF, PMU suggestion and experience from other LGED projects, adequate budget has been allocated for the environmental management for the mitigation and enhancement measures. The subproject activities such as earth work, sand filling, black smoke emission from the subproject vehicles & equipment etc. may degrade the air quality in the subproject area. In addition, mechanical compactor, hydraulic excavator, drum truck, concrete mixer machine, vibrator and subproject vehicles may generate noise nuisance to the surrounding area. For this subproject, there is no provision of the analytical monitoring because air, noise and waste water quality data from the previous subproject (MGSP/ELN/2015-2016/W2) can be used as a baseline data to evaluate the impacts.

The budget for labor shed and site office construction is included in the civil works items. Therefore, it is not included in the environmental budget. It should be noted that the contractor will be paid as per actual work done.

# Public Consultation and participation

  - Methodology

In the context of preparing the Environmental Assessment (EA), participatory public consultation was conducted for the subproject. The Pourashava Mayor, Officials, Engineers and local individuals as well as LGED and Consultant participants participated. Informal Focus Group Discussions (FGD) and a formal CIP were conducted involving the participants (Photograph 7.1.2). In addition, walk-through informal group consultations and individual interviews were also held. The local communities were informed about the subproject intervention including their benefits. Suggestions made by the participants were listed and incorporated in the EMP accordingly.

Photograph 7.1.1: Consultants Team Visited the Subproject Site with Pourashava Officials, Local Councilor and Mayor of Ellenga Pourashava.

Photograph 7.1.2: Consultation Meeting at Subproject Site with Local Individuals.

  - Issues Raised by the Participants

The participants raised the issues related to the infrastructure development of Ellenga Pourashava. The anticipated environmental and social impacts due to construction works have also been raised and discussed. The participants also discussed about the potential benefit from the subproject.

  - Feedback, Suggestions, and Recommendations of the Participants

The participants were presented with feedback, suggestions, and recommendations listed below:

- The FGD results confirmed that an improved communication network is needed for future development of Ellenga Pourashava;

- Most of the participants expressed that the number of subproject that have been selected for each financial year is insufficient;

- The participants stated that the public water supply facilities, sanitation facilities, and access road are not adequate;

- The participants stated that construction works should be scheduled properly;

- Local people also showed expectation for the increased opportunities for employment for unskilled or semi-skilled labor in the construction work;

- Local people expressed that construction of new road and drain at Ellenga Bazar area will minimize the suffering of peoples and enhance the economic growth of the subproject area.

# Conclusions and Recommendations

Environmental impacts of the proposed road and drainage subproject have been assessed. The overall conclusion is that if the mitigation, compensation and enhancement measures are implemented in full, there should be no significant negative environmental impacts as a result of location, design, construction or operation of the subproject. Major improvements in quality of life and public welfare will result once the scheme is in operation.

A few key recommendations are outlined below:

- All mitigation, compensation, and enhancement measures proposed in this report should be followed by the concern authorities for implementing the subproject;

- The environmental management and monitoring plan proposed in this report also needs to be followed;

- Regular monitoring by  Pourashava, PMU (LGED) and DSM consultants is needed;

- The contractor will ensure using of the PPEs by the workers where applicable;

- The contractor is responsible for the proper disposal of the generated wastes materials from the subproject sites and campsite.

# 9  References

- Bangladesh Meteorological Department, 2013

- Geological Survey of Bangladesh,1978

- Wikipedia, Ellenga Pourashava, 2017

- Population and Housing Census, 2011

- GoB (1997), Environmental Conservation Rules 1997

- Environmental Assessment Volume 1: Overall Environmental Assessment [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) And Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh

- Environmental Assessment Volume 2: Environmental Management Framework (EMF) [Draft Final Report], September 2013. Municipal Governance and Services Project (MGSP), Local Government Engineering Department (LGED) And Bangladesh Municipal Development Fund (BMDF), Ministry of Local Government, Rural Development and Cooperatives, Government of the People's Republic of Bangladesh

### APPENDIX

### Appendix- I: Environmental Monitoring Checklist

Local Government Engineering Department

Municipal Governance Services Project (MGSP)

Environmental Compliance Monitoring Form

Part A: General Subproject Information

| Subproject Name | (a) Construction of RCC Road from Ellenga College Moar to Ellenga Bazar (Ch.0+00- Ch. 700m) including Link Road at Ellenga Bazar Area (Ch.0+00- Ch. 160m); (b) Construction of RCC Drain in Link Road from Ch.0+00- Ch. 180m. |
| --- | --- |
| Package No. | MGSP/ELN/2017-18/W-07 |
| ULB Name | Ellenga Pourashava |
| Approved Estimated Cost in BDT |  |
| Contract Amount in BDT |  |
| Contractor Name |  |
| Date of Commencement |  |
| Target Date of Completion |  |
| Physical Progress (%) |  |
| Financial Progress (%) |  |
| Person Responsible (PIU) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Person Responsible (DSM) (Name, Designation &Phone) for the Overall Subproject Management |  |
| Form Completed by (Name, Designation &Phone) |  |
| Overall Environmental Description of the Subproject |  |

Part B: Design, Preparation, and Legal Requirements

| Environmental Concerns (PMU, PIU & DSM) (Name, Designation & Phone) | PMU- |  |
| --- | --- | --- |
|  | PIU- |  |
|  | DSM- |  |
| Subproject Category | DoE-BD- | WB- |
| Environmental Clearance Received? | Yes | No |
| EA Required? | Yes | No |
| EA Prepared and Delivered? | Yes | No |
| EMP Prepared & Delivered Separately? | Yes | No |
| Items and Cost of EMP Implementation Included in the Contract? | Yes | No |
| EMP Included in the Procurement Documents? | Yes | No |
| Inspection Schedule/Last Inspection/Monitoring by PMU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by PIU Environmental Concerns | Date- |  |
|  | Key findings- |  |
| Inspection Schedule/Last Inspection/Monitoring by DSM Environmental Safeguard Specialist/Jr. Environmental Specialist | Date- |  |
|  | Key findings- |  |

Part C: Key Environmental Impacts

| The Subproject Results in any of the following Impacts? | Yes | No | NA | If yes, is the impact (give observation) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Significant | Moderate | Minor |
| Felling of the trees |  |  |  |  |  |  |
| Clearing of the vegetation that increase the risk of increased soil degradation or erosion |  |  |  |  |  |  |
| Disturbance of the terrestrial and or aquatic specifies |  |  |  |  |  |  |
| Noise pollution |  |  |  |  |  |  |
| Air pollution |  |  |  |  |  |  |
| Adverse effects on the quantity or quality of the surface water or groundwater |  |  |  |  |  |  |
| Production or increase the production of the solid waste |  |  |  |  |  |  |
| Drainage congestion |  |  |  |  |  |  |
| Water logging that increases the risk of the water related diseases |  |  |  |  |  |  |
| Traffic congestion |  |  |  |  |  |  |
| Public safety |  |  |  |  |  |  |

Part D: Work Place Environment and Gender Equity

| The Subproject Results in any of the following Impacts? | Yes | No | NA | Observations |
| --- | --- | --- | --- | --- |
| Does the contractor pay to the workers regularly? |  |  |  |  |
| Is there any discrepancy between the male and female workers regarding the wages or salary for the same works? |  |  |  |  |
| Is the contractor complying with the GOB labor law concerning the hiring of the workers? |  |  |  |  |
| Does the contractor engage women labors and does the project have suitable works for them? |  |  |  |  |
| Does the contractor engage child labor (less than 18 years) and aged people (more than 65 years old)? |  |  |  |  |
| Does the contractor force to the workers for the completion of the works? |  |  |  |  |
| Do the workers involve with the political activities, crime, drugs addiction and other forms of unwanted activities? |  |  |  |  |
| Are construction camps adequately equipped with water supply, sanitary toilets, washing facilities and facilities for waste collection and storage? |  |  |  |  |
| Has separate sanitation facilities been provided for women at work camps and the construction site? Do the laborers load heavy items on their heads or shoulders? |  |  |  |  |
| Has the contractor undertaken an awareness program for the sexually transmitted diseases especially for HIV-AIDS and other infectious diseases like TB? |  |  |  |  |
| Are first aid kits readily available for the workers at the job site along with the instructions for use? |  |  |  |  |
| Are supervisors or other site personnel trained in the basic first aid emergency response measures? |  |  |  |  |
| Has the contractor provided necessary safety equipment to the workers and training for use? |  |  |  |  |

Part E: Potential Impacts, Mitigation Measures, and Monitoring Indicator Mentioned in the EMP

| Activity/ Issues | Potential Impacts | Proposed Mitigation and Enhancement Measures | Monitoring Method | Monitoring Frequency | Implementation Status (Yes/No /NA) | Impact significance if not implemented |  |  |  | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Major | Moderate | Minor | No |  |
| Construction and operation of labor shed for the workers (Workforce and labor shed management) | Generation of sewage and solid waste may cause water/ environmental pollution | - Construction of sanitary latrine considering 15 persons for one toilet at the labor shed and separate toilet for male and female; - Ensure provision of waste bins (introduce separate waste bins for organic and inorganic wastes); - Ensure wastes (solid wastes and other forms of the wastes) disposal at Moshajan dumping site. |  |  |  |  |  |  |  |  |
|  | Health of workers | - Conduct formal and unofficial discussion to increase awareness about hygiene practices among the workers; - Ensure availability and access to first-aid equipment (at least one first aid box for one labor shed) and medical supplies for the workers. |  |  |  |  |  |  |  |  |
|  | Possible development of labor camp into permanent settlement | - Contractor to remove labor camp at the completion of contract. |  |  |  |  |  |  |  |  |
|  | Outside labor force causing negative impact on health and social well-being of local people | - Ensure that contractor employ local work force to provide work opportunity to the local people and conduct formal and unofficial awareness program for the health and social well-being of the local people. |  |  |  |  |  |  |  |  |
| General construction works | Air pollution | - Check regularly and ensure that all the subproject vehicles are in good operating condition; - Ensure contractor spray water on dry surfaces regularly to reduce dust generation; - Maintain adequate moisture content of soil and sand for transportation, compaction, bed preparation, backfilling and handling; - Ensure contractor sprinkle and cover stockpiles of loose materials (e.g. fine aggregates). |  |  |  |  |  |  |  |  |
|  | Traffic congestion, effect on traffic and pedestrian safety | - Ensure schedule deliveries of materials/ equipment during off-peak hours; - Construction work should be performed section wise; - Inform the local people about the subproject activities; - Provision of alternative and connecting road for Light and non-motorized vehicles to avoid undue traffic congestion and accidents. |  |  |  |  |  |  |  |  |
|  | Noise pollution | - Check and maintenance the equipment properly; - Avoid using of construction equipment producing excessive noise at night; - Regulate use of horns and avoid undue use of hydraulic horns in subproject vehicles. |  |  |  |  |  |  |  |  |
|  | Water and soil pollution | - Prevent discharge of fuel, lubricants, chemicals, and wastes into adjacent water bodies (ponds, ditches, low wetlands and River). |  |  |  |  |  |  |  |  |
|  | Accidents | - Conduct formal and informal discussion for creating awareness about the accidents; - Provide PPEs and ensure using of the personal protective equipment by the workers. |  |  |  |  |  |  |  |  |
|  | Spills and leaks of oil, toxic chemicals | - Proper handling of lubricating oil and fuel so that it does not fall on the soil; - Collection, proper treatment and disposal of the spills. |  |  |  |  |  |  |  |  |
| All construction works | Beneficial impact on employment generation | - Employ local people in the subproject activities as much as possible; - Give priority to poor people living within subproject area in subproject related works (e.g., excavation and other works, which do not require skilled manpower). |  |  |  |  |  |  |  |  |
|  | General degradation of the environment | - Ensure environmental enhancement measures, such as plantation, traffic/ direction signs and cautionary signs. |  |  |  |  |  |  |  |  |
| Measures for the RCC road and drain |  |  |  |  |  |  |  |  |  |  |
| Excavation/ Dismantle work for existing HBB and BC road /Site clearing | Generation of solid and construction wastes; Generation of loose and clay soil due to the earth excavation work. | - Exposed earth works with much fabric and cover; - Disposal of soil and construction wastes at Pourashava designated Moshajan dumping site. |  |  |  |  |  |  |  |  |
|  | Accidents | - Carefully handle of the hydraulic excavator and hammer. |  |  |  |  |  |  |  |  |
|  | Possible damage of road side infrastructure due to earth excavation for drain construction | - Ensure drum sheet palisading work for shallow depth to stabilize the structure; - Ensure plunk palisading work for shallow depth to stabilize the structure; - Bolly drive for deep depth construction works; |  |  |  |  |  |  |  |  |
|  | Air pollution | Regular maintenance of the equipment. |  |  |  |  |  |  |  |  |
| Sand filling/ Back filling work | Air and dust pollution affecting nearby settlements | - Maintain adequate moisture content of soil and sand during transportation, compaction and handling; - Use cover for carrying sand and soil. |  |  |  |  |  |  |  |  |
| Cutting & welding of the reinforcement for RCC road and drain | Noise pollution due to rod cutter and welding machine if any | - Avoid using of rod cutter and wielding machine at night; - Avoid prolonged exposure to noise (produced by equipment) by workers. |  |  |  |  |  |  |  |  |
|  | Potential health and safety risks from rod cutter and welding machine if any | - Ensure use of the personal protective equipment (helmet, goggles, gloves, safety boot); - Availability and access to first-aid equipment and medical supplies in case of any accidents. |  |  |  |  |  |  |  |  |
| RCC (reinforcement concrete) work for RCC drain and road | Air pollution due to black smoke emission from concrete mixer machine and vibrator machine | - Regular maintenance of the concrete mixer and vibrator machine to avoid any black smoke emission. |  |  |  |  |  |  |  |  |
|  | Noise nuisance from concrete mixer machine and vibrator machine | - Avoid operation of the concrete mixer and vibrator machine at night; - RCC work should be avoided at schooling time; - Inform local people about casting work and potential impacts. |  |  |  |  |  |  |  |  |

Prepared by-                                                                                                 Signature-                                                                   Date-

Copies to 1.  PIU

2. SME

3. AME

4. Jr. ES