GUIDANCE NOTES FOR PREPARATION OF ESTIMATE

Name of the Project: e„nËi PÆMÖvg MÖvgxY AeKvVv‡gv Dbœqb cÖKí-3

Typical Item of Construction of Upazila, Union & Village  Road including Culverts/U-Drain/L-Drain for BC Surface

Section -1: Traffic Maintenance and Site Facilities

| Chapter | Sl. No. | Item Code | Brief Description of Items | Unit | Quantity | Engineer’s Estimate (Tk.) | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Road Works | 1. | 3.12.9 | Maintenance and protection of traffic at all times (at all hours during the day and night) along the existing roads, rivers or canals throughout the working period by maintaining all detours, including additional embankment filling, necessary barricades, warning lights and guide signs as well as other equipment, removal of all temporary constructions on completion of the bridge, etc. all complete as per requirement and instruction of E-I-C. | LS | 01 | 5000.00 to 10000.00 | Depending on Traffic Intensity |
|  | 2. |  | Erect sign board with details name of the project, cost and others as per specification and direction of the E-I-C. | LS | 02 | 10000.00 | For more than 1 km Road length |
|  | 3. |  | Re-location of public utilities | LS | Ps |  | If needed, actual cost will be quoted |

Section -2: Earth Works

| Chapter | Sl. No. | Item Code | Brief Description of Items | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- |
| Earth Work | 01. | 2.01.1 | Clearing and Grubbing | gvwUi Kv‡Ri cÖ‡qvRb n‡j wbw©Ïó Gwiqv‡Z G AvB‡Ug e¨envi Ki‡Z n‡e¦ ZvQvov mo‡Ki dvD‡Ûkb, Mv‡Qi †Mvov AcmviY, A_ev G RvZxq †Kvb AvB‡Ug cÖ‡qvRb n‡j Zvi `i †_vK wnmv‡e †ei Ki‡Z n‡e¦ |
|  | 02. | 2.02.2.01 | Earth filling Work with specified soil (85% Compaction) | Earth to be arranged by the Contractor mo‡K eZ©gvb Ae¯’vq wecyj cwigvb gvwUi Kv‡Ri cÖ‡qvRb n‡j Zvi ‡hŠw³KZv cÖ‡qvRb¦ mo‡Ki cy‡iv ‰`‡N¨© †j‡fj mv‡f©i wfwË‡Z G ‡hŠw³KZv cÖgvY Kiv AwaKvsk †¶‡Î KwVb¦ GQvov mo‡Ki Right of Way we‡eP¨¦ ZvB †PB‡bR Iqvix Top_Bottom_Height (TBH) c×wZ‡Z †KejgvÎ Shoulder, Slope Gi †fs‡M hvIqv Ask Ges Protective Kv‡Ri cv‡k gvwUi Kv‡Ri cÖv°jb cÖ¯‘Z K‡i ¯‹x‡g AšÍfy©³ Ki‡Z n‡e¦ mo‡Ki wbw©Ïó ‰`‡N¨© †j‡fj mv‡f© Kiv m¤¢e n‡j gvwUi cÖKvi‡f‡` I ¯’vb we‡ePbvq 1:1 Xvj wba©viY K†i gvwUi cwigvb †ei Ki‡Z n‡e¦ Z‡e 1.00 jÿ UvKvi AwaK gvwUi †ÿ‡Î †j‡fj mv‡f© Ges md&UIq¨v‡ii gva¨‡g gvwUi wnmve cÖ`vb Ki‡Z n‡e¦ |
| cwiKíbv Kwgk‡bi †ivW wWRvBb AbymiY K‡i mo‡Ki †kÖYx Abyhvqx R¨vwgwZK cÖ¯’‡”Q` (Geometric cross section) wbwðZ Ki‡Z n‡e¦ |  |  |  |  |

Section-3(B): Pavement by BC

| Chapter | Sl. No. | Item Code | Brief Description of Items | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- |
| Road Works by BC | 01. | 3.01.4.2 | Earth work in box cutting on road crest up to 450mm depth… |  |
|  | 02. | 3.02.1.1 | Sand (FM-0.5) filling on the road bed in ISG Layer……………….. |  |
|  | 03. | 3.03.3.4 | Sand Sub-Base course with Brick Chips & Sand in proportion (1:1)……………….… |  |
|  | 04. | 3.04.3.2 | Brick on End Edging (125mm across) |  |
|  | 05. | 3.05.7.1 | WBMBC |  |
|  | 06. | 3.11.13 | Guide wall/Toe wall | Of necessary |
|  | 07. | 3.06.1.1 | Providing Prime Coat @ 1.2 Litre/sqm |  |
|  | 08. | 3.06.4.1 | Providing 25 mm thick bituminous carpeting | Union & Village Road |
|  | 09. | 3.06.5.1 | Providing 40 mm thick bituminous carpeting | Upazila Road |
|  | 10. | 3.06.3 | Providing 7mm thick Seal Coat |  |
| Dc‡Rjv †iv‡Wi †ÿ‡Î cvKv mo‡Ki cÖ¯’ 3.7 wgUvi; BDwbqb I Mªvg mo‡Ki †ÿ‡Î 3.00 wgUvi cÖ¯’ wbwðZ Ki‡Z n‡e¦ mo‡Ki evuK (Curve) cwijwÿZ n‡j Extra Widening I Super elevation cÖv°j‡b AšÍ©f~³ Ki‡Z n‡e¦ cÖwZwU Extra Widening ‰`N¨© 10.00 wgUvi Ki‡Z n‡e¦ Ges PIov (0+0.9+0)/3 n‡e¦ moK ïiyi †ÿ‡Î I Extra Widening cÖ`vb Ki‡Z n‡e¦ |  |  |  |  |

Section-4: Structure (Construction of Box Culvert)

| Chapter | Sl. No. | Item Code | Brief Description of Items | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- |
| Bridges/ Culverts | 1 | 4.05.01.01.01 | Earth work in excavation in foundation trenches |  |
|  | 2 | 4.06.01.01 | Sand filling in prepared foundation FM0.8 |  |
|  | 3 | 4.06.02 | Single layer Brick flat Soling |  |
|  | 4 | 4.06.03 | Cement concrete in foundation (1:3:6) |  |
|  | 5 | 4.09.03.01 | RCC work in bottom part (1:1.5:3) |  |
|  | 6 | 4.09.03.02.01 | RCC work up to 5m (1:1.5:3). |  |
|  | 7 | 4.09.03.03.01 | RCC work height 5m in slab (1:1.5:3) |  |
|  | 8. | 4. 13.04 | Weep hole in abutment, wing wall |  |
|  | 9. | 4.09.03.08.01 | RCC work in railing and post (1:1.5:3) |  |
|  | 10. | 4.11.01.02 | MS ribbed bar (deformed) |  |
|  | 11. | 4.13.01 | 50mm av. wearing course (1:1.5:3) |  |
|  | 12. | 4.13.02.07 | 50mm PVC Rain water pipe |  |
|  | 13. | 4.11.05 | Nosing with MS angle |  |
|  | 14. | 4.13.03 | Back filling of abutment |  |
|  | 15. | 4.3.01 | Construction of temporary diversion |  |
|  | 16. | 2.02.2.01 | Earth filling |  |
|  | 17. | 4.05.03 | Pumping and Bailing out of water |  |

Section-5: Protective Works (Photograph needed where necessary)

| Chapter | Sl. No. | Item Code | Brief Description of Items | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- |
| Protective Works | 01. | 2.02.2 | Earth filling Work with specified soil | Road Slope ‡givg‡Zi wbwg‡Ë †h †Kvb cÖKvi Protective Work wbiƒrmvwnZ Kiv n‡”Q Gi cwie‡Z© †fs‡M hvIqv Ask Kgc‡ÿ 1:2 Slope G ch©vß gvwU cÖ`vb K‡i Road Slope Restore Ki‡Z n‡e¦ |
|  | 02. | 2.02.3.2 | Earth filling Work with specified soil (outside municipal area) | Where suitable soil are not available beside the pond. |
|  | 03. | 3.11.14 | Pre Cast RCC Post with Brick Retaining Wall Palisading | Relatively low height, this item can be provided. |
|  | 04. | 3.11.13 | Pre Cast RCC Post with cast in situ RCC wall Palisading | Note: Otherwise separate design has to be furnished on case by case basis. |

Section-6: L-Drain Works

| Chapter | Sl. No. | Item Code | Brief Description of Items | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- |
| L-Drain Works | 01. | 3.01.3.1 | Box cutting |  |
|  | 02. | 3.01.2 | Box cutting |  |
|  | 03. | 3.08.4.1 | BFS |  |
|  | 04. | 3.11.01 | CC (1:3:6) |  |
|  | 05. | 3.11.02.2 | Brick Work |  |

Section-7: Road Safety Works

| Chapter | Sl. No. | Item Code | Brief Description of Items | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- |
| Road Safety Works | 01. | 3.12.01.1 | Construction & Installation of kilometer Post | Where kilometer posts are not available. |
|  | 02. | 3.12.01.2 | Construction & Installation of Road Name Plate | Where Road Name Plate are not available. |
|  | 03. | 3.12.02 | Construction & Installation of Guide Post | Guide Posts can be provided at narrow bridge approaches only. |
|  | 04. | 3.12.04.1 3.12.04.2 3.12.04.3 3.12.04.4 3.12.04.5 | Supply & Installation of Pre-Cautionary, Informative & Mandatory Road Sign | Pre-Cautionary, Informative & Mandatory Road Sign can be provided |
|  | 05. | 3.12.05.1/ 3.12.05.2 | Painting of Railing, Rail Post & Guide Post | Existing Railing, Rail Post and Guide Post. |
|  | 06. | 3.12.06 | Painting & Writing on Existing Kilometer Post and Road Name Plate | Existing Kilometer Post and Road Name Plate. |
|  | 07. | 3.12.07.1/ 3.12.07.2 | Road marking paint on Existing Road Hump, Pedestrian Crossing, etc. | Existing Road Hump, Pedestrian Crossing, etc. |
|  | 08. | XXXXXXXX | Consider Curve widening or Extra widening at Black spots/hazardous locations and Road Inter-Sections; | Road curvature should be designed and extra widening can be provided by same road materials. If required, please provide detailed estimate in separate sheet. |
|  | 09. | XXXXXXXX | Two Road Hump (Flat & Round Type) with road marking and two traffic sign (Rail Crossing related) can be provided at each side of Rail Crossing. | If required, please provide detailed estimate in separate sheet. |

Section-8: Environmental Mitigation and Enhancement Works

| Chapter | Sl. No. | Item Code | Brief Description of Items | Unit | Quantity | Engineer’s Estimate | Guidance Note for Engineer’s Estimate Preparation |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1. |  | Dust suppression measures (excluding watering for compaction) to the entire satisfaction of the E-I-C. | Km |  | 2000.00 | for BC road |
|  | 2. |  | Providing and maintaining adequate potable water supply (Shallow Tube well-01) at camp site and work site to the entire satisfaction of E-I-C. | No. | 1 | 10000.00 | Depending on Project Size (more than 100. 00 lakh) |
|  | 3. |  | Providing and maintaining adequate sanitation facilities (01 nos for women and 01 nos for men) at camp site and work site to the entire satisfaction of E-I-C. | No. | 2 | 10000.00 | Depending on Project Size (more than 100. 00 lakh) |

D‡jøL¨ †h, hw` mo‡Ki Salvage _v‡K Z‡e gvV ch©v‡q we¯ÍvwiZ Inventory K‡i Rate Schedule Abyhvqx  Salvage wnmve  Ki‡Z n‡e I cªv°j‡b AšÍ©f~³ Ki‡Z n‡e  Ges KvwiMix cÖwZ‡e`‡b Zv D‡jøL Ki‡Z n‡e| hw` mo‡K m¤ú~b©/AvswkK e¨envi Dc‡hvMx evwj _v‡K †m‡ÿ‡Î cÖ‡`q evwj n‡Z ev` w`‡Z n‡e| Avi hw` e¨envi Dc‡hvMx evwj bv _v‡K Z‡e KvwiMix cÖwZ‡e`‡b e¨envi bv Kivi KviY D‡jøL Ki‡Z n‡e|